How NBo2 Sensor Input Controls Fuel Delivery

Revision 1   04/02/2017

*************** Not for Commercial Use ****************

* FOR EDUCATIONAL PURPOSES ONLY. USE AT YOUR OWN RISK *

*************** Not for Commercial Use ****************

Copyright 2017, Tom Oliphint (84Elky) All Rights Reserved

The goal of this document was to produce a single point of reference for: (1) Narrow Band Oxygen Sensor (NBo2)-related engine management information, and (2) details on the use and effect of o2 related Calibration items. It includes independent research and content from various articles and on-line forum posts.

Robert Rauscher published an excellent paper covering ECM Proportional Control which addresses o2 activities in the ‘7747 ECM (Post #11 here:

http://www.thirdgen.org/forums/diy-prom/404747-o2-swing-points.html).

Rather than approach things from the proportional control perspective, this document is geared toward how Injector Base Pulse Width (BPW) is calculated, and then how it is corrected in Closed Loop (CL) with NBo2 sensor input. Also addressed is the effect on BPW if o2-related Calibration values are changed. BPW determination in Open Loop (OL) is also addressed.

While this document specifically applies to Speed Density $8d (AUJP), commentary and hacks of other GM OBDI masks indicate their usage of o2 sensor input is similar to $8d. That is, the process of arriving at a BPW along with subsequent CL corrections may be slightly different, but the end result is the same. Every line of the $8d fueling-related code was examined. Only the activities of the NBo2 sensor are discussed since that’s the sensor controlling BPW.

There is a Summary below but also much detail that follows. Some may find the detail over-kill, but it’s provided for the single point of reference goal mentioned above, and for understanding how the code works. Even if not interested in the details, scanning the Fuel Minor Loop Code Flow section is recommended as it contains commentary not provided in the Summary.

Also, thanks to John Pell (JP86SS on thirdgen.org) for his contribution of Logs for analysis.

References are made to Calibration Values and Code Variables:

- Calibration Values (indicated by ‘0xnnn’) – Scalars, Switches and Tables are identified by hex addresses where ‘0x’ indicates a hex value, and ‘nnn’ is the hex location of the item from the beginning of the BIN file. Calibration values referenced are S_AUJP v4/v5 defaults. The ‘0xnnn’ hex notation is used rather than XDF Item Descriptions because the descriptions are not consistent among various XDF files.

- Code Variables (indicated by ‘Lnnnn’) - ‘L’ indicates a variable and ‘nnnn’ is the variable’s hex address in the code that can be used for ALDL Stream reporting (Logging). Variables are used by the code to store the state of various conditions, sensor data and calculated values. Many are reported in the ALDL Stream for presentation in data Logs. They are discussed here should there be a desire to output them in the ALDL Stream. Lacking this discussion, their existence would only be known by reviewing a source code hack.

Links to Document Sections:

Definitions

The NBo2 Sensor

Summary

SLOW And FAST o2 Voltages Explained

SLOW and FAST o2 Voltage Errors and going-Rich/Lean Explained

BPW Calculation and Correction

Effect on BPW and VE if Stoich AFR is Changed

o2 Calibration Values

Differences in GM 5.7L TPI o2 Calibration Values

Altering Factory o2 Calibration Values

1. Changing o2 Voltage Boundaries and Swing Points

2. Changing o2 Scalars that Define Error Trigger Points

3. Changing o2 Proportional Gain Count Table Values

4. Changing the Frequency of INT Increment/Decrement (+/-)

Code Simulations

Simulation A – Effect of BLM and INT on BPW

Simulation B - Effect of MAT and Coolant Temperature on BPW

How Switch 0x018 b4 Affects BPW

Simulation C – How Changing o2 Calibration Parameters Affects BPW

Summary of o2 Log Results

o2 Cross-Counts and Lack Thereof

Code Flow:

Fuel Minor Loop: Calculate and Correct BPW

Fuel Out Routine: Determine Final Fuel Out BPW

Fuel Minor Loop: BLM Update

Appendix A: o2 Calibration Differences-AUJP versus Other $8d 5.7L BINs

Appendix B: Code Simulation C: BPW Correction Example

Appendix C: Visual Representation of Reading the o2 Sensor and FAST and SLOW Voltage Errors

Appendix D: Log Results Using Various o2 mv Values

Appendix E: S_AUJP v5 o2 Sensor and Fueling Related Calibration Values

Document Revisions

Definitions

Before beginning, it’s necessary to define some terms to make this complex subject easier to understand:

- Calibration: A set of data residing in the BIN file used by the code in its execution. Calibration values can be changed using an XDF file or by directly changing values at their addresses in the BIN.

- XDF File: A file providing the ability to view and edit Calibration data in a BIN.

- Threshold or Boundary: Used interchangeably to define a Calibration o2 voltage, which when exceeded results in an o2 Voltage Error.

- o2 Voltage Error: An o2 Voltage Error occurs anytime o2 sensor voltage is above or below a Calibration o2 voltage Threshold. It’s not really an error -- just what it’s called. While there is only one voltage read from the o2 sensor, the code uses a technique called “Lag Filtering” to create two o2 voltages and associated Voltage Errors: SLOW and FAST (see SLOW and FAST o2 Voltages Explained below for details). o2 voltage is designated hereafter as FAST or SLOW where appropriate. FAST o2 Voltage is that reported in the ALDL Stream.

- BPW: Injector Base Pulse Width. A momentary fuel provision based on current engine conditions and Calibration values. Units are milliseconds (ms) which define how long all injectors are to be held open providing fuel.

- MAT and IAT: Used interchangeably to describe the incoming air temperature used in the BPW calculation (MAT=Manifold Air Temperature; IAT=Intake Air Temperature) .

- CL: Closed Loop. CL is the condition after the NBo2 sensor has been declared active, and engine run-time, coolant temperature and BLM Learn thresholds have been exceeded. The calculated BPW is corrected using NBo2 sensor input. Throughout this document, anytime a BPW correction is mentioned, it only applies to CL because there is no “o2-sensor-related” BPW correction in OL. The terms ‘correct’, ‘adjust’, and ‘update’ are used interchangeably to describe a change in BPW.

- OL: Open Loop. BPW is calculated the same as in CL, but is not corrected because NBo2 sensor input is ignored.

- BLM: The CL Block Learn Multiplier. GM defines this as the CL Long Term Fuel Trim.

- INT: The Integrator. GM defines this as the CL Short Term, or instantaneous, Fuel Trim. The distance from 128 of both the INT and BLM is used to correct BPW in CL.

The term “Trim” used to describe the INT and BLM is somewhat misleading as it implies a reduction. BPW is also increased. “Correction” or “Adjustment” might be a better descriptor.

- Desired Idle RPM: Closed Throttle Idle RPM from a Calibration Table based on Coolant Temperature. Also called “Commanded Idle RPM”.

- Fuel Minor Loop: A segment of the code that primarily computes and corrects the BPW. It is executed every 12.5ms or 80 times a second.

- Spark Minor Loop: A segment of the code executed immediately before the Fuel Minor Loop. It primarily computes the spark advance/retard required based on engine conditions, but also Lag Filters SLOW o2 voltage. It is also executed 80 times per second.

- Fuel Out Routine: A segment of the code called for execution by both the Fuel and Spark Minor Loops, causing it to be executed every 6.25ms or 160 times per second. It adds compensations to the corrected BPW and sends this Final Fuel Out BPW to the ECM firmware to fire the injectors.

- “Idle/Low MPH” and “NOT Idle” Conditions: This is something not well understood. The Idle/Low MPH and NOT Idle conditions determine the o2 Boundaries and Swing Points used for o2-related decisions:

Idle/Low MPH: 	Defined as TPS% <= 2.3 and MPH <= 15

NOT Idle: 		Any condition other than above

The Idle/Low MPH condition differs from the more familiar Closed Throttle Idle condition defined by:

TPS < 0.78% and MPH < 0.9.
In the remainder of this document, “Idle”, “Idling” or “Idle/Low MPH” refers to the above o2-related Idle/Low MPH state. “NOT Idling” refers to other than the Idle/Low MPH state. Closed Throttle Idle or Idling is indicated as “Closed Throttle Idle” to distinguish it from Idle/Low MPH.

- o2 Sensor Declared “Active”: The sensor is declared “active” the first time o2 sensor voltage is above an upper, or below a lower, sensor-is-active Calibration o2 voltage threshold.

- “going”-Rich/Lean: The use of the word “going” in conjunction with Rich and Lean is important in understanding how the SLOW and FAST o2 voltages are used. If a voltage is above 450mv, it is indicating a rich condition; and if below 450mv, a lean condition. But it’s the “going”-Rich/Lean conditions that are used. “Going” indicates the direction the o2 voltage is headed when compared to a previous voltage. For example, assume the previously recorded o2 voltage was 300mv and the current voltage is 400mv. In this instance, the voltage is said to be “going-Rich” (increasing from 300mv to 400mv, or “going” in the Rich direction), even though both voltages are Lean because they are < 450mv. This is a very important concept.

- FAST o2 going-Rich/Lean: A FAST o2 going-Rich/Lean condition is indicated every pass through the Fuel Minor Loop. It indicates the current direction of FAST o2 voltage.

- FAST o2 Voltage Swing Point:  A Calibration value around which o2 voltage is targeted to cycle above and below over time. Although not a requirement, a Swing Point is generally equidistant from the Upper and Lower o2 Boundaries.

- FAST o2 Voltage Error: FAST o2 voltage is compared to a Swing Point +/- 53 millivolts (mv) (53mv =Calibration item at 0x49C). We’ll call this the ‘FAST o2 Swing Point +/- 53mv’. This is the “trigger point” for a FAST o2 Voltage Error which occurs when FAST o2 Voltage is above or below the Swing Point +/- 53mv. Although computed, the magnitude of this error is not used. A FAST o2 Voltage Error is only used to indicate that FAST o2 voltage is “going”-Rich or Lean.

When a FAST o2 Voltage Error occurs, a FAST o2 going-Rich/Lean condition is indicated in the same direction as the error. If a FAST o2 Error does not occur, the current FAST voltage is compared with the FAST voltage recorded the prior pass through the Fuel Minor Loop (12.5ms ago) to set the condition. This ensures a FAST o2 going-Rich/Lean is indicated each pass through the Fuel Minor Loop. The condition determines the direction of INT and BLM changes and the direction of BPW correction.

- Cross-Counts: A Cross-Count occurs when there is a transition from a FAST o2 going-Rich to a FAST o2 going-Lean direction, and vice-versa (we’ll call it a ‘FAST o2 going-Rich/Lean Transition’). Cross-Counts reflect how frequently FAST o2 voltage is changing direction which indicates the responsiveness/condition of the o2 sensor. Cross Counts accumulate from 1 to 255 and are then reset. Cross Counts per second can be determined by examining ALDL data in Logs.

- SLOW o2 Voltage Error and SLOW o2 going-Rich/Lean: SLOW o2 Rich or Lean Voltage Errors occur when SLOW o2 Voltage is, respectively, above an Upper or below a Lower SLOW o2 Boundary. A SLOW o2 going-Rich/Lean condition occurs concurrent with and in the same direction as a SLOW o2 Voltage Error. A SLOW Error is the difference between the SLOW o2 voltage and the Boundary. Unlike a FAST Error, the magnitude of a SLOW Error is used (see % Adjusted SLOW o2 Voltage Error below).

- % Adjusted SLOW o2 Voltage Error: A SLOW o2 Voltage Error may be percentage adjusted downward by varying percentages depending upon whether Idling or NOT Idling. This % Adjusted SLOW o2 Voltage Error value is the prime determinant for incrementing/decrementing (+/-) the INT and correcting the BPW. The larger the % Adjusted SLOW o2 Voltage Error, the larger the BPW correction (and the larger the CL Proportional Gain Counts discussed below). Said another way: The higher the voltage, the more the ECM reduces BPW. The lower the voltage, the more the ECM increases BPW.

- CL o2 Proportional Gain Counts: Are used:

  - to compute an INT-based “Rough” BPW Correction using Counts from a Calibration Table based on the % Adjusted SLOW o2 Voltage Error. When these Counts are > 0, they provide a BPW correction in addition to that provided by the INT’s deviation from 128.

  - to provide a “Fine” BPW Correction to offset the effect of the Rough Correction.

Each of these BPW corrections is described below. Hereafter for brevity, these “Counts” will be referred to as “Prop Gain Counts”.

- INT-based “Rough” BPW Correction = (INT +/- Prop Gain Counts): Corrects the BPW based on the INT’s deviation from 128 and any Prop Gain Counts. Will be referred to as the ‘Rough BPW Correction’.

- “Fine” BPW Correction: This correction is ½ of the Prop Gain Counts, but is made in the opposite direction of the Rough BPW Correction to somewhat offset it.

- Summed Proportional Gain Counts: These Counts are different from, and not to be confused with, the above Prop Gain Counts. Summed Counts are from two Calibration Tables. One Table contains Counts based on RPM; the other based on the % Adjusted SLOW o2 Voltage Error. These Counts are summed after being obtained from the Tables. Each Table’s Counts represents one pass through the Fuel Minor Loop (12.5ms). They are used to determine when to later correct BPW if conditions did not initially warrant it. The Summed Proportional Gain Counts will be referred to hereafter as ‘Summed Prop Counts’.

If after these definitions you’re in overload, don’t worry. This is not easy. Hopefully that which follows will simplify understanding and put the various pieces into perspective.

The NBo2 Sensor

NBo2 sensor voltage controls BPW correction in CL. The senor does not provide reliable output until it reaches approximately 600°F. Once a single o2 sensor voltage exceeds an upper (Calibration item at 0x49F =699mv), or falls below a lower (Calibration item at 0x490 =199mv) sensor-is-active voltage threshold, the sensor is declared “active”. Thereafter, FAST o2 voltage direction begins to be recorded. This voltage direction is used to make FAST going-Rich/Lean-related decisions, even though not yet in CL. CL occurs later once engine run-time, coolant temperature and BLM Learn thresholds have been exceeded.

GM describes a properly functioning sensor as one oscillating smoothly above a minimum of 600mv and below a maximum of 300mv.

Note: When the sensor voltage is evenly cycling above and below 450mv, the Calibration Stoich AFR is being targeted because the average of all voltages approximates Stoich voltage and thus Stoich AFR, or a Lambda of 1.0. With an irregular o2 voltage pattern, Stoich AFR may not be consistently approximated. Refer to Appendix D o2 patterns obtained with different o2 Calibration values.

Lacking continuous sensor voltage movement, sensor Error Codes will be indicated:

- Error 13: Occurs with an open ECM-to-sensor circuit, or along with other conditions when

o2 voltage is between 300 and 600mv for 30 seconds

- Error 44: Lean too long (voltage < 200mv for > 50 seconds)

- Error 45: Rich too long (voltage > 700mv for > 50 seconds)

OL is forced upon encountering any of these error conditions.

Debunking some NBo2 sensor and o2-related beliefs:

- Belief: The sensor voltage automatically swings high and low.
Truth: It swings due to changes in the oxygen content in the exhaust (the difference between ambient o2 and exhaust o2). This oscillation results from changes in the fuel mixture (BPW). Constantly changing INT, BLM and Prop Gain Counts continually adjust the BPW in one direction and then in opposite direction, which changes the oxygen content of the exhaust. That drives the o2 Sensor to oscillate.

- Belief: The sensor reports voltages which approximate AFR.
Truth: The sensor only knows 3 things: Stoich voltage (approximately 450mv), the fuel mixture is Rich voltage (voltage is above Stoich voltage), or the fuel mixture is Lean voltage (voltage is below Stoich voltage).

- Belief: A Cross-Count occurs when o2 voltage crosses 450mv.
Truth: In $8d and likely other masks as well, a Cross-Count occurs when there has been a change in the direction of the FAST o2 voltage, and not when it crosses 450mv.

Various published material provides the following about NBo2 sensors in general:

- The NBo2 sensor’s primary purposes are:

  - to approximate Stoich AFR in CL by cycling above and below 450mv to hopefully provide an optimal fuel mixture to reduce emissions.

  - to satisfy catalytic converters. Converters need this cycling at proper amplitude and frequency to function at their maximum efficiency. The fluctuating BPW causes the sensor to report a Lambda of approximately 0.96 to 1.04, or within about 4% of Stoich. This process saturates and then starves the converter (running rich of 14.7:1 [saturate] and then lean of 14.7:1 [starve]).

Opinion: If catalytic converters are not installed, there is no reason to use CL. It greatly complicates the fueling process. Replace the NBo2 sensor with a WBo2 sensor, run in OL and use the reported AFR to tune the VE Tables. In CL, BPW is calculated and then corrected based on o2 voltage. In OL, it’s calculated identically as in CL, but there is no correction based on o2 voltage.

- A minimum sensor cycle frequency of 1 Hz, or 1 cycle per second is generally acceptable.

- An aging or fouled sensor tends to have a slower response time and may indicate a leaner voltage causing the engine to run rich, both of which can degrade engine performance and gas mileage

- o2 sensors over time tend to shift their cycling to the upper (Rich) side of the voltage scale.

- Sensor output varies with exhaust gas temperature.

- An o2 sensor with a high voltage reading does not necessarily mean the mixture is rich. An EGR valve problem can send the o2 voltage high.

- Silicone is the leading cause of sensor contamination

- Even though an o2 sensor is cycling and crossing 450mv, it does not mean it is working properly

A comment about 450mv: There is nothing in the $8d code that measures anything against 450mv. The Calibration item at 0x3F3 does however contain 450mv, but it is only used at ECM power-on to initialize three o2 variables:

L008E=Current Filtered o2 Voltage for FAST Error

L0090=Filtered o2 Voltage for SLOW Error

L0092=Filtered FAST o2 Voltage for AIR (every 100ms)

These variables are initialized to 450mv to provide a “previous (old)” value needed for Lag Filtering the voltages after the o2 sensor is first read, (see SLOW And FAST o2 Voltages Explained for an explanation of Lag Filtering).

Summary

The BPW determination process using the input of the o2 sensor is complicated. It occurs in sections of the code called the Fuel Minor Loop and the Fuel Out Routine. This summary will serve as a starting point for describing and simplifying that process.

Before covering how BPW is calculated and corrected, it’s necessary to understand the FAST and SLOW o2 voltages which are used to guide the process:

SLOW and FAST o2 Voltages Explained

As mentioned in the Definitions, although only one voltage is obtained from the sensor, SLOW and FAST voltages are computed from this voltage using “Lag Filtering”. Lag Filtering uses two input values and a smoothing factor to provide a “smoothed”, Lag Filtered result somewhere between the two inputs. The two inputs are a New (more recent) value, and an Older (previous) value with which the New value is “filtered” or “smoothed”. The Lag Filtering formula is:

NewFilteredValue = OldFilteredValue + ((NewValue – OldFilteredValue) * SmoothingFactor)

The Smoothing Factor plays an important role in the result:

- The larger the smoothing factor, the closer the result is to the New input value. A factor of 100% yields a result equal to the New input.

- The smaller the factor, the closer the result is to the Old input value. A factor of 0% yields a result equal to the Old input.

- A smoothing factor of 50% provides a result exactly half way between the Old and New input values.

The o2 voltage variables used for FAST and SLOW o2 Voltage Error determination are:

- L008E=Current Filtered o2 Voltage for FAST Error: This voltage is Lag Filtered in the Fuel Minor Loop using the voltage just read from the sensor (New Value) and the voltage Lag Filtered the previous pass through the Fuel Minor Loop (Old Filtered Value 12.5ms ago). The smoothing factor is a constant 93.7% (Calibration item at 0x3F1).

- L0090=Filtered o2 Voltage for SLOW Error: This voltage is Lag Filtered in the Spark Minor Loop 6.25ms after the Fuel Minor Loop is executed. The New input value is L008E above, which was Lag Filtered in the Fuel Minor Loop 6.25ms ago. The Old input value is L0090, which was Lag Filtered in the previous pass through the Spark Minor Loop 12.5ms ago.

The smoothing factor for L0090 is a combination of:

- A multiplier from the Table at 0x4C6=SLOW o2 Lag Filtering Coefficient based on a previously calculated representation of Ideal Gas Law Air Flow (L006B):

If Air Flow =   0 gm/sec	Coefficient = 0.063

If Air Flow = 64 gm/sec	Coefficient = 0.172

- Coolant Temperature (L005D)

- A Scalar =60.2%, used to factor the Coolant Temperature (Calibration item at 0x4A0=Coefficient for SLOW o2 Filter Constant)

The smoothing factor for L0090 computed from the above items is always varying and significantly less than the constant 93.7% FAST o2 factor. As an example, at 200°F coolant temperature and air flows ranging from 8 to 64 gms/sec, the L0090 SLOW o2 voltage smoothing factor ranges from 6.6% to 16.8% versus the FAST o2 factor of 93.7%.

Here’s a summary of what Lag Filtering looks like for SLOW and FAST o2 voltages:

|  | FAST L008E in Fuel Minor Loop | SLOW L0090 in Spark Minor Loop |
| --- | --- | --- |
| New Input Voltage | L008E (Current from Sensor) | L008E (from Fuel Loop 6.25ms Ago) |
| Old Input Voltage | Old Filtered L008E (12.5ms Ago) | Old Filtered L0090 (12.5ms Ago) |
| Smoothing Factor | Constant (93.7% and Large) | Variable (6.6% to 16.8% and Small) |
| Lag Filtered Voltage | L008E (Is closer to the New Input) | L0090 (Is closer to the Old Input) |

Two things distinguish FAST o2 voltage (L008E) from SLOW o2 voltage (L0090) due to Lag Filtering:

- The New input voltage for L008E is the voltage just obtained from the sensor; but for L0090 filtering in the Spark Minor Loop, the New input voltage is 6.25ms old. The Old input voltage for both is 12.5ms old. The more recent sensor voltage for L008E makes it more current.

- The smoothing factor used for L008E is much larger than that used for L0090, making L008E more closely resemble the New input, which is the most recent o2 sensor voltage (again more current).

So what does all this mean?

- The Lag Filtered L008E voltage is more sensitive to recent changes in o2 sensor voltage. Because of this, L008E is designated as the FAST o2 voltage. Because it’s the most recent voltage, it is used to indicate changes in voltage direction (Cross-Counts). This voltage is reported in the ALDL Stream (Logs).

- On the other hand, the Lag Filtered L0090 o2 voltage will more closely resemble the Old L0090 input value due to the smaller smoothing factor, and will not be very sensitive to recent changes. For this reason, L0090 is designated as SLOW o2 Voltage and is used to determine INT +/- and BPW Rough and Fine BPW Corrections. This “slower” voltage used for this purpose moderates the INT changes and BPW Corrections.

SLOW and FAST o2 Voltage Errors and going-Rich/Lean Explained

o2 Calibration Items

If Idling, a single o2 Swing Point and a single Lower and Upper o2 voltage Boundary is used for o2 voltage error determination. Each resides in a Calibration Scalar. If NOT Idling, multiple MAP(kPa)-based Swing Points and Upper/Lower Boundaries residing in three separate Tables are used for o2 voltage error determination:

|  | For SLOW Error | For FAST Error | For SLOW Error |
| --- | --- | --- | --- |
|  | LOWER Boundary | SWING POINT | UPPER Boundary |
| Idling/Low MPH (Scalars) Calibration Item: Value (mv): | 0x498 566 | 0x496 601 | 0x497 601 |
| NOT Idling (Tables) Calibration Item: Value (mv): MAP (kPa): | 0x4AB 530 583 469 20 40/50 100 | 0x4B4 574 627 521 20 40/50 100 | 0x4A2 619 672 553 20 40/50 100 |

SLOW and FAST o2 activities and errors occur independently of each other. Each is summarized below:

SLOW o2

Errors and going-Rich/Lean Conditions

- A SLOW o2 Voltage Error occurs when SLOW o2 voltage exceeds an Idling or NOT Idling SLOW o2 Voltage Boundary. SLOW Errors are % Adjusted and retained for later CL use (see Usage, CL below)

- A SLOW going-Rich/Lean condition occurs concurrent with and in the same direction as a SLOW Voltage Error

How SLOW o2 Errors are Used

General

- The larger a % Adjusted SLOW o2 Voltage Error, the larger the BPW correction

In CL Only

- If a SLOW o2 Voltage Error occurs, a % Adjusted SLOW o2 Voltage Error is computed using varying percentages depending upon whether Idling or NOT Idling

- If the % Adjusted SLOW o2 Voltage Error is:

  - > 17mv:

    - the INT may be +/-‘d if sufficient time has passed and other conditions have been met

    - Prop Gain Counts are obtained from a Calibration Table

    - A Rough BPW Correction value is computed using the Prop Gain Counts

  - <= 17mv

    - Summed Prop Counts define the time after which Prop Gain Counts will later be determined and the Rough BPW Correction computed. The INT is not +/-‘d.

IN OL Only

- SLOW o2 not used in OL.

FAST o2

Errors and going-Rich/Lean Conditions

- A FAST o2 Voltage Error occurs if FAST o2 voltage exceeds the ‘FAST o2 Swing Point +/- 53mv’. Unlike a % Adjusted SLOW o2 Error, the magnitude of a FAST o2 Error is not used.

- A FAST o2 going-Rich/Lean condition (voltage direction) is immediately indicated concurrent with and in the same direction as a FAST o2 Voltage Error.

- If a FAST o2 Voltage Error does not occur, current FAST o2 voltage is compared to that 12.5ms ago for determination of FAST o2 Voltage direction:

  - If current FAST voltage is > than that 12.5ms ago, a FAST going-Rich condition is indicated

  - Else if <=, a FAST going-Lean condition is indicated

Once a FAST going-Rich/Lean condition has been determined, there is no further use of a FAST o2 Error.

How FAST o2 going-Rich/Lean is Used

In CL Only

- Determines the direction of an INT adjustment if certain SLOW o2 voltage conditions exist and sufficient time has passed:

  - Increment the INT to increase fuel if both SLOW and FAST o2 are going-Lean and the INT <> Max Allowed

(Max Allowed =Calibration item at 0x493: Default =180)

  - Decrement the INT to reduce fuel if both SLOW and FAST o2 are going-Rich and the INT <> Min Allowed

(Min Allowed =Calibration item at 0x492: Default =80)

- Determines the direction of the Rough BPW Correction:

  - If FAST o2 going-Rich (BPW will be leaned):
     Rough BPW Correction = INT minus Prop Gain Counts

  - Else if FAST o2 going-Lean (BPW will be richened):
     Rough BPW Correction = INT plus Prop Gain Counts

- Used to determine BLM adjustment if sufficient time has passed since the last BLM update and the INT is more than 5 counts from 128:

  - If the INT is Lean (> 128) & FAST o2 going-Lean

    - Increase BLM 2 counts in the current BLM Cell

  - Else if the INT is Rich (< 128) & FAST o2 going-Rich

    - Decrease BLM 2 counts in the current BLM Cell

The BLM follows the INT and FAST o2 direction.

- Used to set the state of the Check Engine Light (CEL) once-per-second during o2 sensor ALDL Diagnostics with engine running and ALDL connector Pins A&B shorted:

going-Rich:	CEL On

going-Lean:	CEL Off

In Both OL and CL

- Causes the INT to be forced to 128 if any one of several conditions exist (see Check the status of INT related Switches below for details)

- Causes a Cross-Count to be recorded in variable L00FC when the FAST o2 voltage direction changes (indicated by a change in L0047 b6), and causes L00E9=# 12.5ms Fuel Minor Loop Passes without Cross-Count to be cleared

- If no Cross-Count, causes L00E9=# 12.5ms Fuel Minor Loop Passes without Cross-Count to be incremented

- Determines the direction of the 3 BPW corrections:

  - based on the BLM’s distance from 128

  - based on the Rough BPW Correction (=INT +/- Prop Gain Counts)

  - based on the Fine BPW Correction (½ Prop Gain Counts in the opposite direction of the Rough BPW Correction)

The above corrections are also performed in OL, but there is no change to BPW because values or correction factors =128 or =0.

Summarizing the effect of SLOW and FAST o2 Direction:

going-Lean

- o2 voltages are falling and possibly below Swing Point (FAST) and/or Lower Boundary (SLOW)

- If the % Adjusted SLOW o2 Voltage Error is > 17mv AND both FAST and SLOW o2 are going-Lean, the INT will be incremented, Prop Gain Counts determined and the Rough BPW Correction computed

- A Rough BPW Correction, if computed, will be > 128 (=INT + Prop Gain Counts), and BPW will be increased more than the INT’s deviation from 128: BPW + ABS(Rough BPW Correction - 128)

going-Rich

- o2 voltages are rising and possibly above Swing Point (FAST) and/or Upper Boundary (SLOW)

- If the % Adjusted SLOW o2 Voltage Error is > 17mv AND both FAST and SLOW o2 are going-Lean, the INT will be decremented, Prop Gain Counts determined and the Rough BPW Correction computed

- A Rough BPW Correction, if computed, will be < 128 (=INT - Prop Gain Counts), and BPW will be decreased more than the INT’s deviation from 128: BPW - ABS(Rough BPW Correction - 128)

BPW Calculation and Correction

Remember from above that NBo2 sensor input is only needed for emissions and catalytic converters. So before delving into the nuances of how the NBo2 sensor controls fueling, it is again my opinion that if catalytic converters are not installed, OL is preferred to CL provided a WBo2 sensor is used to indicate AFR. A WBo2 is necessary because the INT and BLM always =128 in OL.

In CL, the INT and BLM are constantly changing and BPW is constantly being corrected based on o2 sensor input. In OL, things are much simpler because BPW is not being corrected. If cylinder volume and injector flow rate are properly defined, OL relies primarily on the VE Tables for BPW calculation to provide the correct AFR.

Overview

The BPW calculation and correction process begins with reading the NBo2 sensor to get a voltage. That’s followed by using current engine conditions, a VE Table value and other Calibration items to calculate a BPW. Next, based on the direction of SLOW and FAST voltages, the INT is possibly +/-‘d, Prop Gain Counts determined and the Rough BPW Correction computed. These items, along with the current BLM, are used to correct the calculated BPW. This occurs 80 times a second.

BPW Correction: When SLOW and FAST voltages are going-Rich, BPW is reduced which in turn begins to reduce (pull down) the sensor’s voltage. When both SLOW and FAST o2 are going-Lean, BPW is increased. This begins to increase (pull up) the sensor’s voltage. As the magnitude of % Adjusted SLOW o2 Voltage Errors increase, so do the magnitude of BPW corrections which ultimately causes a reversal in voltage direction. These activities define the sensor’s oscillation.

INT and BLM: o2 voltage determines when the INT will be +/-d’, which affects the BLM; and both affect BPW correction. The INT is +/-‘d more frequently as RPM and air flow increase. The INT and BLM above and below 128 are a reflection of the direction and magnitude of a BPW correction (below 128, reducing BPW; above 128, increasing BPW). BPW is corrected significantly more based on the BLM’s deviation from 128 than the INT’s deviation from 128 (see Simulation A - BLM and INT Effect on BPW below).

Fuel Out Routine: After BPW is corrected (no correction in OL), control is transferred to the Fuel Out Routine which further adjusts BPW to arrive at a Final Fuel Out BPW. This Final fuel Out BPW is sent to the ECM firmware to fire all injectors for a time in milliseconds represented by the Final Fuel Out BPW. Control is then returned to the Fuel Minor Loop to possibly update the BLM.

Sequence of BPW-related Activities

Relevant BPW-related activities are summarized below in the order they occur and are indicated as they occur in OL only, CL only, or in both OL and CL. Each of these activities is covered in detail in the Fuel Minor Loop Code Flow section:

In OL and CL

- Set L00E3 b2 which primarily determines which set of o2 Boundaries and Swing Points will be used for o2 Error Thresholds:
   1=Low MPH/Idling

0=NOT Idling

- Determine a theoretical AFR depending upon engine state:

    - AFR is calculated for each of these states:
    in PE; in OL and NOT in PE; in CL Highway Fuel Mode

    - CL AFR, if not in Highway Fuel Mode, is always Stoich AFR defined in the Calibration item at 0x41A (Default=14.7:1)

- Test for an Idling/NOT Idling FAST o2 Voltage Error (when FAST o2 voltage exceeds the ‘FAST o2 Swing Point +/- 53mv’)

- Indicate in one of two ways a FAST going-Rich/Lean condition (voltage direction) for later use:

    - If a FAST o2 Voltage Error occurred, the going-Rich/Lean condition is indicted in the same direction as the error

    - If NO Error, then if current FAST o2 voltage differs from that 12.5ms ago:

      - If >, indicate FAST o2 going-Rich

      - Else if <=, indicate FAST o2 going-Lean

This FAST o2 going-Rich/Lean condition is used to later determine the direction of BPW correction and the direction of INT and BLM changes.

- Test for a change in the FAST o2 voltage direction (a FAST o2 going-Rich/Lean Transition [L0047 b6 will have changed])

    - If voltage direction has changed

      - Record a Cross-Count in L00FC and clear L00E9=# 12.5ms Fuel Minor Loop Passes without Cross-Count

    - Else if voltage direction has NOT changed

      - Increment L00E9=# 12.5ms Fuel Minor Loop Passes without Cross-Count

In OL Only

- Force the INT to 128

In CL Only

- Force the INT to 128 if in CL and:

    - If IN PE or Deceleration Fuel Cut Off (DFCO)

    - If NOT in PE or DFCO but IN Highway Fuel Mode

    - If NOT in PE or DFCO and NOT in Highway Fuel Mode

      - If Switch 0x018 b5=1 and the BLM Cell Address has changed

      - If Switch 0x018 b1=1 and in MAP of TPS AE

      - If Switch 0x019 b3=1 and CCP Active, or
CCP NOT Active and either of the following conditions exist:

{

In Decel Enleanment		---AND---

Fast o2 going-Lean		---AND---

INT >= 128 (Lean)

}

---OR---

{

NOT in Decel Enleanment	---AND---

kPa <= 0			---AND--- (MAP sensor problem)

RPM > 1200			---AND---

Fast o2 going-Lean		---AND---

INT >= 128 (Lean)

}

- Compute Summed Prop Counts for possible later use if the % Adjusted SLOW o2 Voltage Error <= 17mv

- Test for an Idling/NOT Idling SLOW o2 Voltage Error (when SLOW o2 voltage exceeds a SLOW o2mv Upper/Lower Boundary):

    - If a SLOW o2 Voltage Error occurred:

      - Set a SLOW o2 going-Rich/Lean condition in the same direction as the error

      - Possibly downwardly percent-adjust the SLOW o2 Voltage Error

    - Else if NO SLOW o2 Voltage Error:

      - The % Adjusted SLOW o2 Voltage Error is set =0 (no error)

- If the % Adjusted SLOW o2 Voltage Error > 17mv ---AND---

FAST and SLOW o2 voltages are both either going-Rich or Lean:

- +/- the INT if the required amount of time has passed

- Determine Prop Gain Counts

The more the % Adjusted SLOW o2 Voltage Error exceeds 17mv, the larger the Prop Gain Counts and the larger the BPW correction.

- Compute Rough BPW Correction = INT +/- Prop Gain Counts

- Else if the % Adjusted SLOW o2 Voltage Error <= 17mv

    - Set the % Adjusted SLOW o2 Voltage Error =0

    - Do not +/- the INT, and do not determine Prop Gain Counts or compute the Rough BPW Correction. However, if SLOW Errors continue to be <= 17mv for a period of time, the Summed Prop Counts will determine when to obtain Prop Gain Counts and compute the Rough BPW Correction.

(See Compute Summed Prop Counts for details.)

In OL and CL

- Obtain a value from the appropriate VE Table

- VE, RPM, MAP kPa and EGR Partial Pressure are used to compute L006F=Grams Air per Cylinder

- RPM and Grams Air per Cylinder above are used to compute L006B=Ideal Gas Law Air Flow in gm/sec for later use

- Calculate Speed Density BPW:

    - Grams Air per Cylinder computed above, AFR(L00F1), and the Calibration Injector Flow Rate at 0x41C are multiplied to compute the BPW

- This is the point at which the Calculated BPW is corrected in three ways:
These corrections are also executed in the code in OL, but there is no change to BPW because the corrections =128 or =0:

- BPW is first corrected based on the BLM’s distance from 128:

If the BLM < 128

BPW is reduced (leaned)

Else if the BLM > 128

BPW is increased (richened)

There is no BLM-based BPW correction if BLM =128.

- Next, BPW is adjusted using the Rough BPW Correction:

Rough BPW Correction = INT +/- Prop Gain Counts (from Table at 0x4DE)

BPW = BPW +/- ABS(Rough BPW Correction - 128)

(“+” if going-Lean; “-” if going-Rich)

- After the BLM and Rough corrections, if BPW is > 0, a Fine BPW Correction is made. This Fine correction is ½ of the Prop Gain Counts in the opposite direction of the Rough BPW Correction.

Aside: the BLM’s percentage deviation from 128 proportionally corrects BPW. This is why if tuning in CL it’s best to initially set the BLM minimum and maximum values to =128 (Scalars, respectively, at 0x51E and 0x51D). This forces the BLM =128 and will prevent the BLM from affecting the BPW in correction 1 above.

Also, in corrections 2 and 3 above, if all Prop Gain Counts in the Table at 0x4DE are set =0 they will have no impact on calculated BPW, and only a change in the INT will affect it. This means the calculated BPW is now primarily influenced by VE values.

This process is not 100% pure because the INT can’t be changed until both SLOW and FAST voltages are going in the same direction, and the % Adjusted SLOW o2 Voltage Error is > 17mv. But it’s the best indication of BPW correctness available short of WBo2 AFRs. Corrections to VE Tables can be made based on some percentage of the INT’s deviation from 128.

- If in Decel Enleanment, BPW is slightly decreased (details not covered).

- If sufficient time has passed since the last BLM update and the INT is more that 5 counts from 128, the BLM in the current BLM Cell is changed +/- 2 counts. The BLM is not recomputed if in OL because the INT will not have changed from 128.

See Fuel Minor Loop Code Flow below for additional details.

Robert Rauscher in the above-referenced article summed-up this process:

Another way to look at it: during closed loop operation the stoich AFR is used in the PW calculation. This is to create an initial PW. This PW is then modified with the INT & BLM values (and proportional gains), to create the PW used at the injectors.

The feedback from the o2 sensor is used by the ECM to aim at the target AFR. The fueling will be changed (via INT/BLM) to move the PW to the targeted o2 switch point (the window).

The bottom line is that in closed loop operation the ECM is always targeting the o2 switch point(s).

The statement in that last sentence can be visually seen in the Log results in Appendix D.

Summarizing the Conditions Under Which BPW is Corrected:

First, BPW is always corrected based on the BLM’s and INT’s deviation from 128 even if the % Adjusted SLOW o2 Voltage Error is <= 17mv:

  - If the % Adjusted SLOW o2 Voltage Error is > 17mv AND
FAST and SLOW o2 going in the same direction

    - If it’s time to +/- the INT

      - +/- the INT (will increase the size of the BPW correction)

      - Determine Prop Gain Counts and compute the Rough BPW Correction

    - Else if it’s NOT time to +/- the INT

      - The INT will not be +/-‘d but Prop Gain Counts will be determined and the Rough BPW Correction computed

  - Else if the % Adjusted SLOW o2 Voltage Error is <= 17mv

    - The INT will not be +/-‘d

    - If Summed Prop Counts >= L00E9=# 12.5ms Fuel Minor Loop Passes without Cross-Count, Prop Gain Counts will be determined and the Rough BPW Correction computed. Otherwise, no.

If interested, Appendix C provides a visual representation of reading o2 sensor voltage and how SLOW and FAST Errors are determined for BPW correction.

Effect on BPW and VE if Stoich AFR is Changed

The Factory Default Calibration Scalar at 0x41A =14.7 AFR. That Stoich value is an AFR estimate for use in the BPW calculation, regardless of OL/CL status.

If Calibration Stoich AFR is increased to reflect a leaner value, calculated BPW is decreased; and if reduced to reflect a richer value, BPW is increased. That’s obvious, but:

- How does a Calibration AFR change actually affect BPW?

- Is a change to VE Table values required?

- Are there different considerations for OL?

All are legitimate questions and are addressed below.

How does a Calibration AFR change actually affect BPW?

The Tables below answer the first question. They show the effect of reducing Stoich AFR from 14.7 to 14.2 as an AFR estimate for E10, 10% Ethanol, fuel (a 3.4% reduction). All BPW differences were computed using the parameters of the Factory o2 Code Simulations in Appendix B.

The first Table below shows the calculated BPW at 14.7 and 14.2 Stoich AFR and the BPW percent change. These values are before Fuel Minor Loop BPW corrections, so they also represent OL BPW values. The BPW at AFR=14.2 was calculated using the same VE values used for the BPW at AFR=14.7.

| Calculated BPW |  |  |
| --- | --- | --- |
| Stoich AFR | BPW(ms) | % Change |
| 14.7 | 1.465 |  |
| 14.2 | 1.526 | +4.2% |

The second Table below shows BPW values and percent changes after CL correction, but before Fuel Out compensations (see Fuel Out Routine and Appendix B for details). The Fuel Out BPW compensations are omitted because they are the same for any corrected BPW regardless of AFR.

| After BPW Correction |  |  |
| --- | --- | --- |
| Stoich AFR | BPW(ms) | % BPW Change |
| ***Factory Going Lean*** |  |  |
| 14.7 |  |  |
| With BLM +2 / INT +4 Lean | 2.853 |  |
| With BLM/INT=128 | 2.762 |  |
| 14.2 |  |  |
| With BLM +2 / INT +4 Lean | 2.945 | +3.2% |
| With BLM/INT=128 | 2.853 | +3.3% |
| ***Factory Going Rich*** |  |  |
| 14.7 |  |  |
| With BLM -2 / INT -4 Rich | 2.533 |  |
| With BLM/INT=128 | 2.640 |  |
| 14.2 |  |  |
| With BLM -2 / INT -4 Rich | 2.625 | +3.6% |
| With BLM/INT=128 | 2.731 | +3.5% |
|  | Average | 3.4% |

As seen in these Tables, the 3.4% reduction in Stoich AFR is inversely proportional to the 3.4% increase in BPW.

Is a change to VE Table values required?

Addressing the second question on changing VE Tables: Based on the above results, it seems logical that VE Table values will need to be reduced to provide less fuel. However that’s not necessarily the case. Why? Because E10 fuel burns leaner than unleaded without ethanol (E0 fuel). How much leaner? That data could not be found, but research did provide the BTU energy rating of these fuels. Unfortunately they vary depending upon the publication. However, what is consistent among publications is that the BTU energy ratings for E10 fuel are 2-3% below E0. That means it will take more E10 fuel to provide the same power output as E0. Would that be 3.4% more E10 fuel as in our example? That’s unknown without detailed analysis with a WBo2 or by otherwise comparing BLMs, but it’s likely very close indicating no change to VE Tables will be required.

Are there different considerations for OL?

Now to the third question about different considerations if operating in OL. Again, the change to VE Table values will be dependent upon BLMs or WBo2 AFRs. Unlike CL with a 3.4% BPW increase, OL provides a 4.2% BPW increase which may necessitate a slight decrease in overall VE.

o2 Calibration Values

Differences in GM 5.7L TPI o2 Calibration Values

Five, 1990 to 1992 5.7L TPI $8d Calibrations were analyzed and compared to AUJP to determine where GM set o2 Calibration values. The differences are provided in Appendix A. The Broadcast Codes (BCCs) examined were:

| BCC | Description |
| --- | --- |
| ANJF | 1990 F-Car TPI |
| AXCN | 1991 Corvette TPI 6-Speed Manual with 2.73 |
| ARFP | 1990 Corvette TPI 6-Speed Manual with 2.59 |
| AXCS | 1991 Corvette TPI Auto with 2.73 |
| ANHT | 1990 Corvette TPI Auto with 3.07 |
| AUJP | 1990-92 TPI “VIN 8” L98 |

It’s believed AUJP contains the latest version of the $8d Calibration.

GM changed only 5 o2-related Calibration items in the above BCCs:

| Calibration Address | Table |
| --- | --- |
| 0x4B4 | FAST o2 NOT Idling Rich/Lean Threshold .vs. MAP (Swing Point) |
| 0x4AB | SLOW o2 NOT Idling Rich/Lean LOWER Threshold .vs. MAP |
| 0X4A2 | SLOW o2 NOT Idling Rich/Lean UPPER Threshold .vs. MAP |
| 0X4BD | SLOW o2 INT Delay Time .vs. Air Flow |
| 0x4DE | CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error |

We know the above Calibration differences only affect NOT Idling conditions because GM only changed values in Tables (Idling/Low MPH values are in Scalars). Details of the differences are provided in Appendix A.

Altering Factory o2 Calibration Values

The frequency of INT +/- and the frequency and magnitude of BPW correction can be altered by changing one or more of the following:

- o2 Voltage Boundaries and Swing Points

- Scalars that determine o2 Boundary and Swing Point error triggers

- Prop Gain Count Table Values

- Changing the Frequency of INT +/-

Each is discussed below.

CAUTION: If catalytic converters are installed, one should proceed with caution making any changes discussed here so as to not materially alter the converter’s saturate/starve requirements (see above The NBo2 Sensor where catalytic converter characteristic are discussed).

- Changing o2 Voltage Boundaries and Swing Points

The Factory Boundaries and Swing Points are:

|  | For SLOW Error | For FAST Error | For SLOW Error |
| --- | --- | --- | --- |
|  | LOWER Boundary | SWING POINT | UPPER Boundary |
| Idling/Low MPH (Scalars) Calibration Item: Value (mv): | 0x498 566 | 0x496 601 | 0x497 601 |
| NOT Idling (Tables) Calibration Item: Value (mv): MAP (kPa): | 0x4AB 530 583 469 20 40/50 100 | 0x4B4 574 627 521 20 40/50 100 | 0x4A2 619 672 553 20 40/50 100 |

Analyzing the Factory values:

- All have a significantly rich bias which contributes to a rich BPW. No value is on the lean side of 450mv.

- All of the MAP based Tables begin with a rich value at 20 kPa, rise to an even richer value at 40/50 kPa where Closed Throttle Idling and cruising generally occur, and then taper down to a value at 100 kPa that is lower than the 20 kPa value.

- The Idling/Low MPH Upper Boundary equals the Swing Point. As shown above in Appendix C-Visual Representation of Reading the o2 Sensor and FAST and SLOW Voltage Errors, this will tend to richen the Idle.

- The NOT Idling Swing Points are for the most part 45mv equidistant from the Upper and Lower Boundaries. This indicates GM felt this spacing was optimal. Below is a visual of Boundaries and Swing Points at each kPa in the Factory o2 Tables:

| mv  kPa  | LOWER Boundary | Spacing  | SWING POINT | Spacing  | UPPER Boundary |
| --- | --- | --- | --- | --- | --- |
| 20 | 530 | 44 | 574 | 45 | 619 |
| 30 | 566 | 43 | 609 | 45 | 654 |
| 40 | 583 | 43 | 627 | 45 | 672 |
| 50 | 583 | 43 | 627 | 45 | 672 |
| 60 | 566 | 43 | 609 | 45 | 654 |
| 70 | 548 | 44 | 592 | 44 | 636 |
| 80 | 495 | 44 | 539 | 44 | 583 |
| 90 | 492 | 38 | 530 | 36 | 566 |
| 100 | 469 | 52 | 521 | 32 | 553 |

o2 Boundaries and SLOW o2 Errors

Changing o2 Boundaries can have unexpected results. This is seen visually with the SLOW o2 Voltage Error chart. The upper part of the chart shows SLOW o2 going-Rich, and the bottom going-Lean -- each with Factory Upper and Lower o2 Boundaries and those reduced by 50mv.

SLOW o2 Voltage Error

Factory	-50mv

L0090=SLOW o2 mv		------------	900mv

300mv

Factory Upper Boundary	------------	600mv

350mv

-50mv Upper Boundary	------------	550mv

Factory Lower Boundary	------------	400mv

250mv

-50mv Lower Boundary	------------	350mv

300mv

L0090=SLOW o2 mv		------------	100mv

--- SLOW o2 Voltage Boundaries and Errors -----

Summarizing the above chart:

- When Upper and Lower o2 Boundaries are both reduced 50mv from Factory settings, the Upper -50mv Boundary is 50mv below the Factory Upper Boundary, and the Lower -50mv Boundary is 50mv below the Factory Lower Boundary.

- Clearly that’s obvious, but the chart makes something clear that is not so obvious: Reducing Upper and Lower Boundaries by an equal amount creates unequal SLOW o2 voltage errors.

- Consider the chart where the L0090 SLOW o2 voltage cycles between 100mv and 900mv. Assuming the sensor was read at exactly these voltages, the Factory SLOW o2 voltage error would be 300mv going-Rich (900mv minus 600mv) and 300mv going-Lean (400mv minus 100mv).
But this is not the case with -50mv Boundaries. Here, the SLOW o2 going-Rich error would be 350mv (900mv minus 550mv .vs. Factory of 300mv), and the SLOW o2 going-Lean error would be 250mv (350mv minus 100mv .vs. Factory of 300mv).

- This continuing inequality will cause more fuel to be removed when going-Rich than will be added when going-Lean. Why? Because an INT +/- is a function of the % Adjusted SLOW o2 Voltage Error, and BPW correction is a function of the magnitude of that Error. The larger a % Adjusted SLOW o2 Voltage Error, the larger the Prop Gain Counts from the Table at 0x4DE used to compute the Rough BPW Correction. And the larger the Rough BPW Correction, the larger the BPW correction. If going-Rich and going-Lean conditions occurred exactly 50% of the time, reducing both Upper and Lower Boundaries by an equal amount would result in a leaner BPW (see also below 3. Changing o2 Proportional Gain Count Table Values, Third Prop Gain Count Table).

The opposite of the above would be observed if SLOW o2 Upper and Lower Boundaries were equally increased.

Is changing Boundaries in this way a problem? Not necessarily. But it’s important to understand the implication of such changes.

This analysis suggests that changes to o2 Boundaries should generally be directionally equidistant from the Swing Point as opposed to just reducing/increasing them by an equal amount. Being directionally equidistant from the Swing Point is in fact what the Factory NOT Idling Tables reflect as discussed above. With directionally equidistant values, the BPW reduction/increase will be approximately the same when going-Rich as going-Lean; and a smoother voltage pattern will be achieved which will result in a better approximation of Stoich AFR.

Now, having suggested that Boundaries should generally be adjusted to be directionally equidistant from the Swing Point, there might be an exception. For big cam engines that fight idling in CL, significantly reducing the Idle/Low MPH Boundaries and Swing Point to lower-than-Factory values (say to 400mv Swing Point and 400/300mv Upper/Lower Boundaries) should result in larger and more frequent BPW adjustments when going-Rich and a smaller and less frequent adjustments when going-Lean. This will cause a leaning of the Idle/Low MPH BPW without adjusting the VE Tables. This is what GM did with the Factory Idle/Low MPH values, but with a rich-side bias where the Swing Point and Upper Boundary both =601mv.

- Changing o2 Scalars that Define Error Trigger Points

In addition to altering o2 Boundaries and Swing Points, the following Scalars can also be changed to alter the voltage at which an o2 error will be triggered:

0x49C  =0x0C  =12 dec  =53 mvDC  =DIFF TO MAKE R/L WINDOW (FAST o2)

0x49E  =0x04  =04 dec  =17 mvDC  =MIN SLOW o2 ERROR TO MAKE INT CORRECTION

Changing 0x49C=DIFF TO MAKE R/L WINDOW (FAST o2) (Default =53mv):

This value is added/subtracted to/from Calibration Swing Points to define the voltage that must be exceeded to trigger a FAST o2 voltage Error:

- A value less than 53mv will increase the number of FAST o2 Voltage Errors and FAST going-Rich/Lean conditions (Cross-Counts are increased)

- A value more than 53mv will decrease them (Cross-Counts are decreased)

Changing 0x49E MIN SLOW o2 ERROR TO MAKE INT CORRECTION (Default =17mv):

This value must be exceeded by a % Adjusted SLOW o2 Voltage Error before the INT can be +/-‘d:

- A value less than 17mv will cause it to be exceeded more quickly by a % Adjusted SLOW o2 Voltage Error. This will cause the INT to be +/-‘d more frequently, which will result in larger and more frequent BPW corrections.

- A value more than 17mv will cause it to be exceeded less quickly by a % Adjusted SLOW o2 Voltage Error. This will cause the INT to be +/-‘d less frequently, which will result in smaller and less frequent BPW corrections.

If these FAST and SLOW o2 Scalars are changed, they should generally be moved in the same direction. Changing in opposite directions, or changing one and not the other, could have the effect of:

- changing the frequency of an INT +/- because the timing and number of concurrent FAST and SLOW going-Rich/Lean conditions may have been affected. Also, BPW correction could occur at a higher or lower rate.

- BLM changes being recalculated more or less frequently due to a change in the INT +/- frequency.

- Changing o2 Proportional Gain Count Table Values

There are three (3) o2-related Proportional Gain Count Tables. When viewing these Tables using an XDF file, the values might be displayed as seconds, milliseconds, loops, counts, etc. That’s just how the XDF is converting or describing the underlying Calibration hex value. It’s the hex values that matter. They are “Counts”. Each Count represents one 12.5ms pass through the Fuel Minor Loop.

Two of the Three SLOW o2 Proportional Gain Count Tables

The third Table is discussed below (see Third Prop Gain Count Table).

NOTE: These two Tables should not normally be changed but are discussed for completeness. They contribute to the Summed Prop Counts previously discussed.

The two Tables contain SLOW o2 Voltage Error-related Counts based on: (1) the size of the % Adjusted SLOW o2 Voltage Error, and (2) RPM:

- 0x4EA=SLOW o2 Proportional Step Size .vs. % Adjusted SLOW o2 Voltage Error
            (higher Error =higher Counts)

0mv Error =   4 Counts (50ms)

388mv Error =240 Counts (3000ms)

- 0x4D8=SLOW o2 Proportional Step Size .vs. RPM
            (higher RPM =lower Counts)

400 RPM =20 Counts (250ms)

2400 RPM =  2 Counts (25ms)

Counts from these two Tables are always summed; but they are only used if the % Adjusted SLOW o2 Voltage Error is <= 17mv, indicating the INT will not be +/-‘d and BPW will not be corrected. Independently, if a Cross-Count has not occurred, L00E9=# 12.5ms Fuel Minor Loop Passes without Cross-Count will have been incremented. As long as the % Adjusted Error continues to remain <= 17mv, Summed Prop Counts are compared to Passes through the Fuel Minor Loop without a Cross-Count to determine when the BPW will be corrected. This ensures a BPW correction will eventually occur. When > 17mv, the Rough BPW Correction will be computed using the Prop Gain Counts from the Third Table discussed below.

The following scenarios affect the Summed Prop Counts from the two Tables:

Scenario #1

If NO SLOW o2 Voltage Error

With no SLOW o2 error, the % Adjusted SLOW o2 Voltage Error =0. That means 4 Counts (50ms) will be returned by the % Adjusted SLOW o2 Voltage Error based Table. In this situation if RPM is <= 1600, Counts from the RPM-based Table will predominately determine the number of Summed Counts because RPM <= 1600 returns 8 to 20 Counts. Above 1600 RPM only 4 Counts are returned, and both RPM and % Adjusted SLOW o2 Voltage Error will equally contribute to the Summed Counts.

This suggests the RPM-based Table is for Idle/Low MPH situations where it will speed up a BPW correction as RPM drops.

Scenario #2:

Had a SLOW o2 Voltage Error ---BUT---
the % Adjusted SLOW o2 Voltage Error <= 17mv

(the INT is NOT +/-‘d)

Here, because % Adjusted SLOW o2 Voltage Error-based Counts are directly proportional to the size of the % Adjusted SLOW o2 Error, they will likely dominate. These Counts can range from 4 to 240, compared to RPM-based Counts ranging from 4 to 20.

Higher Summed Counts increase the frequency of BPW corrections but not their size. To achieve more frequent BPW corrections under the above Scenarios:

- Increase Counts at lower mv in the % Adjusted Voltage Error Table at 0x4EA

- Increase Counts at higher RPMs in the RPM Table at 0x4D8

Despite the above understanding, note in Appendix A that GM did not alter in the BCCs analyzed any values in the two SLOW o2 Prop Gain Count Tables. Given GM chose not to change them, changes should be made with caution.

Third Prop Gain Count Table

This Table provides Counts used to compute the Rough BPW Correction when the % Adjusted SLOW o2 Voltage Error is > 17mv, or when <= 17 and Summed Prop Counts indicate it’s time to correct the BPW:

0x4DE= CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error

This is a Table that GM chose to adjust in many of the Calibrations in Appendix A.

The Factory Table provides Counts in the range 4 to 12. The larger the % Adjusted SLOW o2 Voltage Error, the greater the adjustment to BPW due to larger Counts from the Table.

Bottom line, reducing values in this Third Prop Gain Count Table will reduce the magnitude of BPW corrections; increasing them increases the magnitude. If RPM surging is noticed at constant throttle, one might consider reducing these Prop Gain Counts to yield smaller BPW adjustments – the surging possibly being caused by a BPW that is fluctuating too rapidly.

- Changing the Frequency of INT +/-

The values in the following Tables can be changed to affect the frequency of an INT +/- regardless of whether Idling or NOT Idling:

0x504=INT Delay Factor .vs. % Adjusted SLOW o2 Voltage Error

0mv Error =1.0 Factor		388mv Error =0.25 Factor

0x4BD=SLOW o2 INT Delay Time .vs. Air Flow

8 gm/sec =600ms		64 gm/sec =150ms

If conditions required to +/- the INT are met, a value from each of the above Tables is multiplied to create what we’ll call the “Calculated # of Passes through the Fuel Minor Loop required before the INT can be +/-‘d”. As can be seen from these Tables:

- The larger the % Adjusted SLOW o2 Voltage Error, the smaller the factor from Table 0x504 and thus the product, resulting in more frequent INT changes.

- The higher the air flow, the shorter the time coefficient in Table 0x4BD and thus the product, resulting in more frequent INT changes.

Reducing the values in one or both Tables will result in more frequent INT changes, and increasing them will provide less frequent changes. The magnitude of the INT change is not affected because the INT is always only changed by 1 count. Like the possible symptoms from changing the Third Prop Gain Count Table discussed above, a more rapidly changing INT could cause surging at constant RPM.

If in an Idling/Low MPH state, the frequency of INT +/- can be additionally decreased by increasing the Scalar at 0x49A=’# 12.5ms Fuel Loops to Add Delay to INT Corr at IDLE’ from its default value of zero. This Scalar value is added to the above Calculated # of Passes before INT +/-. Each increment will delay +/- of the INT by 12.5ms. Reducing this value to reduce the frequency of INT +/- may also help big cam engines at idle.

Code Simulations

Code Simulation is performed by first assigning values to appropriate Calibration items which become input to the simulation. The code segment being simulated is then executed line-by-line in a debugger program to yield results. Calibration input values are changed as required and the simulation repeated to obtain results under varying conditions.

Three Code Simulations were performed on the Fuel Minor Loop. The purpose was to determine:

Simulation A - the effect on BPW with varying BLM and INT deviations from 128

Simulation B - the effect on BPW with varying MAT and Coolant Temperature

Simulation C – the effect of INT, BLM and o2 sensor voltages on BPW after reducing o2
                        Calibration Swing Points and Boundaries by 50mv

Simulation A - Effect of BLM and INT on BPW

Simulations show that a BLM deviating from 128 corrects BPW significantly more than the same INT deviation from 128. The examples below held all engine conditions constant except BLM and INT. The two Tables below assume BPW before adjustment =5.00ms:

BLM-based Adjustment - Effect on BPW

| BLM | % BLM Deviation from 128 | % BPW Correction | Adjusted BPW (ms) |
| --- | --- | --- | --- |
| 108 (BLM Min) | -15.6% | -15.8% | 4.21 |
| 128 | 0.0% | 0.0% | 5.00 |
| 160 (BLM Max) | +25.0% | +25.0% | 6.26 |

Note that a BLM-based BPW adjustment is percentage-wise the same as the BLMs deviation from 128.

INT-based Adjustment - Effect on BPW

An INT-based BPW adjustment is significantly less than a BLM-based adjustment. INT-based BPW adjustments approximately range from -6% to +16%, compared to BLM-based adjustments approximately ranging from -16% to +25%:

| INT | % INT Deviation from 128 | % BPW Correction | Adjusted BPW (ms) |
| --- | --- | --- | --- |
| 108 (INT Min) | -15.6% | - 6.0% | 4.70 |
| 128 | 0.0% | 0.0% | 5.00 |
| 160 (BLM Max) | +25.0% | + 9.8% | 5.49 |
| 180 (INT Max) | +40.6% | +16.0% | 5.80 |

As shown below, an INT-based BPW adjustment is approximately 39% of the INT’s deviation from 128, compared to a BLM-based BPW adjustment equaling 100% of the deviation from 128:

-  6.0% (% INT-based BPW Correction) / -15.6% (% INT Deviation from 128)  = 38.5%

+16.0% (% INT-based BPW Correction) / +40.6% (% INT Deviation from 128) = 39.4%

These results exclude the effect of the Rough BPW Correction that would increase/reduce the BPW correction in a range of 1 to 12 Prop Gain Counts because:

Rough BPW Correction = INT +/- Prop Gain Counts

BPW = BPW +/- ABS(Rough BPW Correction - 128)

This analysis indicates the importance of accurate VE Tables to avoid excess BPW correction.

Simulation B - Effect of MAT and Coolant Temperature on BPW

Code Simulation while cruising at a steady 2000 RPM under the conditions listed in Appendix B objectively revealed what we intuitively know:

- BPW decreases as MAT increases regardless of coolant temperature (mixture leaner at higher MAT)

- BPW decreases as coolant temperature increases regardless of MAT (mixture leaner at higher coolant temp)

The two Tables below summarize the results of Simulation B:

Table #1: BPW Decreases as MAT Increases (at constant Coolant Temp)

|  | Coolant Temperature (°F) |  |
| --- | --- | --- |
| MAT (°F) | 155 | 200 |
| 44 | 1.800ms | 1.740ms |
| 75 % BPW Change with MAT Increase (44 to 75°F) | 1.740ms -3.4% | 1.678ms -3.6% |
| 99 % BPW Change with MAT Increase (75 to 99°F) | 1.709ms -1.8% | 1.633ms -2.7% |
| 121 % BPW Change with MAT Increase (99 to 121°F) | 1.663ms -2.7% | 1.602ms -1.9% |
| 146 % BPW Change with MAT Increase (121 to 145°F) | 1.632ms -1.9% | 1.587ms -0.94% |

Observations:

- The rate BPW decreases as MAT rises slows as coolant temperature rises.

- The 2.7% reduction in BPW at coolant temperature of 155°F and MAT increasing from 99°F to 121°F is not an error despite it being out of line with decreasing values. It is due to the way L006D=BPW Scaling Factor is computed, which is a function of the coolant temperature/MAT differential (see How Switch 0x018 b4 Affects BPW below for details). L006D=BPW Scaling Factor is an important variable in the calculation of BPW.

Table #2: BPW Decreases as Coolant Temperature Increases (at constant MAT)

| MAT (°F) | BPW(ms) @ 155°F Coolant Temp | BPW(ms) @ 200°F Coolant Temp | % BPW Change with Coolant Temp Increase |
| --- | --- | --- | --- |
| 44 | 1.800 | 1.740 | -3.3% |
| 75 | 1.740 | 1.679 | -3.5% |
| 99 | 1.709 | 1.633 | -4.4% |
| 121 | 1.663 | 1.602 | -3.7% |
| 146 | 1.632 | 1.589 | -2.6% |

How Switch 0x018 b4 Affects BPW

This is a very important Switch because it significantly affects the calculation of the BPW Scaling Factor (L006D), which in turn significantly affects the BPW calculation.

All Code Simulations discussed in this document used the Factory AUJP default setting of Switch 0x018 b4=1 to compute BPW. When b4=1, values from the following Calibration Tables are used to compute the BPW Scaling Factor based on air flow and the coolant temperature/MAT differential:

- 0x7FD=Air Flow .vs. MAT or Coolant/MAT Differential

- 0x80E=COOLANT/MAT DELTA MULT .vs. AIR FLOW

When b4=0, these Tables are not used, and only MAT is used to compute the BPW Scaling Factor. The Table below provides BPW values with the Switch set and not set (again while cruising at 2000 RPM and coolant temperature =200°F, plus the other conditions shown in Appendix B).

Effect of Switch 0x018 b4 on BPW (Coolant =200°F, Air Flow =32 gm/sec)

|  | BPW (ms) |  |  |
| --- | --- | --- | --- |
| MAT (°F) | Factory Default Switch Set=1 | Switch Not Set=0 | Switch Not Set .vs. Set % BPW Change |
| 44 | 1.740 | 1.938 | +11.4% |
| 75 | 1.679 | 1.816 | +8.2% |
| 99 | 1.633 | 1.755 | +7.5% |
| 121 | 1.602 | 1.709 | +6.7% |
| 146 | 1.632 | 1.645 | +0.78% |

Observations:

- BPW is considerably richer at or below 121°F when only MAT is used to compute the BPW Scaling Factor (when b4 =0); versus using air flow to factor the coolant temperature/MAT differential (when b4 =1).

- Regardless of the Switch setting, BPW changes very little after MAT =121°F as it approaches 146°F.

- The above BPW changes at varying MAT are “virtually identical” at any coolant temperature because coolant temperature is not used in the calculation of BPW when b4=0. “Virtually identical” is used because coolant temperature is used to compute an INT update delay factor that could have a minor effect on BPW correction.

These results were also confirmed with Logs under varying MAT and coolant temperatures. This is why it’s important to minimize temperature fluctuations and always tune in a relatively constant MAT and coolant temperature range where normal engine operation occurs. Otherwise there will be unwanted deviations in INT and BLM values. The results also show that a more consistent tune can be achieved with a relocated MAT/IAT sensor that minimizes air temperature fluctuations, even with Switch 0x018 b4=1.

The Factory MAT sensor is actually a solid core coolant temperature sensor that is subject to heat-soaking and more prone to reporting the temperature of the TPI plenum than actual incoming air temperature. This is why replacing the factory MAT sensor with a true IAT sensor provides a more accurate MAT (for example, using a LT1 IAT sensor: AC Delco 213-243 [GM 12160244]). The relocation is best in the air intake as far from the engine and radiator as possible to ensure measuring true air temperature. Note that MAT relocation will require reduction in VE Table values due to the richer BPW.

If the MAT sensor is replaced and relocated, Switch 0x018 b4=0 could be considered. The theory being that the varying air-flow-related coolant temperature/MAT differential will be eliminated from affecting BPW, and the true intake air temperature will ensure MAT-based values from the VE Tables will produce a more consistent MAT-based BPW.

But setting b4=0 comes with plusses and minuses. When =0, different ambient temperature-based VE Table tunes might be required to match varying intake air temperatures. That could be a problematic where there are significant temperature fluctuations during a day. When =1, a consistent tune cannot be obtained until both coolant temperature and MAT have stabilized. And with the stock MAT sensor, that may take a while because the sensor will continue to report ever-increasing MAT as the temperature of the heat-soaked plenum continues to rise. Pick your poison understanding the limitations of each b4 setting.

If drag racing, setting b4=0 is likely the best option because tunes are guaranteed to provide a BPW directly related to IAT. For a street car, Simulations and Logs show that the best configuration is to set b4=1, replacing the factory MAT sensor with an IAT sensor and relocating it outside the engine compartment if possible. This ensures the coolant/IAT differential is relatively constant at engine operating temperature because the IAT will not be significantly changing. BLM values from multiple Logs also support these suggestions.

Simulation C – How Changing o2 Calibration Parameters Affects BPW

Code Simulations were conducted with the following o2 values:

- Factory:	Used GM Factory SLOW and FAST o2 Boundaries and Swing Points

- -50mv: 	All GM Factory o2 Boundaries and Swing Points were reduced by 50mv

Summarizing the corrected BPW results (before Fuel Out Routine compensations):

Both FAST and SLOW o2 going-Lean

|  | BPW (ms) | BPW % Change from Factory CL |
| --- | --- | --- |
| CL Factory | 2.853 | ---- |
| CL -50mv | 2.823 | -1.05% |
| OL | 2.716 | -4.80% |

Both FAST and SLOW o2 going-Rich

|  | BPW (ms) | BPW % Change from Factory CL |
| --- | --- | --- |
| CL Factory | 2.533 | ---- |
| CL -50mv | 2.502 | -1.22% |
| OL | 2.716 | +7.22% |

The -50mv BPW was 1.05% lower than the Factory BPW when going-Lean and 1.22% lower when going-Rich, indicating slightly more fuel was being removed with the 50ms lower o2 values than with Factory o2 values. This gain verifies that lowering Calibration o2 values leans BPW; increasing them richens BPW.

The parameters used in Simulation C and the detailed results are provided in Appendix B.

Summary of o2 Log Results

Four Logs were made using 4 different BINs. The purpose was to examine the effect of changing various o2 Calibration values.

Log Set #1

This first set of 2 Logs mirrored conditions used in Simulation C above:

r67 (Factory Log): The BIN contained Factory o2 Boundaries and Swing Points.

r67 -50 (-50mv Log): All Factory o2 Boundaries and Swing Points were reduced 50mv.

Log Set #2

The second set of 2 Logs varied o2 Calibration values in different ways:

r67 +50 Log: All Factory o2 Boundaries and Swing Points were increased 50mv

r67 450 +/-50 Log: All Swing Points were changed to 450mv; all Upper Boundaries set to 500mv (50mv above the Swing Point), and all Lower Boundaries set to 400mv (50mv below the Swing Point). This provided approximately the same spacing above and below the Swing Point as the Factory Calibration (45 Factory .vs. 50). The purpose was to force the FAST o2 voltage to swing around Stoich voltage of 450mv (Lambda =1.0).

The following o2 Calibration items were changed in the 4 BINs that were logged. Otherwise the BINs were identical:

Boundaries and Swing Points

|  | For SLOW Error | For FAST Error | For SLOW Error |
| --- | --- | --- | --- |
|  | LOWER Boundary | SWING POINT | UPPER Boundary |
| Idling/Low MPH (Scalars) Calibration Item: Factory: -50mv: | 0x498 566 400 | 0x496 601 450 | 0x497 601 500 |
| NOT Idling (Tables) Calibration Item: Factory Values(mv): MAP(kPa): -50mv Values(mv): MAP(kPa): | 0x4AB 530 583 469 20 40/50 100 All =400 At 20 to 100 | 0x4B4 574 627 521 20 40/50 100 All =450 At 20 to 100 | 0x4A2 619 672 553 20 40/50 100 All =500 At 20 to 100 |

A known good beginning Calibration that had BLMs fairly well dialed-in was used as a starting point. Every attempt was made to conduct all Logs under identical conditions. The same route and driving pattern was followed as closely as possible. The engine was in CL at coolant temperature of 205-210°F, with MAT at 110-120°F. Switch 0x018 b4=1 (AUJP Default – see discussion above about the implications of How Switch 0x018 b4 Affects BPW).

Log Results not discussed below are provided in Appendix D. The ‘r67 450 +/-50’ Log clearly produced a couple of things not seen in the other Logs:

- It provided the best overall FAST o2 voltage pattern and should be considered if CL is used where catalytic converters are not installed. The percentage of time FAST o2 voltage spent above and below 450mv was approximately 50% which is what was targeted. The Idle/Low MPH voltage distribution was 5% on the rich side likely due to the cam. This indicates there is room to move the Idle/Low MPH Swing Point to something less than 450mv in addition to reducing the Boundaries as suggested above in the discussion of big cam engines with the Voltage Error and Swing Point charts. There was also a noticeable smoothness of operation not experienced with the other Logs. All other Logs had a going-Rich/Lean distribution that was far from 50%/50%.

- Median o2mv voltage was significantly lower than that of the other Logs (on average, 48% lower). The Median voltage is the midpoint of FAST o2 voltages reported in all Log samples. While still on the rich side of 450mv at Idle/Low MPH, median voltage was slightly lower than 450mv when NOT Idling.

Interestingly, there were no significant identifiable differences in BLM or INT among the 4 Logs. While there were some minor differences, they were within a 4% margin of error and were ignored.

It should also be noted that AFR was not measured with a WBo2. That was not part of this study because it solely focused on the effect of NBo2 sensor input on BPW.

o2 Cross-Counts and Lack Thereof

Cross-Counts are important and indicate a properly functioning sensor, but equally important is the lack of Cross-Counts.

When a Cross-Count occurs:

  - L00FC=Cross-Counts is incremented for reporting in the ALDL Stream.

  - L003D b4 is set =1. This bit is only used in CL Field Service Mode to indicate an o2 sensor going-Rich/Lean condition (engine is running and ALDL connector Pins A&B are shorted). In CL Field Service Mode, the CEL is turned ON if FAST o2 voltage is going-Rich, OFF if going-Lean.

Outside of Field Service Mode in normal ECM operating mode, Cross-Counts are not used. But when a Cross-Count does not occur, L00E9=# 12.5ms Fuel Minor Passes without Cross-Count is incremented. This lack of Cross-Counts combined with Summed Prop Counts is a fail-safe that ensures BPW will eventually be corrected should the % Adjusted SLOW o2 Voltage Error remain <= 17mv for an extended period. Remember that as long as the % Adjusted SLOW o2 Voltage Error remains <= 17mv, the INT is not +/-‘d and BPW is not corrected. While in this condition, the continuously incremented L00E9=Passes Without a Cross-Count is compared with Summed Prop Counts to determine if it’s time to correct BPW. When Summed Prop Counts are >= Passes without a Cross-Count, Prop Gain Counts are determined and the Rough BPW Correction is calculated (see above 3. Changing o2 Proportional Gain Count Table Values).

Clearly there is a balance between Cross-Counts occurring and not. A properly functioning sensor provides that balance.

Code Flow - Fuel Minor Loop: Calculate and Correct BPW

This section presents the details of how BPW is calculated, how it is corrected in CL and how the INT and BLM are adjusted.

As the activities below are reviewed, keep in mind everything occurs in a single 12.5ms pass through the Fuel Minor Loop. That’s 80 times a second. The tendency may be to create a Log to observe real-time o2 Voltage Errors, FAST and SLOW o2 voltage changes, etc.; but that cannot be done. At 8192 baud (the 1227730 ECM’s approximate bit-per-second communication rate), the maximum output frequency of the 65 byte ALDL Mode 1 Stream is approximately 10 to 12 samples per second. That means activity occurring in 68 to 70 of the 80 passes through the Fuel Minor Loop in one second are not reported. Granted the sample output rate could be increased if the number of bytes reported is reduced, but it will never approach 80 samples per second.

For simplicity, calculations described here are as they would be performed using base 10 math. But in the code they are performed using base 16 hex math and values, which affects the precision because there are no fractional values except in division.

Fuel Minor Loop Preliminaries

Before voltage is obtained from the NBo2 sensor to begin the BPW calculation/correction process, the Fuel Minor Loop performs the following tasks irrespective of CL/OL status:

  - If the ignition has been off for less than 3.2 seconds, the ECM is kept-alive until 3.2 seconds have passed. This allows time for the IAC pintle to be fully opened and for certain values to be saved to non-volatile RAM for use at the next ECM power-on. After 3.2 key-off seconds, the ECM is powered-down.

  - The TPS sensor is tested for a low voltage Error 22, and the error bit is set if not previously set.

  - If the TCC is locked and a downshift has occurred, the TCC is unlocked if a change in TPS% has exceeded Calibration gear-dependent thresholds

  - If the engine is cranking, the needed BPW is obtained from Tables based on a combination of coolant temperature, MAT and barometric pressure, and the cranking BPW is factored based on TPS%. Using values from the Table at 0x3BD:

    - If TPS% is <= 87.5%, calculated BPW is reduced by 25%.

    - Above 87.5%, BPW is reduced to zero and no fuel is provided. This is “Clear Flood Mode” to resolve a flooded condition.

  - The Idle/Low MPH bit is set or cleared (L003E b2; 1=Idle/Low MPH, 0=NOT Idle). Used later for o2 Boundary and Swing Point determination.

  - The following are enabled if the required conditions are met, which will affect BPW (not covered):

    - Deceleration Enleanment

    - DFCO

    - DFCO Stall Saver

    - Enable, Continue or Disable MAP Acceleration Enrichment (MAP AE)

    - Power Enrichment (PE)

***** Begin the Fuel Minor Loop Code Flow at Label LC7AB: *****

Get o2 Sensor Voltage

- Save the NBo2 sensor voltage obtained 12.5ms ago to L00EA for later use.

- If S_AUJP v4/v5 and the appropriate WBo2 bits are set, read the WBo2 sensor and save the raw voltage or compute and save the AFR for the selected sensor.

- Read the NBo2 sensor and save this voltage to L008E=Current o2 Voltage for FAST Error.

- Lag Filter L008E voltage to create Current Filtered o2 Voltage for FAST o2 Error.

Compute an AFR

AFR is a “theoretical” value and a component of the BPW calculation. It is theoretical in the sense that it is either a predefined Stoich value if in CL, or a calculated value based on approximations if not in CL – meaning both can be incorrect. The CL AFR can be incorrect because the type or make-up of the fuel being used does not match the Calibration Stoich value. The OL AFR can be incorrect if the values used in its calculation are not optimal. The details of the calculations are provided below.

AFR is determined and saved to L00F1=AFR. AFR is irrespective of Idling or NOT Idling except OL NOT in PE AFR which is Idle/NOT Idle differentiated and also considers startup conditions. Despite this Idle/NOT Idle differentiation, there is actually no difference because the Idle/Low MPH Calibration value that would affect the outcome =0. AFR is determined for:

- PE

- CL Highway Fuel Mode

- OL NOT in PE

- CL

PE AFR:

    - A Lambda W (Lw) value is obtained from one of two Tables:

      - at 0x617 =Power Enrich Fuel/Air % Chg .vs. RPM, or

      - at 0xB3A =S_AUJP v4/v5 Extended PE Table (if 0x99E b7=1)

Table 0x617:				400 RPM=1.0313		6400 RPM=0.9766

Table 0xB3A (Extended PE):		400 RPM=1.0313		8000 RPM=0.9766

    - 128 is added to Lw (= Lw + 128)

    - (Lw + 128) is added to a Lambda C (Lc) value from the Table at 0x60D/E=Power Enrich Fuel/Air % Chg based on Coolant Temp
     Table 0x60D/E:	-40°F=54.6%		304°F=22.7%

This Table is indexed in the S_AUJP v4/v5 XDF on 0x60E, but the Table actually begins at 0x60D, which defines the number of rows in the Table minus 1. That’s why it’s referenced here as 0x60D/E.

    - The sum of the Lambdas and Stoich AFR are multiplied to yield PE AFR, which is stored in L00F1=AFR.

For the programmers: Given this math is done in hex, the rightmost and leftmost bytes are discarded after the 16 bit x 16 bit multiply. The result resides in the middle 2 bytes. For example:

Lw (@ 5000 RPM) + 128 = 0x7a + 0x80(128d) = 0xFA

Lc (@ 202°F) = 0x3A

(Lw+128) + Lc = 0xFA + 0x3A = 0x0134

0x0134 * 0x01BD (Stoich AFR of 14.7) = 0x00021764

PE AFR = 0x0217 = 535d (after discarding ‘00’ and ‘64’)

PE AFR = 6553.6 / 535 = 12.25

Looking at the GM Factory Lw and Lc Table values, Lc provides no change in PE AFR at or above coolant temperature of 133°F because all Lc values are the same (=0.227). So at coolant temperatures >= 133°F, the real change in PE AFR comes from RPM-based Lw.

PE AFR can be approximated using decimal values from the Lw Tables (0x617 or 0xB3A) and Lc (0x60D/E) Table as:
	PE AFR = Stoich AFR / (Lw + Lc)

Using the values in the above example, the PE AFR would be:

PE AFR = 14.7 / (0.9766 + 0.227) = 14.7 / 1.2036 = 12.21 (.vs. 12.25 above)

If for example Stoich AFR is changed to 14.2 for E10 fuel; under the same assumptions as above, the calculated AFR is 11.80 .vs. 12.25 (3.7% richer). This says the following should be done to maintain Lambda consistency if Calibration Stoich AFR is changed:

- The values in the Lw Table (at 0x617 or 0xB3A) should be percentage-adjusted so they produce the same calculated PE AFR as with Stoich AFR of 14.7

CL Highway Fuel Mode AFR (Leans AFR)

    - Highway Fuel Mode requires that all the following conditions be met:

in CL 								---AND---

kPa < 65		(Calibration item at 0x480)	---AND---

MPH > 53  		(Calibration item at 0x484)	---AND---

Coolant > 139°F	(Calibration item at 0x481)

    - If qualified and if NOT currently in Highway Fuel Mode

    - Wait 10 seconds before entering Highway Fuel Mode
(10 seconds =Calibration item at 0x482)

    - If qualified and currently IN Highway Fuel Mode

    - Must have been in Highway Fuel Mode < 60 seconds to continue it
(60 seconds =Calibration item at 0x483)

    - Once all the above conditions are met, an AFR value from the Highway AFR Table at 0x485 based on MAP is obtained:
     Table 0x485: 30 kPa=16.0 (highest Table AFR)		60 kPa=14.7 (lowest Table AFR)

    - After a lot of hex math (not covered), the extracted Table value becomes the Highway Fuel Mode AFR (14.7 to 16.0 depending upon MAP kPa). This value is stored in L00F1=AFR (details of how this occurs are not relevant).

Notes:

- If Stoich AFR at 0x41A is changed from 14.7 and Highway Mode Fuel is activated in the Calibration, the values in the Table at 0x485 should be adjusted by the percentage change in the Stoich AFR. This will ensure consistency of Highway Mode AFR based on the revised Stoich AFR.

- Highway Mode Fuel can be disabled by setting the MPH threshold value at 0x484 =255. This will cause the above tests to fail and ensure Highway Fuel Mode will not be considered.

OL NOT in PE AFR:

    - Sum the following 3 items:

    - If engine is starting or just started and in the start-up delay period, use various values stored in L000D from Start-up Enrichment Tables based on Coolant Temperature or MAT (if not engine startup, this value =0)

    - 0x45D Scalar =OL AFR % Change .vs. Coolant Temp
     0xDD =221d

    - A value stored in L00F0 from the Table at 0x45E=OL AFR % Change .vs. Coolant Temp
     -40°F through 153°F  =  +94.53% Change down to +14.8% Change
     154°F through 304°F  =  +  9.77% Change

    - If Idling, add the value in the Calibration item at 0x41F=’0% Open Loop AFR % Change At Idle’ to the above sum (S_AUJP v4/v5 =0)

    - The sum of the above items is temporarily stored in L00F1=AFR.

    - Get a value from the Table at 0x420=OL AFR % Change .vs. MAP

20 kPa = +2 Percent Change	85-100 kPa = +14 Percent Change

    - Add the temporary AFR value in L00F1 to this Table’s percentage change

    - Multiply the above sum and Stoich AFR to yield OL AFR, which is stored in L00F1=AFR

During startup and the startup delay period where the startup enrichment factor L000D is not zero, or when coolant temperature is < 68°F, the calculated OL AFR will be significantly richer than Stoich AFR (not covered).

After startup and at 200°F, Code Simulation shows OL AFR is calculated as shown in the Table below:

| MAP(kPa) | AFR |
| --- | --- |
| 30 | 15.3 |
| 40 | 15.2 |
| 50 | 14.7 |
| 60 | 14.3 |
| 80 | 13.6 |
| 100 | 13.4 |

So what about this AFR calculation?

    - This is not an absolutely-accurate AFR calculation because it does not take into account any engine conditions other than coolant temperature and MAP kPa.

    - It suggests that Cylinder Volume, Injector Flow Rate and particularly VE Tables had better be spot-on when in OL. Why? Because those values along with the calculated OL AFR will be used to compute the BPW fuel requirement – and there won’t be any o2-sensor-based BPW correction.

CL AFR:

    - No AFR calculation. Always =Stoich AFR at 0x41A (Factory =14.7:1).

Determine current BLM Cell Number and get BLM for ALDL Stream Reporting

- After computing AFR, a new BLM Cell number is calculated based on current RPM & MAP. The BLM in this cell is extracted and stored in ‘L00E0=BLM’ for ALDL Stream reporting. If the BLM cell changed since the last pass through the Fuel Minor Loop, L0046 b2=BLM Cell Changed is set =1 for later use.

Get Calibration Swing Point Voltage

- Determine applicable o2 Swing Point and save for later use:

Idling: 	0x496 Scalar (Constant 601mv)

NOT Idling:	0x4B4 Table (FAST o2 Rich/Lean Thresholds .vs. MAP):
                        20 kPa =574mv     40/50 kPa =672mv     100 kPa =521mv

Determine if o2 sensor voltage is adequate to indicate the sensor is Active

- Indicate the sensor is active if L008E=Current Filtered o2 Voltage for FAST Error is:

Above Upper Threshold: 0x48F =699mv

---OR---

Below Lower Threshold: 0x490 =199mv

An “Active” sensor does NOT indicate CL. It’s just an indication that the sensor is working properly by moving above or below an o2 sensor-is-active voltage boundary. CL is determined elsewhere in the code after engine run-time, coolant temperature and BLM Learn thresholds have been exceeded.

Set FAST o2 going-Rich/Lean (detects and records direction of sensor voltage)

- Set FAST o2 going-Rich (L0047 b6=1) if:

L008E=o2 Voltage > ‘FAST o2 Voltage Swing Point + 53mv’

---OR---

L008E=o2 Voltage <= ‘FAST o2 Voltage Swing Point + 53mv’	---BUT---

L008E=o2 Voltage >    FAST o2 Voltage 12.5ms Ago (Higher =going-Rich)

- Set FAST o2 going-Lean (L0047 b6=0) if:

L008E=o2 Voltage < ‘FAST o2 Voltage Swing Point - 53mv’

---OR---

L008E=o2 Voltage >= ‘FAST o2 Voltage Swing Point - 53mv’	---BUT---

L008E=o2 Voltage <=  FAST o2 Voltage 12.5ms Ago (Lower =going-Lean)

Check for a Cross-Count

- A Cross-Count is recorded anytime there has been a transition from FAST o2 going-Rich to Lean or vice-versa (the state of L0047 b6 has changed: 1=going-Rich; 0=going-Lean):

  - If the FAST o2 going-Rich/Lean status has changed:

    - Increment L00FC=o2 Cross-Counts

    - Set L003D b4=Had Cross-Count

Note: L003D b4 is only used during o2 sensor testing in ALDL Diagnostics Mode with ALDL Pins A&B shorted and the engine running to blink-out a CL going-Rich or going-Lean status; or if in OL, the CEL blinks 2.5 times a second.

    - Clear L00E9=# 12.5ms Fuel Minor Passes without Cross-Count

  - Else, if the FAST o2 going-Rich/Lean status has NOT changed:

    - Increment and save L00E9=# 12.5ms Fuel Minor Passes without Cross-Count

L00FC contains rolling cumulative o2 Cross-Counts in the ALDL Stream from 1 to 255 and then resets to 0 and not 1.

Continue below.

Check OL/CL State

- If IN OL

  - Go to LCA90: to force the INT to 128

- Else if

  - IN CL ---AND---

{

IN DFCO or IN PE

---OR---

NOT in DFCO, NOT in PE but IN Highway Fuel Mode

}

      - Go to LCA8D: to indicate the INT was forced to 128 while in CL

  - Else

    - Continue below

- Continue below

Check the status of INT related Switches

- If NOT in DFCO, PE or Highway Mode Fuel

  - If 0x018 b5=1 =Reset INT if BLM Changed (Default =0 =Don’t Reset)

    - Go to LCA90: to force the INT to 128

  - Else  if IN TPS and/or MAP Acceleration Enrichment (AE) ---AND---

0x018 b1=1 =Reset the INT if IN AE (Default =0 =Don’t Reset)

      - Go to LCA90: to force the INT to 128

  - Else  if CCP NOT Active

    - Go to LCA6F: CCP Not Active

  - Else if CCP Active ---AND---

0x019 b3=1 =Force the INT to 128 if CCP Active (Default =1 =Force to 128)

      - Go to LCA90: to force the INT to 128

  - Else, Continue below

- Else, Continue below

LCA6F: CCP Not Active

  - If In Decel Enleanment

    - Go to LCA81: to bypass kPa and RPM Tests

  - Else NOT in Decel Enleanment

    - If MAP kPa > 0

      - Go to LCA9F: to determine if there has been a SLOW o2 Voltage Error

    - Else if MAP kPa <= 0 (MAP sensor problem)

      - If RPM <= 1200

        - Go to LCA9F: to determine if there has been a SLOW o2 Voltage Error

      - Else if RPM > 1200 (continue below)

LCA81: Decel Enleanment Active-Bypass kPa and RPM Tests

        - If FAST o2 going-Lean AND the INT >= 128

          - Go to LCA8D: to indicate the INT was forced to 128 while in CL

        - Else if Fast o2 going-Rich OR going-Lean and the INT < 128

          - Go to LCA9F: to determine if there has been a SLOW o2 Voltage Error

-----------------------------------------

LCA8D: INT was forced to 128 while in CL

- Indicate the INT was forced to 128 while in CL (Set L0042 b2=1)

Continue below.

LCA90: Force the INT to 128

- Set L0046 b0=1 =‘Don’t Save INT to L00D2 for EGR Diagnostics’

- Force the INT (L00EB) to 128

- Clear L00EC=Prop Gain Counts (=0)

- Clear L00E6=CL INT Update Delay Timer (ensures the Timer is ready for the next time the INT is considered for +/-)

- Go to LCBE1: to bypass INT +/- and bypass calculation of the Rough BPW Correction

-----------------------------------------

LCA9F: Determine if there has been a SLOW o2 Voltage Error

- L0090=Filtered o2 Voltage for SLOW Error is compared to a SLOW o2 voltage boundary, possibly adjusted for AIR (see bullet below). If SLOW o2 voltage is above/below the SLOW Boundary, temporarily save the SLOW o2 Error and set a SLOW going-Rich or Lean condition:

L0047 b5=1 if SLOW o2 going-Rich

L0047 b5=0 if SLOW o2 going-Lean

- If AIR is being diverted and a BIN prior to S_AUJP v4 is being used, 100mv (Calibration item at 0x49D), is subtracted from the applicable Upper or Lower Boundary. S_AUJP v4/v5 changed the default of 0x49D from 100mv to 0mv so there is no reduction in a Boundary when AIR is being diverted. Without the 100mv reduction, the boundaries are considerably on the going-Rich (removing fuel) side of 450mv.

Continue below.

Compute % Adjusted SLOW o2 Voltage Error

- A SLOW o2 Voltage Error may be percentage-adjusted downward to arrive at a % Adjusted SLOW o2 Voltage Error:

  - If SLOW o2 voltage exceeded a SLOW o2 Boundary

    - IF SLOW o2 going-Rich (L0047 b5=1):

Default:	Reduce the SLOW Voltage Error by 12.5% to 87.5% of its

value using the Calibration item at 0x49F=87.5% SLOW

o2 going-Rich factor. If NOT Idling, this becomes the

NOT Idling % Adjusted SLOW Error.

Idling:	Reduce the SLOW Voltage Error to 55% of its original value by

applying 0x49B=62.5% Idling factor to the previously computed

Default 87.5% value above:

(Calibration item at 0x49F=0.875  x  0x49B=0.625 = 0.55)

    - If SLOW o2 going-Lean (L0047 b5=0):

NOT Idling:	No downward percentage reduction (Use 100% of the SLOW

Error)

Idling: 	Reduce the SLOW Error to 62.5% of its original value using

The Calibration item at 0x49B=62.5% SLOW o2 Idle factor

Summary of % Adjusted SLOW o2 Voltage Errors:

|  | % of SLOW o2 Error |  |
| --- | --- | --- |
| If SLOW o2: | NOT Idling | Idling |
| going-Rich | 87.5%(*) | 55.0%(*) |
| going-Lean | 100.0% | 62.5% |
| going-Neither Rich nor Lean | 0.0% | 0.0% |

(*) The % Adjusted SLOW o2 Voltage Error is always 12.5 percentage points higher
     going-Lean than going-Rich. This will provide larger BPW Corrections when
     going-Lean which will cause BPW to be slightly on the rich side due to more fuel
     added than removed when going-Rich.

  - Else if SLOW o2 voltage did NOT exceed a SLOW o2 Boundary

    - Set % Adjusted SLOW o2 Voltage Error =0

    - Go to LCB00 to compute Summed Prop Counts

As discussed in more detail below, if the % Adjusted SLOW o2 Voltage Error exceeds 17mv, the INT will be +/-‘d, Prop Gain Counts determined and the Rough BPW Correction computed. If going-Lean, it will be easier for the % Adjusted SLOW o2 Voltage Error to exceed 17mv than if going-Rich because the going-Lean % Adjusted Error is always 12.5 percentage points larger than if going-Rich. That means when going-Lean, the potential for INT +/- will occur more quickly along with a quicker change in voltage direction than when going-Rich. Also, % Adjusted SOW o2 Voltage Errors will be larger providing greater BPW corrections than when going-Rich. This will force a movement toward the rich side quicker than to the lean side when going-Rich.

Continue below.

L00B0: Compute Summed Prop Counts

- Get a value from the Table at 0x4EA=SLOW o2 Proportional Step Size .vs. % Adjusted SLOW o2 Voltage Error. The “Counts” in this Table are equivalent to 12.5ms passes through the Fuel Minor Loop:

Table 0x4EA:	  0mv Error =4 Counts (50ms)	388mv Error =240 Counts (3000ms)

- Get a value from Table at 0x4D8=SLOW o2 Proportional Step Size .vs. RPM. The “Counts” in this Table are also equivalent to 12.5ms passes through the Fuel Minor Loop:

Table Ox4D8:	  400 RPM =20 Counts (250ms)	2400 RPM =2 Counts (25ms)

- Sum the “Count” values from these two Tables to yield Summed Prop Counts. They represent the calculated number of 12.5ms passes through the Fuel Minor Loop needed to compute the Rough BPW Correction if the % Adjusted SLOW o2 Voltage Error remains <= 17mv for a period of time.

- If:

both FAST and SLOW o2 Voltages are going-Rich or Lean
(L0047 b5 & b6 both =1 or both =0)

---AND---

the % Adjusted SLOW o2 Voltage Error is > 17mv

    - Go to LCB3D: to determine if can +/- the INT

- Else if:

Only one o2 Voltage (SLOW or FAST) is going-Rich or Lean (not both)

---OR---

Both FAST and SLOW o2 Voltages are going-Rich or Lean  ---AND---

The % Adjusted SLOW o2 Voltage Error is <= 17mv

Continue below.

INT will NOT be +/-‘d:

Determine if the Rough BPW Correction will be computed

- Clear L00E6=CL INT Update Delay Timer (ensures the Timer is ready for the next time the INT is considered for +/-)

- If Summed Prop Counts < L00E9=# 12.5ms Fuel Minor Passes without Cross-Count:

  - Don’t determine Prop Gain Counts. This indicates the time (time =Counts =# passes through the Fuel Minor Loop) without a Cross-Count is not yet sufficient to warrant a BPW correction in excess of the INT’s deviation from 128.

    - Go to LCBAB: Don’t determine Prop Gain Counts

- Else if Summed Prop Counts >= L00E9=# 12.5ms Fuel Minor Passes without Cross-Count:

  - Determine Prop Gain Counts. This indicates sufficient time has passed to warrant a BPW Correction in excess of the INT’s deviation from 128 because too much time has passed without a Cross-Count.

    - Go to LCBAF: Determine Prop Gain Counts (L00EC)

-----------------------------------------

LCB3D: Determine if can +/- the INT (% Adjusted SLOW o2 Voltage Error > 17mv)

Determine the number of passes through the Fuel Minor Loop needed before the INT can be +/-‘d:

- A value is extracted from the Table at 0x504=INT Delay Factor based on the % Adjusted SLOW o2 Voltage Error:

| % Adjusted SLOW Voltage Error (mv) | INT Delay Factor |
| --- | --- |
| 0 to 106 | 1.0 |
| 107 to 141 | 1.00 to 0.50 |
| 142 to 176 | 0.49 to 0.25 |
| 177 to 388 | 0.24 to 0.00 |

After an INT delay factor is obtained from the Table, coolant temperature is inverted. The inverted coolant is then reduced by 90°F (equivalent to 176°F coolant temp). Depending upon the % Adjusted SLOW o2 Voltage Error and coolant temp, the factor can be upwardly adjusted as shown below (coolant temperature is used as opposed to inverted temperature to make the Table easier to read).

Possible INT Delay Factor % Change

|  | % Adjusted SLOW o2 Voltage Error (mv) |  |  |
| --- | --- | --- | --- |
| Coolant Temp | 0-141 | 141 | 283-388 |
| Factor if > 176°F | 100% | 50% | -40% |
| Factor if <= 176°F | 100% | 50% to 100%(a) | 25% to 87%(b) |

(a)  The Factor increases from 50% to 100% as temperature decreases from  176°F to 6°F.
Is 100% below 6°F

(b)  The Factor increases from 25% to 87% as temperature decreases from 176°F to -38°F.

- A time delay value based on air flow is extracted from the Table at 0x4BD=SLOW o2 INT Delay Time .vs. Air Flow:

| Air Flow (gm/sec) | Time Delay (ms) |
| --- | --- |
| 0 | 600 |
| 64 | 150 |

- The values of the 2 Tables (INT delay factor and delay time) are multiplied:

0x504(Factor, possibly inverted-coolant-temp-adjusted)  x  0x4BD(delay time in ms)

The resulting time is that which much pass before the INT can be +/-‘d. This time is equivalent to passes through the Fuel Minor Loop. Remember, in the Definitions we called the product of this multiplication the ‘Calculated # of Fuel Minor Loop passes needed to +/- the INT’.

A couple of observations for the ‘Calculated # of Fuel Minor Loop passes needed to +/- the INT’:

  - The INT Delay Factor gets smaller as % Adjusted SLOW o2 Voltage Error increases

(larger % Adjusted SLOW o2 Voltage Error = smaller factor = fewer Fuel Minor

Loop passes = quicker INT +/-)

  - As air flow increases, the delay time before INT +/- decreases

(higher air flow = less time = fewer Fuel Minor Loop passes = quicker INT +/-)

Continue below.

+/- the INT if it’s Time

This is the only place in the code the INT is changed. Remember from above the INT is only
+/-‘d if:

- both FAST and SLOW Rich/Lean Errors exist in the same direction  ---AND---

- the % Adjusted SLOW o2 Voltage Error is > 17mv

However, there are other conditions to be met before +/- the INT. The above ‘Calculated # of Fuel Minor Loop passes needed to +/- the INT’ is compared to L00E6=CL INT Update Delay Timer (actually a Counter of the number of passes through the Fuel Minor Loop):

- If L00E6=INT Delay Timer < Calculated # of Passes (# passes * 12.5ms per pass)

  - It’s NOT time to +/- the INT

  - Increment L00E6=INT Delay Timer

  - Go to LCBAF: Determine Prop Gain Counts (L00EC)
(the INT will NOT be +/-‘d)

- Else if L00E6=INT Delay Timer >= Calculated # of Passes

  - Enough time has passed: Eligible to +/- the INT

  - Clear L00E6=INT Delay Timer

  - If SLOW o2 going-Lean (L0047 b5=0)

    - If the INT =180 (Calibration item at 0x493 =Upper INT Limit)

      - Go to LCBAF: Determine Prop Gain Counts (L00EC)
(the INT will NOT be incremented)

    - Else if the INT <> 180

      - Increment the INT (increase fuel) and save to L00EB

      - Go to LCBAF: Determine Prop Gain Counts (L00EC)

  - Else if SLOW o2 going-Rich (L0047 b5=1), the process is more complicated.

Before diving into the going-Rich logic, need to list some terms. They are primarily used in the Spark Minor Loop for Closed Throttle Idle Spark Adjustment, but they also apply here under certain conditions when SLOW o2 is going-Rich:

    - SA LO Gain Desired RPM Threshold (Calibration item 0x1E6 =800 RPM)

    - Desired Idle RPM (L0112=Closed Throttle Idle based on Coolant Temperature)

    - Closed Throttle Idle SA UNDERSpeed mode (L003E b3=1) RPM < Desired Idle

    - Closed Throttle Idle SA OVERSpeed mode (L003E b3=0) RPM >= Desired Idle

    - Closed Throttle Idle SA LO Gain Mode (L0039 b3=1) Desired Idle >= 0x1E6

    - Closed Throttle Idle SA HI Gain Mode (L0045 b5=1) Desired Idle < 0x1E6

    - If the INT =80 (Calibration item at 0x492 =Lower INT Limit)

      - Go to LCBAF: Determine Prop Gain Counts (L00EC)
(the INT will NOT be decremented)

    - Else if the INT <> 80

      - If an o2 sensor Voltage Error 44 or 45 previously detected

        - Decrement the INT (reduce fuel) and save to L00EB

        - Go to LCBAF: Determine Prop Gain Counts (L00EC)

      - Else NO o2 sensor Voltage Error 44 or 45

        - If NOT in Closed Throttle Idle SA HI Gain Mode (L0045 b5=0)

(Implies IN LO Gain Mode)

          - Decrement the INT (reduce fuel) and save to L00EB

          - Go to LCBAF: Determine Prop Gain Counts (L00EC)

        - Else if IN Closed Throttle Idle SA HI Gain Mode (L0045 b5=1)

Closed Throttle Idle SA HI Gain Mode Notes:

- Regarding L00C8=’Err 45 Counter-Set Err if SLOW o2 > 699mv > 50 secs’ below: This counter will generally be =0 except under certain very unusual circumstances.

- Regarding going-Rich and Calibration item at 0x1E6=’Closed Throttle Idle SA LO Gain Threshold’ below, (default =800 RPM): 0x1E6 only comes into play affecting BPW when IN:

Closed Throttle Idle SA HI Gain Mode (Desired Idle RPM < 0x1E6)

---AND---

Closed Throttle Idle SA UNDERSpeed Mode (RPM < Desired Idle)

In this situation, which should rarely occur, there will be a delay in decrementing the INT. This delay will slightly prolong the going-Rich condition because BPW will not be reduced based on a reduction in the INT.

          - If In Closed Throttle Idle SA OVERSpeed Mode (L003E b3=0)

            - Decrement the INT (reduce fuel) and save to L00EB

            - Go to LCBAF: Determine Prop Gain Counts (L00EC)

          - Else if In Closed Throttle Idle SA UNDERSpeed Mode (L003E b3=1)

            - If L00C8=’Err 45 Counter (Set Err 44 if SLOW o2 > 699mv > 50 secs’) < 15 seconds (Calibration item at 0x1E5)

              - Go to LCBAF: Determine Prop Gain Counts (L00EC)
(the INT will NOT be decremented)

            - Else If L00C8=Err 45 Counter >= 15 seconds

              - Decrement the INT (reduce fuel) and save to L00EB

              - Go to LCBAF: Determine Prop Gain Counts (L00EC)

This process of determining if sufficient time has passed to +/- the INT repeats each time L00E6=CL INT Update Delay Timer is cleared.

-----------------------------------------

LCBAB: Don’t Determine Prop Gain Counts

Insufficient time has passed to warrant a BPW Correction in excess of the INT’s deviation from 128:

- Set L00EC=Prop Gain Counts =0

- Go to LCBD4: to determine the Rough BPW Correction

(Because Prop Gain Counts =0, the Rough BPW Correction will equal the INT)

-----------------------------------------

LCBAF: Determine Prop Gain Counts (L00EC)

Sufficient time has passed to warrant a BPW Correction in excess of the INT’s deviation from 128. The Counts determined here are used to define the magnitude of the Rough and Fine BPW Corrections (see LCBD4: below):

- Get Counts from the Table at 0x4DE=CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error:

| % Adjusted SLOW o2 Voltage Error(mv) | Counts |
| --- | --- |
| 0 through 176 | 4 |
| 177 through 247 | 5 to 11 |
| 248 through 388 | 12 |

The larger the % Adjusted SLOW o2 Voltage Error, the larger the Counts and the larger the BPW correction.

- Prop Gain Counts Determination:

  - If Idling

    - L00EC =25% of the Counts from the Table at 0x4DE (25% =Calibration item at 0x499)

Range = 1 to 3 Counts (equals 25% of Table 0x4DE value of 4 to 12)

  - If NOT Idling

    - Get a value from the Table at 0x4CF=NOT Idling Proportional Step Size Gain Multiplier based on air flow.

Values are all hex 0xFF (~= 100% factor = 1.0)

    - Multiply the 0x4CF Table value and that extracted from the Table at 0x4DE above. The product yields a range of 4 to 12 Prop Gain Counts (higher Counts at larger % Adjusted SLOW o2 Voltage Errors).

Note: Because all the values in the Table at 0x4CF are 100% (0xFF), the values in the Table at 0x4DE are used without change (4 to 12), and as a result represent the L00EC=Prop Gain Counts.

L00EC=Prop Gain Counts will =0 under the following conditions, which means BPW correction will limited to the INT’s deviation from 128:

    - In CL and the INT is or has been forced to 128

    - In OL where the INT =128

    - % Adjusted SLOW o2 Voltage Error is <= 17mv indicating the voltage error is too small to warrant a correction ---AND--- Summed Prop Counts < L00E9=# 12.5ms Fuel Minor Passes without Cross-Count. This indicates the time (# passes through the Fuel Minor Loop) without a Cross-Count is not yet sufficient to warrant a BPW correction in excess of the INT’s deviation from 128.

Continue below.

LCBD4: Compute L00E8=Rough BPW Correction

The Rough BPW Correction is the current INT (possibly +/-‘d above), plus or minus the L00EC=Prop Gain Counts determined above:

Rough BPW Correction = INT  +/-  Prop Gain Counts

(“+” if FAST o2 Voltage going-Lean; “-” if FAST o2 Voltage going-Rich)

- If insufficient time has passed to warrant a BPW Correction in excess of the INT’s deviation from 128, L00EC=Prop Gain Counts will =0 (see LCBAB: above)

The Rough BPW Correction will equal the INT and limit the BPW correction to the INT’s deviation from 128.

- Else if sufficient time has passed to warrant a BPW Correction in excess of the INT’s deviation from 128, L00EC=Prop Gain Counts will be >0

The Rough BPW Correction will increase the BPW correction beyond the INT’s deviation from 128 because it is added/subtracted to/from the INT.

Continuing:

- If FAST o2 is going-Lean (L0047 b6=0)

  - Go to LCBE1: to compute Rough BPW Correction: FAST o2 going-Lean

- Else, if FAST o2 is going-Rich (L0047 b6=1)

Compute the Rough BPW Correction with the previously computed Prop Gain Counts:

NOT Idling Adjustment:  	0 to 12 Counts

Idle/Low MPH Adjustment:	0 to 3 Counts (25% of NOT Idling Counts)

L00E8=Rough BPW Correction = INT  -minus-  L00EC Prop Gain Counts

  - Go to LCBE7: to get VE from Table

-----------------------------------------

LCBE1: (1) Bypass INT +/- and Bypass Calculation of the Rough BPW Correction

---OR---

(2) Compute the Rough BPW Correction: FAST o2 going-Lean

Here:

(1) The INT is or was forced to =128 and L00EC=Prop Gain Counts for =0
         (there will be no BPW correction)

---OR---

(2)  FAST o2 is going-Lean (L0047 b6=0), so compute the Rough BPW Correction:

Compute the Rough BPW Correction with the Prop Gain Counts below:

NOT Idling Adjustment:  	0 to 12 Counts

Idle/Low MPH Adjustment:	0 to 3 Counts (25% of NOT Idling Counts)

L00E8=Rough BPW Correction = INT  -plus-  L00EC Prop Gain Counts

Continue below.

LCBE7: Get VE from Table

- Get a VE value based on RPM and MAP from the appropriate Table:

Lower RPM Table at 0x6EB (if <= 1600 RPM)

Upper RPM Table at 0x797 (if >=1600 RPM up to 5600 RPM), or

S_AUJP v4/v5 Extended Table at 0x99D (1600 up to 6400 RPM) [if 0x99E b7=1]

- BARO-adjust the extracted VE value based on MAP (kPa) using the Table at 0x81F=VE Mult vs. BARO:

0 to 65 kPa = 1.188 Multiplier, dropping to 1.000 Multiplier at 105 kPa

- The BARO-adjusted VE is stored in L0067=BARO Adjusted VE and is reported in the ALDL Stream in S_AUJP v5. This adjustment richens the calculated BPW if < 100 kPa.

- If TPS < 1.17% and RPM <= 1200, The current BARO Adjusted VE (L0067) is Lag Filtered with its value 12.5ms Ago.

Calculate Speed Density BPW (L00E2)

After all the above, BPW is finally calculated:

- Multiply MAP, possibly reduced by EGR partial pressure, and the previously calculated L006D=BPW Scaling Factor (L006D is calculated in a separate routine 10 times per second).

As an aside, the setting of Switch 0x018 b4=’Use Air Flow for Coolant/MAT Diff Multiplier from Table L880E’ affects the BPW Scaling Factor as follows:

b4=1, (Default): Uses air flow and the Tables at 0x80E and 0x7FD

Air flow is first used to extract a Coolant/MAT differential multiplier from the Table at 0x80E. That multiplier is then used to factor the temperature differential for use in obtaining a representation of air flow from the Table at 0x7FD (the larger the temperature differential, the larger the air flow from the Table). This air flow is then used along with Cylinder Volume in the calculation of L006D=BPW Scaling Factor.

b4=0, Only MAT will be used without consideration of air flow or coolant temp

Using the default setting of 0x018 b4=1, regardless of coolant temp, produces the following:

    - At constant air flow, the BPW Scaling Factor decreases at MAT increases (BPW decreases)

    - At constant MAT, the BPW Scaling Factor increases as air flow increases (BPW increases)

When coolant temperature decreases:

    - the BPW Scaling Factor increases (BPW increased)

Bottom line, as MAT increases and the coolant temperature/MAT differential decreases, the BPW Scaling Factor decreases at any air flow, along with calculated BPW (= Leaner fuel mixture).

If interested, an explanation of the above-referenced Tables is provided here in Post #1: http://www.thirdgen.org/forums/diy-prom/265869-mat-pw-compensation-8d.html.

- Multiply the above product of MAP and BPW Scaling Factor, and BARO-adjusted VE, to yield a representation of Grams Air/Cylinder (L006F)

- Divide this representation of Grams Air/Cylinder by RPM represented by ECM firmware clock ticks to yield a representation of grams per second air flow based on the Ideal Gas Law. The result is stored in L006B.

This L006B air flow value is NOT used in the BPW calculation. It is stored for later use in obtaining values from various Tables based on ideal gas law air flow. Tables accessed:

0x4C6	SLOW o2 SLOW o2 Lag Filtering Coefficient (64 gm/sec max)

0x4BD	SLOW o2 INT Delay Time .vs. Air Flow (64 gm/sec max)

0x4CF	NOT Idling Proportional Step Size Gain Mult .vs. Air Flow (64d max)

- Multiply L006F=Grams Air/Cylinder and 0x41C=Injector Flow Rate to yield an Intermediate BPW

- Multiply the Intermediate BPW and the previously determined L00F1=AFR to yield L00E2=Speed Density BPW.

Correct BPW based on o2 sensor input

This is where the calculated Speed Density BPW is potentially corrected. Potentially corrected? Yes, o2-sensor-related BPW correction only occurs in CL, but this code is also executed if in OL. Appendix B provides the results of BPW calculation and subsequent corrections in both OL and CL under a simulated set of conditions.

The three possible CL BPW corrections are:

- Speed Density BPW is corrected based on the BLM’s deviation from 128 to arrive at what we’ll call the “BLM-adjusted BPW”. This BPW correction can be significant as shown in Appendix B.

- If the BLM < 128, BPW is reduced (leaned)

- Else if the BLM > 128, BPW is increased (richened)

- If the BLM =128, there is no BLM-based BPW change

- Next, the BLM-adjusted BPW is adjusted using the Rough BPW Correction. We’ll call the result the “Roughly-corrected BPW”:

Rough BPW Correction = INT +/- Prop Gain Counts

  - If the Rough BPW Correction <  128 (Rich)

    - BLM-adjusted BPW is reduced (leaned) by the value of the inverse (absolute value=ABS) of the Rough BPW Correction’s deviation from 128:
    BLM-adjusted BPW = (BLM-adjusted BPW)  minus  ABS(Rough BPW Correction - 128)

  - Else, if the Rough BPW Correction >= 128 (Lean)

    - BLM-adjusted BPW is increased (richened) by the absolute value of the Rough BPW Correction’s deviation from 128:
    BLM-adjusted BPW = (BLM-adjusted BPW)  plus  ABS(Rough BPW Correction - 128)

  - If the Rough BPW Correction =128 (as in OL, or in CL and the INT is or has been forced to 128)

    - BLM-adjusted BPW is not corrected

Remember, Prop Gain Counts can range from 0 to 12. If the % Adjusted SLOW Voltage Error is <= 17mv, Counts =0. This means the voltage error was too small to warrant an o2-sensor-related BPW correction. In this instance, the Rough BPW Correction =the INT:

Rough BPW Correction = INT +/- 0

This means the BPW correction here will be based only on the INT’s deviation from 128:

BPW = BPW +/- ABS(Rough BPW Correction[the INT] – 128)

When the % Adjusted SLOW Voltage Error is >= 17mv, the error is deemed large enough to warrant a BPW Correction using 1 to 12 Prop Gain Counts  (1 to 3 if Idling; 4 to 12 if NOT idling). These Counts are proportional to the magnitude of the % Adjusted SLOW Voltage Error >= 17mv. Higher counts result in a larger BPW correction.

Look at the following examples where Counts =0 and Counts =8. Assume a Roughly-corrected BPW =5.0ms =0x148, INT =130 (0x82) and SLOW o2 going-Lean:

Prop Gain Counts =0

Rough BPW Correction = INT(0x82) - Prop Gain Counts(0x00)

Rough BPW Correction = 0x82 – 0x00 = 0x82

BPW = BPW(0x148) +/- ABS(Rough BPW Correction(0x82) – 128[0x80])

BPW = 0x148 + ABS(0x82 - 0x80) = 0x148 + 0x02 = 0x14a (330d)

BPW = 330 * 0.015259 = 5.04ms (a 0.08% correction)

(Correction is solely based on the INT’s 2 point deviation from 128)

Prop Gain Counts =8

Rough BPW Correction = INT(0x82) - Prop Gain Counts(0x08)

Rough BPW Correction = 0x82 – 0x08 = 0x74

BPW = BPW(0x148) +/- ABS(Rough BPW Correction(0x74) – 128[0x80])

BPW = 0x148 + ABS(0x74 - 0x80) = 0x148 + 0x06 = 0x154 (340d)

BPW = 340 * 0.015259 = 5.19ms (a 3.8% correction)

(Correction is based on the INT’s 2 point deviation from 128 + the 8 Prop Gain Counts)

This Roughly-corrected BPW is stored back to L00E2=BPW.

- Apply an offset to the Roughly-corrected BPW to soften the effect of the Rough BPW Correction. This is the  Fine BPW Correction:

If the Roughly-corrected BPW =0

    - Go to the Fuel Out Routine to send NO fuel to the ECM (see Fuel Out Routine - Final BPW below).

Else if the Roughly-corrected BPW <> 0

    - Adjust the Roughly-corrected BPW in the opposite direction of the Rough BPW Correction using ½ of the L00EC=Prop Gain Counts (0 to 12):

          - If FAST o2 going-Lean (the Rough correction increased BPW)

Subtract from Roughly-corrected BPW, ½ of L00EC=Prop Gain Counts (range is 0 to 6, possibly +1). Roughly-corrected BPW is decreased.

      - If FAST o2 going-Rich (the Rough correction decreased BPW)

Add to Roughly-corrected BPW, ½ of L00EC=Prop Gain Counts (range is 0 to 6, possibly +1). Roughly-corrected BPW is increased.

The “possibly +1” above will occur if ½ of L00EC=Prop Gain Counts is an odd number (no fractions when dividing by 2 in hex), ---AND--- the injectors were just fired (L003E b0=1 =Injector Bookkeeping flip/flop bit).

We’ll call this the “Finely-corrected BPW”.

    - If IN Decel Enleanment, slightly decrease Finely-corrected BPW by a Decel Enleanment factor calculated earlier in the Fuel Minor Loop (not covered).

IMPORTANT NOTE: The Finely-corrected BPW is that reported in the ALDL Stream. This BPW does not include adjustments and compensations made in the Fuel Out Routine.

Continue to the Fuel Out Routine below.

Code Flow-Fuel Out Routine: Determine Final Fuel Out BPW

Control is transferred from the Fuel Minor Loop to the Fuel Out Routine which is executed every 6.25ms, or 160 times per second. This routine adjusts the Finely-corrected BPW for a number of conditions and then adds low PW and injector voltage offset compensations. Complete details are not provided before discussing the added compensations, but are summarized below for completeness:

Adjustments to Finely-corrected BPW

- Check MAP AE:

- If active

  - Determine the amount of any remaining MAP AE fuel to be delivered and add it to the Finely-corrected BPW. We’ll call this ‘Preliminary Fuel Out BPW #1’

- Else if MAP AE fuel requirement has been used up (fully decayed-out)

  - Turn off MAP AE

- Provide no fuel (BPW =0) if:

- VATS enabled and failed	---OR---

- Ignition is off		---OR---

- Ignition is on and IN DFCO

- Provide no fuel (BPW=0) if:

- DFCO not enabled		---AND---

- RPM > cutoff threshold (resume fuel calculations if RPM < return fuel threshold):

0x3F5=Fuel Cutoff: 0x0094 =0148d =6642 RPM (S_V2: 0x0062 =0098d =10031 RPM)

0x3F8=Fuel Resume: 0x0097 =0151d =6510 RPM (S_V2: 0x0067 =0103d = 9544 RPM)

- Check for TPS AE

- If TPS AE is active and/or TPS AE fuel is to be limited

  - Preliminary Fuel Out BPW #1 is reduced by a TPS AE coolant-based limiting factor

- Else if TPS AE is NOT active and/or TPS AE fuel is NOT to be limited:

  - There is no adjustment and Preliminary Fuel Out BPW #1 is used which includes applicable AE MAP adjustments

We’ll call the resulting BPW ‘Preliminary Fuel Out BPW #2’.

; [Here, L00E2=BPW from the Fuel Minor Loop is adjusted in several possible ways before adding BPW compensations:

;	1. If both of the bits below are set:

;		L0045 b1=AE MAP-Decay L00A5 if Time

;		L0045 b2=AE MAP-Time to Decay L00A5

;	   and if L00A5=AE MAP-Factor for BPW Adj or AE MAP Done > 0, then BPW from the Fuel Minor Loop is factored by

;	   the value in L00A5 to yield an AE MAP BPW adder, which when added to BPW from the Fuel Minor Loop yields a AE MAP-

;	   adjusted BPW. If L00A5 =0, L00E2 from the Fuel Minor Loop is unaffected.

;	2. If RPM exceeds fuel cutoff thresholds, L00E2=BPW from Fuel Minor Loop and L00F8=Accumulated Asynch BPW are set =0.

;	3. If AE TPS is Active, or L0045 b6=AE TPS-LIMIT Fuel Out PW Calculation is ON, or both; the AE MAP Adjusted

;	   BPW is factored by an AE TPS Coolant-based limiting factor from Table L8563. Then 2 BPW differences are calculated

;	   to determine the BPW subject to upward adjustments before being sent to the ECM. This means that the coolant-based

;	   BPW limiting factoring and differences are an adjustment for AE TPS and may result in a reduction of BPW because AE

;	   TPS is active and either just provided a pump shot, or will be providing a pump shot in Asynch Fuel Out executed

;	   immediately following this Fuel Out Routine. If AE TPS is OFF, there is no coolant-based limiting factoring, and

;	   the AE MAP adjusted BPW is used,]

- Determine if fuel is to be delivered in Single Fire or Double Fire mode by examining Preliminary Fuel Out BPW #2:

- Use Double Fire Mode if Preliminary Fuel Out BPW #2 > 1.1ms

=1x the PW, twice per Engine Cycle (one 1x firing per 360 crankshaft degrees)

- Use Single Fire Mode if Preliminary Fuel Out BPW #2 <= 0.854ms

=2x the PW, once per Engine Cycle (one 2x firing per 720 degrees)

Added Compensations

We’re now ready to consider the two added BPW compensations. They are provided to compensate for the injector latency (dead-time), where the injectors are not flowing at their theoretical linear flow rate, which occurs at opening, closing and at low BPW:

- Low PW Compensation: If Preliminary Fuel Out BPW #2 is less than a hard-coded 3.90ms, a ‘Low PW Compensation’ from the Table at 0x40B=Injector PW Correction if PW < 3.90ms (Low PW Offset) is added. Despite the Table providing compensation if the Preliminary Fuel Out BPW #3  is < 3.90ms, the GM Factory Calibration only provides compensation if BPW is < approximately 2.20ms because all values from 2.20ms to 3.90ms =0. The compensation is also quite small (in microseconds [uS]):

For BPW =2.20ms =  15.26uS (0.015ms)

For BPW =0.49ms =274.66uS (0.270ms)

We now have ‘Preliminary Fuel Out BPW #3’.

Aside: If injectors are changed from the Factory OEM versions, it may be desired to change the PW at which the low PW compensation is applied. Because this is not a Calibration item, a change can only be accomplished by modifying the hard-coded 3.90ms value directly in the BIN. In S_AUJP v5, the value is 0x0100 and is located at 0x6C3E/F when viewing the BIN in a hex editor. The BPW formula is:

0x0100 = 256d * 0.015259 = 3.90ms

- Voltage Offset Compensation: The final compensation is from the Table at 0x3FA=Injector PW Correction .vs. Fuel Pump or Ignition Switch Voltage. This Table contains the injector manufacturer’s voltage offsets. An offset PW resides in the variable L00EE=Injector BPW Correction for Voltage (uSec), having been previously extracted from the Table based on fuel pump or ignition switch voltage. This value is added twice to Preliminary Fuel Out BPW #3. Despite the Table PW units being microseconds, this can be a significant adjustment. For example:

at 12.8v, 1.59ms are added (793.47uS * 2 = 1586.94uS / 1000 = 1.59ms)

at 14.4v, 1.16ms are added (578.84uS * 2 = 1157.68uS / 1000 = 1.16ms)

See Appendix B, Fuel Out Routine Corrections area for the effect of these adjustments on BPW.

Note: Switch 0x016 b6 determines the voltage source used for Injector Voltage Offset Table lookup:

b6=1 =Use Voltage at the ignition switch (the ECM uses the voltage at the

ignition switch and not voltage directly at the battery)

b6=0 =Use Fuel Pump Voltage (default in all AUJP versions)

This resulting ‘Final Fuel Out BPW’ is sent to the ECM firmware to open the injectors for the specified time in milliseconds.

Return to the Fuel Minor Loop below.

Code Flow - Fuel Minor Loop: BLM Update

After the Final Fuel Out BPW has been sent to the ECM in the Fuel Out Routine, control is returned to the Fuel Minor Loop to consider updating the BLM. If updated, the BLM is always adjusted +/- 2 counts. Only the BLM in the current BLM Cell Number is updated.

A BLM update is considered every 50ms (every 4th pass through the Fuel Minor Loop), but a BLM update will only occur only if:

- BLM Learn is Enabled (L0046 b1 =1)					---AND---

- The INT differs from 128 by more than +5(Lean) or-5(Rich)	---AND---

(Calibration item for Lean BLM Threshold at 0x51F=5, and for Rich Threshold at 0x520=5)

- It’s been at least 1.0 second (S_AUJP v4/v5), or 1.4 seconds (prior to S_AUJP v4/v5), since the last BLM update (Calibration item at 0x51B).

Notes:

- If the BLM Cell Number just changed, BLM update will be delayed for consideration until the next pass through the Fuel Minor Loop, effectively delaying it 50ms.

- There is no change in the BLM if the INT does not differ from 128 by more than +/- 5.

- BLM Learn (L0046 b1=1) will only be enabled if to allow BLM update if:

- in CL							---AND---

- AFR =Stoich AFR as defined in the Scalar at 0x41A 	---AND---

- MAP kPa > 0

If any of the above conditions is not met, BLM update will not occur.

We won’t deal with all the nuances of the BLM update process. But here’s a summary of what occurs if the INT is more than 5 counts (4%) away from 128. Update the BLM by 2 counts if the INT is:

- Rich (from minimum allowed =108, to 122), or

- Lean (from 134 to maximum allowed =160):

| If INT | and FAST o2 | BLM Action |
| --- | --- | --- |
| > 128 (Lean) | going-Lean | Increase BLM 2 counts |
| > 128 (Lean) | going-Rich | No BLM Update |
| < 128 (Rich) | going-Rich | Decrease BLM 2 counts |
| < 128 (Rich) | going-Lean | No BLM Update |

The 2 count change to BLM is defined by the value in the Calibration Scalar at 0x51C

Finally, during the BLM update process:

- If CCP voltage =0  ---AND---  the current BLM Cell # =4

  - Cell #4 will be updated with the new BLM

- Else if CCP voltage >0

  - Cell #4 BLM update will be bypassed and remain unchanged

********** END CODE FLOW **********

Appendix A

o2 Calibration Differences: AUJP versus Other $8d 5.7L BINs

Calibrations Analyzed:

ANHT	90 Corvette TPI Auto with 3.07 (only minor o2 difference from AUJP-see Table 0x4DE below)

AXCS	91 Corvette TPI Auto with 2.73 (only minor o2 difference from AUJP-see Table 0x4DE below)

ARFP	90 Corvette TPI 6-Speed Manual with 2.59 (major o2 differences)

AXCN	91 Corvette TPI 6-Speed Manual with 2.73 (major o2 differences)

ANJF	90 F-Car TPI (o2 values Identical to AUJP)

AUJP	90-92 TPI “VIN 8” L98

The Calibration items below show the o2-related differences among the above BCCs. All are ‘NOT Idling’ values. There are no differences in Idle/Low MPH values.

Values in the “Diff” Columns below reflect AUJP values --minus-- other BCCs. A positive value indicates the AUJP value is greater than another BCC; negative = AUJP lower.

Table 0x4A2=SLOW o2 NOT Idling Rich/Lean UPPER Threshold .vs. MAP

|  | kPa | AUJP | AXCN | Diff |  | ARFP | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 | 619 | 619 | 0 |  | 619 | 0 |
|  | 30 | 654 | 654 | 0 |  | 654 | 0 |
|  | 40 | 672 | 672 | 0 |  | 672 | 0 |
|  | 50 | 672 | 583 | 89 |  | 583 | 89 |
|  | 60 | 654 | 583 | 71 |  | 583 | 71 |
|  | 70 | 636 | 583 | 53 |  | 583 | 53 |
|  | 80 | 583 | 583 | 0 |  | 583 | 0 |
|  | 90 | 566 | 583 | -17 |  | 583 | -17 |
|  | 100 | 553 | 583 | -30 |  | 583 | -30 |

- AXCN and ARFP are identical and have significantly lower SLOW o2 UPPER error boundaries at normal operating kPa (50-70)

Table 0x4AB=SLOW o2 NOT Idling Rich/Lean LOWER Threshold .vs. MAP

|  | kPa | AUJP | AXCN | Diff |  | ARFP | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 | 530 | 530 | 0 |  | 530 | 0 |
|  | 30 | 566 | 566 | 0 |  | 566 | 0 |
|  | 40 | 583 | 583 | 0 |  | 583 | 0 |
|  | 50 | 583 | 495 | 88 |  | 495 | 88 |
|  | 60 | 566 | 495 | 71 |  | 495 | 71 |
|  | 70 | 548 | 495 | 53 |  | 495 | 53 |
|  | 80 | 495 | 495 | 0 |  | 495 | 0 |
|  | 90 | 482 | 495 | -13 |  | 495 | -13 |
|  | 100 | 469 | 495 | -26 |  | 495 | -26 |

- AXCN and ARFP are identical and have significantly lower SLOW o2 UPPER error boundaries at normal operating kPa (50-70)

Table 0x4B4=FAST o2 NOT Idling Rich/Lean Threshold .vs. MAP (Swing Point)

|  | kPa | AUJP | AXCN | Diff |  | ARFP | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 | 574 | 574 | 0 |  | 574 | 0 |
|  | 30 | 609 | 609 | 0 |  | 609 | 0 |
|  | 40 | 627 | 627 | 0 |  | 627 | 0 |
|  | 50 | 627 | 539 | 88 |  | 539 | 88 |
|  | 60 | 609 | 539 | 70 |  | 539 | 70 |
|  | 70 | 592 | 539 | 53 |  | 539 | 53 |
|  | 80 | 539 | 539 | 0 |  | 539 | 0 |
|  | 90 | 530 | 539 | -9 |  | 539 | -9 |
|  | 100 | 521 | 539 | -18 |  | 539 | -18 |

- AXCN and ARFP are identical and have significantly lower SLOW o2 UPPER error boundaries at normal operating kPa (50-70)

Table 0x4BD=SLOW o2 Filter Coefficient (time in seconds) .vs. Air Flow (gm/sec)

|  | Air Flow | AUJP | AXCN | Diff |  | ARFP | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 0.600 | 0.400 | 0.200 |  | 0.400 | 0.200 |
|  | 16 | 0.550 | 0.350 | 0.200 |  | 0.350 | 0.200 |
|  | 24 | 0.300 | 0.350 | -0.050 |  | 0.350 | -0.050 |
|  | 32 | 0.275 | 0.350 | -0.075 |  | 0.350 | -0.075 |
|  | 40 | 0.250 | 0.300 | -0.050 |  | 0.300 | -0.050 |
|  | 48 | 0.225 | 0.250 | -0.025 |  | 0.250 | -0.025 |
|  | 56 | 0.200 | 0.200 | 0.000 |  | 0.200 | 0.000 |
|  | 64 | 0.175 | 0.200 | -0.025 |  | 0.200 | -0.025 |
|  | 64 | 0.150 | 0.200 | -0.050 |  | 0.200 | -0.050 |

- Increasing/decreasing values in this Table increases/decreases the time before the INT is +/-'d when there is no SLOW o2 Voltage Error. The smallest measured increment is 0.0125 seconds (12.5ms), which represents 1 pass through the Fuel Minor Loop.

- AUJP delays INT +/- at Air Flows <= 16 gm/sec, but accelerates it at Air Flows above 16 gm/sec.

Table 0x4DE=CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error (mv)

|  | mv Error | AUJP | AXCN | Diff |  | ARFP | Diff |  | AXCS | Diff |  | ANHT | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 4 | 3 | 1 |  | 3 | 1 |  | 4 | 0 |  | 4 | 0 |
|  | 35 | 4 | 3 | 1 |  | 3 | 1 |  | 4 | 0 |  | 4 | 0 |
|  | 70 | 4 | 3 | 1 |  | 3 | 1 |  | 4 | 0 |  | 4 | 0 |
|  | 106 | 4 | 2 | 2 |  | 2 | 2 |  | 4 | 0 |  | 4 | 0 |
|  | 141 | 4 | 2 | 2 |  | 2 | 2 |  | 4 | 0 |  | 4 | 0 |
|  | 176 | 4 | 2 | 2 |  | 2 | 2 |  | 4 | 0 |  | 4 | 0 |
|  | 212 | 8 | 3 | 5 |  | 3 | 5 |  | 4 | 4 |  | 4 | 4 |
|  | 247 | 12 | 4 | 8 |  | 4 | 8 |  | 4 | 8 |  | 4 | 8 |
|  | 282 | 12 | 4 | 8 |  | 4 | 8 |  | 6 | 6 |  | 6 | 6 |
|  | 318 | 12 | 4 | 8 |  | 4 | 8 |  | 8 | 4 |  | 8 | 4 |
|  | 353 | 12 | 4 | 8 |  | 4 | 8 |  | 10 | 2 |  | 10 | 2 |
|  | 388 | 12 | 4 | 8 |  | 4 | 8 |  | 16 | -4 |  | 16 | -4 |

- AXCS and ANHT had no o2-related Calibration differences from AUJP other than those shown in this Table.

- The larger this value, the larger the BPW correction. It is added/subtracted to/from the current INT to compute L00E8=Rough BPW Correction[Rough BPW Correction = INT  +/-  Prop Gain Counts].

- Except when the % Adjusted SLOW o2 Voltage Error =388mv, AUJP will provide larger BPW corrections than the other BCCs due to higher Prop Gain Counts.

Appendix B

Code Simulation C: BPW Correction Example

The Fuel Minor Loop code was simulated to document the calculation of BPW and the effect of o2 sensor voltage on BPW correction. BPW is also shown as calculated in OL. Only one pass through the Fuel Minor Loop was simulated because that’s all that is needed to compute and correct BPW.

Assumptions (the values used here represent cruising at 2000 RPM):

MAP:			45 kPa

Lower o2 Boundary:	642mv at 45 kPa

Upper o2 Boundary	583mv at 45 kPa

o2 Swing Point:	627mv at 45 kPa

Vacuum:		13.3 In Hg (45 kPa above converted to In Hg)

Stoich AFR:		14.2 (E10, 10% Ethanol Fuel))

Injector Flow Rate:	22.08 #/hr

RPM:			2000

TPS:			8%

VE from Table:	58 (0x94 =148d) (148 * 0.390625 = 58)

MAT:			120°F

Coolant Temp:	200°F

Air Flow:		32 gm/sec

Fuel Pump Voltage:	13.5v

L006D:		=0x00EF (This is the BPW Scaling Factor, derived from Air Flow, Coolant
 	Temperature/MAT Differential, and Cylinder Volume)

0x018 b4:	=1 (This is the Factory AUJP default. Air Flow is used to extract a Coolant
       temperature/MAT differential multiplier from the Table at
       0x80E=COOLANT/MAT DELTA MULT .vs. AIR FLOW, which is used in the
       computation of L006D=BPW Scaling Factor)

Two CL Scenarios were Simulated:

Scenario #1: Factory SLOW and FAST o2 both going-Lean

INT:							132 (+4 =Lean)

BLM:							130 (+2 =Lean)

L008E=Current o2 Voltage for FAST Error:		141mv

L0090=o2 Voltage for SLOW Error:			371mv

NOT Idling % Adjusted SLOW o2 Voltage Error:	212mv
(When going-Lean: 100% of 212mv SLOW Error is used)

212mv is the difference between L0090 SLOW voltage (371mv) and the Factory Lower SLOW o2 Boundary (159mv) to yield a 212mv % Adjusted o2 SLOW Voltage Error (212mv is used because it is an exact entry into the 0x4DE=CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error.

Scenario #2: Factory SLOW and FAST o2 both going-Rich

Here, the same INT and BLM distances from 128 were used for the Rich Condition as for the Lean conditions in Scenario #1:

INT:							124 (-4 =Rich)

BLM:							126 (-2 =Rich)

L008E=Current o2 Voltage for FAST Error:		950mv

L0090=o2 Voltage for SLOW Error:			914mv

NOT Idling % Adjusted SLOW o2 Voltage Error:	212mv
(When going-Rich: 87.5% of 242mv SLOW  Error =212mv)

242mv is the difference between L0090 SLOW voltage (914mv) and the Factory Upper SLOW o2 Boundary (672mv) to yield a 242mv Error. When 242mv is % Adjusted using the applicable 87.5% factor, the % Adjusted Error =212mv (again used because it’s an exact entry into the Prop Gain Count Table).

The following Calibration items were changed from Factory values to those shown to produce the -50mv BIN:

0x496=FAST o2 Idle/Low MPH Rich/Lean Threshold (Swing Point)

0x497=SLOW o2 Idle/Low MPH Rich/Lean UPPER Boundary

0x498=SLOW o2 Idle/Low MPH Rich/Lean LOWER Boundary

0x4A2=SLOW o2 NOT Idling Rich/Lean UPPER Threshold .vs. MAP

0x4AB=SLOW o2 NOT Idling Rich/Lean LOWER Threshold .vs. MAP

0x4B4=FAST o2 NOT Idling Rich/Lean Threshold .vs. MAP (Swing Point)

SUMMARY – BPW Calculation and Corrections

Below is a summary of BPW calculation and subsequent corrections for both the Factory and -50mv BINs. Raw BPW values shown are decimal representations of hex values.

SCENARIO #1: o2 going-Lean

|  | Factory |  |  | -50mv |  |  | OL |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BPW Raw / ms | BPW Chg / % Change | Cumulative % Change | BPW Raw / ms | BPW Chg / % Change | Cumulative % Change | BPW ms |
| Calculated Speed Density BPW | 178 / 2.716 |  |  | 178 / 2.716 |  |  | 2.716 |
| After BLM Correction | 180 / 2.747 | + 2 / + 1.1% | + 2 / + 1.1% | 180 / 2.747 | + 2 / + 1.1% | + 2 / + 1.1% | 2.716 |
| After Rough Correction | 192 / 2.930 | +12 / + 6.7% | +14 / + 7.9% | 188 / 2.869 | + 8 / + 4.4% | +10 / + 5.6% | 2.716 |
| After Fine Correction | 187 / 2.853 | - 5 / - 2.6% | + 9 / + 5.1% | 185 / 2.823 | - 3 / - 1.6% | + 7 / + 3.9% | 2.716 |
| After Low PW Compensation | 187 / 2.853 | 0 / 0.0% | + 9 / + 5.1% | 185 / 2.823 | 0 / 0.0% | + 7 / + 3.9% | 2.716 |
| After Injector Voltage Compensation (Final Fuel Out BPW) | 275 / 4.196 | +88 / +47.1% | +97 / +54.5% | 273 / 4.166 | +88 / +47.6% | +95 / +53.4% | 4.059 |

SCENARIO #2: o2 going-Rich

|  | Factory |  |  | -50mv |  |  | OL |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BPW Raw / ms | BPW Chg / % Change | Cumulative % Change | BPW Raw / ms | BPW Chg / % Change | Cumulative % Change | BPW ms |
| Calculated Speed Density BPW | 178 / 2.716 |  |  | 178 / 2.716 |  |  | 2.716 |
| After BLM Correction | 175 / 2.670 | - 3 / - 1.7% | - 3 / - 1.7% | 175 / 2.670 | - 3 / - 1.7% | - 3 / - 1.7% | 2.716 |
| After Rough Correction | 163 / 2.487 | -12 / - 6.9% | -15 / - 8.4% | 159 / 2.426 | -16 / - 9.1% | -19 / -10.7% | 2.716 |
| After Fine Correction | 166 / 2.533 | + 3 / + 1.8% | -12 / - 6.7% | 164 / 2.502 | + 5 / + 3.1% | -14 / - 7.9% | 2.716 |
| After Low PW Compensation | 166 / 2.533 | 0 / 0.0% | -12 / 0.0% | 164 / 2.502 | 0 / 0.0% | -14 / 0.0% | 2.716 |
| After Injector Voltage Compensation (Final Fuel Out BPW) | 254 / 3.876 | +88 / +53.0% | +76 / +42.7% | 252 / 3.845 | +88 / +53.7% | +74 / +41.6% | 4.059 |

- As expected when going-Lean, corrected BPW before Fuel Out compensations increased (Factory +5.1%; -50mv +3.9%). When going-Rich, it was expectedly decreased (Factory -6.7%; -50mv -7.9%). Note the greater percentage correction when going-Rich than going-Lean. Firstly, hex-calculated BLM Correction going-Lean was +2 versus -3 going-Rich, which resulted in a greater going-Rich correction. In theory, absolute BLM correction values should be identical because the lean and rich BLM equally deviated from 128 by 4 points. Secondly, the -50mv going-Rich Rough BPW Correction was significantly larger than all others due to the large % Adjusted SLOW o2 Voltage Error and associated Prop Gain Counts.

Rough BPW Correction = INT +/- Prop Gain Counts    ||    BPW = BPW +/- ABS(Rough BPW Correction – 128)    ||    (“+” if Lean; “-“ if Rich)

|  | Factory BPW before Rough Corr: L=180; R=175 |  |  | -50mv BPW before Rough Corr: L=180; R=175 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| going | % Adj Error / Prop Gain Cnts | Rough BPW Correction | Rough-corrected BPW | % Adj Error / Prop Gain Cnts | Rough BPW Correction | Rough-corrected BPW |
| Lean | 212 / 8 | 140=132+8 | 192=180+ABS(140-128) | 163 / 4 | 136=132+4 | 188=180+ABS(136-128) |
| Rich | 212 / 8 | 116=124-8 | 163=175-ABS(116-128) | 254 / 12 | 112=124-12 | 159=175-ABS(112-128) |

- Going-Lean CL Final Fuel Out BPW (both Factory and -50mv) was richer than OL by approximately 2.7%

- Going-Rich CL Final Fuel Out BPW (both Factory and -50mv) was leaner than OL by approximately 5.2%

DETAILED - BPW Calculation and Corrections

going-Lean:  Pg 1 of 3

The activities below are in the order of code flow for both the Factory Log, and where all Factory o2 Calibration values were reduced 50mv
(-50mv Log).

SCENARIO #1: SLOW and FAST o2 both going-Lean

Not in Decel Enleanment in this example.

Fuel Minor Loop

+/- the INT (L00EB)

|  | INT |
| --- | --- |
| Hex | 84 |
| Dec | 132 |

- The INT was not +/-‘d this pass through the Fuel Minor Loop because the L00E6=CL INT Update Delay Timer =5 counts (5 counts =wait 5 passes through the Fuel Minor Loop =62.5ms before +/- the INT).

Compute L00EC=Prop Gain Counts based on % Adjusted SLOW o2 Voltage Error from Table 0x4DE

|  | Factory | -50mv |
| --- | --- | --- |
| Hex | 8 | 4 |
| Dec | 8 | 4 |

- The -50mv BIN resulted in a reduction in the Prop Gain Counts from 8 to 4. This is because the -50mv % Adjusted SLOW o2 Voltage Error is lower than the Factory error (see Table below). Lower % Adjusted SLOW o2 Voltage Errors yield lower Prop Gain Counts.

|  | Factory | -50mv |
| --- | --- | --- |
| SLOW o2 Error | 212mv | 163mv |
| % Adjusted SLOW o2 Error | 212mv | 163mv |

Note when going-Lean and NOT Idling, there is no difference between the SLOW o2 Error and the % Adjusted Error because a there is no percentage adjustment.

Compute L00E8=Rough BPW Correction (When going-Lean, L00E8 = INT plus L00EC=Prop Gain Counts)

|  | Factory | -50mv |
| --- | --- | --- |
| Hex | 8C | 88 |
| Dec | 140 | 136 |

Factory:	Rough BPW Correction = INT(132) + Prop Gain Counts(8) = 140

-50mv:	Rough BPW Correction = INT(132) + Prop Gain Counts(4) = 136

Compute L00E2=Speed Density BPW											  going-Lean:  Pg 2 of 3

|  | Factory | -50mv | OL BPW |
| --- | --- | --- | --- |
| Hex | B2 | B2 | B2 |
| Dec | 178 | 178 | 178 |
| BPW (ms) | 2.716 | 2.716 | 2.716 |

The BPW values shown in the three Tables below are after the respective correction:

BPW Correction #1: Correct for L00E8=BLM

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | B4 |  |  |  | B4 |  |  |
| Dec | 180 |  |  |  | 180 |  |  |
| BPW (ms) | 2.747 | 1.1% | 1.1% |  | 2.747 | 1.1% | 1.1% |

BPW Correction #2: Correct with L00E8=Rough BPW Correction [BPW = BPW + ABS(Rough BPW Correction - 128)]

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C0 |  |  |  | BC |  |  |
| Dec | 192 |  |  |  | 188 |  |  |
| BPW (ms) | 2.930 | 6.7% | 7.9% |  | 2.069 | 4.4% | 5.6% |

BPW Correction #3: Correct with ½ L00E8=Prop Gain Counts (Fine BPW Correction)

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | BB |  |  |  | B9 |  |  |
| Dec | 187 |  |  |  | 185 |  |  |
| BPW (ms) | 2.853 | -2.6% | 5.1% |  | 2.823 | -1.6% | 3.9% |

This is the BPW reported in the ALDL Stream (Logs).

SUMMARY:

- Compared to Factory, reducing o2 Calibration values by 50mv when going-Lean reduced corrected BPW by 1.1% more than Factory:

(2.823(-50 Log BPW) – 2.853(Factory Log BPW)]  /  2.853(Factory Log BPW)) * 100

Reducing o2 values by 50mv from Factory produces a slightly leaner mixture.

- Before Fuel Out Routine compensations below, the calculated BPW was corrected upward (richened) based on o2 sensor input:

Factory: 	5.1%

-50mv:	3.9%

Fuel Out Routine Compensations											going-Lean:  Pg 3 of 3

Correct BPW if < 2.20ms using the Table at 0x40B=Injector PW Correction if PW < 3.90ms (Low PW Offset)

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction | OL BPW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Approx. Compensation (ms) | 0 |  |  |  | 0 |  |  | 0 |
| Hex | 0 |  |  |  | 0 |  |  | 0 |
| Dec | 0 |  |  |  | 0 |  |  | 0 |
| BPW (ms) | 2.853 | 0.0% | 5.1% |  | 2.823 | 0.0% | 3.9% | 2.716 |

Compensate for Voltage at 13.5v using the Table at 0x3FA=Injector PW Correction .vs. Fuel Pump or Ignition Switch Voltage

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction | OL BPW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 16 |  |  |  | 16 |  |  | 16 |
| Dec | 22 |  |  |  | 22 |  |  | 22 |
| Table Compensation (ms) | 0.671 |  |  |  | 0.671 |  |  | 0.671 |
| 2x Compensation (ms) | 1.343 |  |  |  | 1.343 |  |  | 1.343 |
| BPW (ms) | 4.196 | 47.1% | 54.5% |  | 4.166 | 47.6% | 53.4% | 4.059 |

- All AUJP versions use Fuel Pump voltage for BPW Compensation rather than voltage at the Ignition Switch (0x016 b6=0).

- The voltage compensation is added twice to BPW. Note this compensation increased the BPW by:

Factory:	47.1%

-50mv:	47.6%

OL:		49.4%

This major effect on BPW is why Voltage Offset Tables from the injector manufacturer are so important.

Going-Lean: Final Fuel Out BPW Sent to the ECM Firmware to Fire the Injectors

|  | Factory CL BPW | -50mv CL BPW | OL BPW | OL BPW versus Factory % Difference | OL BPW versus -50mv % Difference |
| --- | --- | --- | --- | --- | --- |
| BPW (ms) | 4.196 | 4.166 | 4.059 | -3.2% | -2.6% |
| Dec | 275 | 273 | 266 |  |  |
| Hex | 113 | 111 | 10A |  |  |

Without o2 sensor-related compensation, the OL BPW was leaner than CL:

Factory (CL):	3.2%

-50mv (CL):	2.6%

SCENARIO #2: SLOW and FAST o2 both going-Rich								  going-Rich:  Pg 1 of 3

Not in Decel Enleanment in this example.

Fuel Minor Loop

+/- the INT (L00EB)

|  | INT |
| --- | --- |
| Hex | 7C |
| Dec | 124 |

- The INT was not +/-‘d this pass through the Fuel Minor Loop because the L00E6=CL INT Update Delay Timer = 5 counts (5 counts = wait 5 passes through the Fuel Minor Loop = 62.5ms before +/- the INT).

Compute L00EC=Prop Gain Counts based on % Adjusted SLOW o2 Voltage Error from Table 0x4DE

|  | Factory | -50mv |
| --- | --- | --- |
| Hex | 8 | 0C |
| Dec | 8 | 12 |

- The -50mv BIN resulted in an increase in the Prop Gain Counts from 8 to 12. This is because the -50mv % Adjusted SLOW o2 Voltage Error is higher than Factory (see Table below). Higher % Adjusted SLOW o2 Voltage Errors yield higher Prop Gain Counts:

|  | Factory | -50mv |
| --- | --- | --- |
| SLOW o2 Error | 242mv | 291mv |
| % Adjusted SLOW o2 Error | 212mv | 254mv |

When going-Rich and NOT Idling, the % Adjusted SLOW o2 Voltage Error is 87.5% of the SLOW o2 Error.

Compute L00E8=Rough BPW Correction (When going-Rich, L00E8 = INT minus L00EC=Prop Gain Counts)

|  | Factory | -50mv |
| --- | --- | --- |
| Hex | 8C | 88 |
| Dec | 140 | 136 |

Factory:	Rough BPW Correction = INT(124) - Prop Gain Counts(8) = 116

-50mv:	Rough BPW Correction = INT(124) - Prop Gain Counts(12) = 112

Compute L00E2=Speed Density BPW											   going-Rich:  Pg 2 of 3

|  | Factory | -50mv | OL BPW |
| --- | --- | --- | --- |
| Hex | B2 | B2 | B2 |
| Dec | 178 | 178 | 178 |
| BPW (ms) | 2.716 | 2.716 | 2.716 |

The BPW values shown in the three Tables below are after the respective correction:

BPW Correction #1: Correct for (L00EB=BLM)

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | AF |  |  |  | AF |  |  |
| Dec | 175 |  |  |  | 175 |  |  |
| BPW (ms) | 2.670 | -1.7% | -1.7% |  | 2.670 | -1.7% | -1.7% |

BPW Correction #2: Correct with L00E8=Rough BPW Correction [BPW = BPW - ABS(Rough BPW Correction - 128)]

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A3 |  |  |  | 9F |  |  |
| Dec | 163 |  |  |  | 159 |  |  |
| BPW (ms) | 2.487 | -6.9% | -8.4% |  | 2.426 | -9.1% | -10.7 |

BPW Correction #3: Correct with ½ L00E8= Prop Gain Counts (Fine BPW Correction)

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A6 |  |  |  | A4 |  |  |
| Dec | 166 |  |  |  | 164 |  |  |
| BPW (ms) | 2.533 | 1.8% | -6.7% |  | 2.502 | 3.1% | -7.9% |

This is the BPW reported in the ALDL Stream (Logs).

SUMMARY:

- Compared to Factory, reducing o2 Calibration values by 50mv when going-Rich reduced corrected BPW by 1.2% more than Factory:

(2.502(-50 Log BPW) – 2.533(Factory Log BPW)]  /  2.533(Factory Log BPW)) * 100

- Reducing o2 values by 50mv from Factory produces a slightly leaner mixture.

- Before Fuel Out Routine compensations below, the calculated BPW was corrected downward (leaned) based on o2 sensor input:

Factory: 	6.7%

-50mv:	7.9%

Fuel Out Routine Compensations												   going-Rich:  Pg 3 of 3

Correct BPW if < 2.20ms using the Table at 0x40B=Injector PW Correction if PW < 3.90ms (Low PW Offset)

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction | OL BPW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Approx. Compensation (ms) | 0 |  |  |  | 0 |  |  | 0 |
| Hex | 0 |  |  |  | 0 |  |  | 0 |
| Dec | 0 |  |  |  | 0 |  |  | 0 |
| BPW (ms) | 2.533 | 0.0% | -6.7% |  | 2.502 | 0.0% | -7.9% | 2.716 |

Compensate for Voltage at 13.5v using the Table at 0x3FA=Injector PW Correction .vs. Fuel Pump or Ignition Switch Voltage

|  | Factory | This % Correction | Cumulative % Correction |  | -50mv | This % Correction | Cumulative % Correction | OL BPW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 16 |  |  |  | 16 |  |  | 16 |
| Dec | 22 |  |  |  | 22 |  |  | 22 |
| Table Compensation (ms) | 0.671 |  |  |  | 0.671 |  |  | 0.671 |
| 2x Compensation (ms) | 1.343 |  |  |  | 1.343 |  |  | 1.343 |
| BPW (ms) | 3.876 | 53.0% | 42.7% |  | 3.845 | 53.7% | 41.6% | 4.059 |

- All AUJP versions use Fuel Pump voltage for BPW Compensation rather than voltage at the Ignition Switch (0x016 b6=0).

- The voltage compensation is added twice to BPW. Note this compensation increased the BPW by:

Factory:	53.0%

-50mv:	53.7%

OL:		49.4%

This major effect on BPW is why Voltage Offset Tables from the injector manufacturer are so important.

Going-Rich: Final Fuel Out BPW Sent to the ECM Firmware to Fire the Injectors

|  | Factory CL BPW | -50mv CL BPW | OL BPW | OL BPW versus Factory % Difference | OL BPW versus -50mv % Difference |
| --- | --- | --- | --- | --- | --- |
| BPW (ms) | 3.876 | 3.845 | 4.059 | 4.7% | 5.6% |
| Dec | 254 | 252 | 266 |  |  |
| Hex | FE | FC | 10A |  |  |

Without o2 sensor-related compensation, the OL BPW is richer than CL:

Factory (CL):	4.7%

-50mv (CL):	5.6%

Appendix C

Visual Representation of Reading the o2 Sensor

and

FAST and SLOW Voltage Errors

The graph below visually shows how the setting of Boundaries and Swing Points determines the direction, frequency and magnitude of BPW corrections. Some assumptions are made for the graph:

- Cross-Counts are occurring at an average of 4 per second, or 1 FAST o2 voltage direction change (Cross-Count) every 0.25 seconds (250ms).

- The sensor is read every 12.5ms, so 20 times in 250ms in this example:

250ms  x  1 Sensor Read  =  20 Sensor Reads in 250ms

12.5ms

- The sensor is swinging uniformly between 100mv to 900mv before BPW corrections cause a change in voltage direction, for an 800mv swing. The 800mv will be traversed in 250ms. Doing the math shows this translates to one sensor read every 40mv of voltage movement:

800mv  =  3.2mv  x  ____12.5ms___  =  40mv per Sensor Read

250ms	        ms	per Sensor Read

Granted things are not exactly this precise, but some assumptions have to be made.

- Also for simplicity:

  - a linear rise and fall of voltage movement is used as opposed to a sine-wave

  - both FAST and SLOW voltages remain at equal values because lag-filtering of each is ignored. The impact is that FAST and SLOW voltages may be going in the same direction in this example for a few ms longer than actual.

- Finally, the example tracks Idle/Low MPH activity using the Factory Boundaries and Swing Point since they are singular values. We could pick any of the NOT Idling MAP kPa-based values and the procedure would be the same, but with slightly different results. Factory Idling/Low MPH values:

|  | Lower Boundary | Swing Point | Upper Boundary |
| --- | --- | --- | --- |
| Idling/Low MPH (Scalars) Calibration Item: Value (mv): | 0x498 566 | 0x496 601 | 0x497 601 |

Remember the Swing Point trigger for a FAST Error is the Swing Point +/- 53mv, so the triggers are:

|  | Swing Point Trigger (mv) |
| --- | --- |
| going-Rich: going-Lean: | 654 = 601 + 53 548 = 601 - 53 |

With the assumptions established, the graph below can now be interpreted to see what’s happening.

First, the scaling is purposefully enlarged to show more detail between 400 and 700mv where all the action occurs. In that range, each increment =10mv:

- Starting at 0 seconds and 100mv, and with both FAST and SLOW voltages going-Rich, it takes 14 sensor reads before there is a SLOW Error large enough, that when % Adjusted, is large enough to consider an INT +/- and additional BPW correction. The first FAST Error is on the 13th read where voltage of 660mv exceeds the 654mv Swing Point +53mv Trigger by 4mv.

Graph of o2 Sensor Reads and FAST and SLOW Voltage Errors

- From 660mv to 900mv, there are 6 more sensor reads. Each produces a FAST Error and a SLOW Error. When the SLOW Errors are % Adjusted, they are all > 17mv which ensures possible INT +/-, determination of Prop Gain Counts and computation of a Rough BPW Correction.

- At 900mv, FAST voltage changes direction from going-Rich to going-Lean, and the first Cross-Count is recorded (indicated by “CC #1”). Successive Cross-Counts are identified by “CC #n” (n =2-4).

- After CC #1 with o2 voltage now going-Lean, there is no significant activity above 540mv. But below 540mv, there are 11 going-Lean sensor reads which result in SLOW Errors large enough that when % Adjusted are > 17mv to begin consideration of an INT +/- and additional BPW correction. The first FAST Error occurs on sensor read 9, after which there are concurrent SLOW and FAST going-Lean conditions, resulting in  possible INT +/-, determination of Prop Gain Counts and computation of the Rough BPW Correction.

- Here’s the important point: With Factory o2 Boundaries and Swing Points, there are 7 going-Rich sensor reads (#14 to #20) that could result in fuel being removed, compared to 11 (#10 to #20) when going-Lean that could add fuel. Over time, this difference will tend to richen the BPW.
Each sensor read providing a SLOW voltage Error > 17mv produces larger and larger % Adjusted SLOW Errors which result in larger and larger BPW corrections, which ultimately cause a change in voltage direction.
If Boundaries and Swing Points are lowered, the BPW will be leaned and if raised, BPW will be richened. See Changing o2 Voltage Boundaries and Swing Points below for more detail.

Appendix D

Log Results Using Various o2 mv Values

The 4 Logs reported in this Appendix had o2 values changed as follows:

- r67 (Factory): Contained Factory o2 values (Swing Points and Boundaries)

- r67 -50: All Factory o2 values reduced 50mv

- r67 +50: All Factory o2 values increased 50mv

- r67 450 +/-50: All Swing Points changed to 450mv; all Upper Boundaries increased 50mv to 500mv, and all Lower Boundaries decreased 50mv to 400mv

| Coolant Temp | 201-208 |  | 203-208 |  | 203-210 |  | 187-208 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MAT | 98-118 |  | 92-106 |  | 108-118 |  | 86-124 |

| Log File Name | r67 |  |  | r67 -50 |  |  | r67 +50 |  |  | r67 450 +/-50 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description | Factory mv |  |  | Factory -50mv |  |  | Factory +50 |  |  | 450mv +/-50mv |  |
|  | Idle | NOT Idle |  | Idle | NOT Idle |  | Idle | NOT Idle |  | Idle | NOT Idle |
|  | (750 RPM) | (2000 RPM) |  | (750 RPM) | (2000 RPM) |  | (750 RPM) | (2000 RPM) |  | (750 RPM) | (2000 RPM) |

(A)

| BLM |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Avg | 129 | 129 |  | 128 | 122 |  | 130 | 129 |  | 128 | 122 |
| Min | 124 | 126 |  | 126 | 118 |  | 128 | 126 |  | 122 | 114 |
| Max | 130 | 132 |  | 128 | 128 |  | 130 | 132 |  | 134 | 130 |

| INT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Avg | 131 | 129 |  | 129 | 122 |  | 129 | 129 |  | 129 | 122 |
| Min | 121 | 111 |  | 123 | 112 |  | 125 | 122 |  | 124 | 114 |
| Max | 139 | 138 |  | 136 | 134 |  | 135 | 137 |  | 138 | 146 |

(A) Had surging at this constant RPM. Prop Gain Counts likely too high for -50mv o2 values.

THESE DATA ARE WITH AN OLD o2 SENSOR POSSIBLY RESULTING IN LOW CROSS-COUNTS

| Log File Name |  | r67 |  |  |  | r67 -50 |  |  |  |  | r67 +50 |  |  |  |  |  | r67 450 +/-50 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  | Factory mv |  |  |  | Factory -50mv |  |  |  |  | Factory +50 |  |  |  |  |  | 450mv +/-50mv |  |  |
| o2 mv |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Median (mid-point) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Idle/Low MPH | 840 |  |  |  |  | 800 |  |  |  |  |  | 831 |  |  |  |  | 674 |
|  | NOT Idle | 747 |  |  |  |  | 698 |  |  |  |  |  | 782 |  |  |  |  | 422 |
|  | All | 747 |  |  |  |  | 712 |  |  |  |  |  | 791 |  |  |  |  | 354 |
| Swing Points |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Idle/Low MPH | 636 |  |  |  |  | 570 |  |  |  |  | 692 |  |  |  |  |  | 491 |
|  | Not Idle | 581 |  |  |  |  | 514 |  |  |  |  | 674 |  |  |  |  |  | 421 |
| Cross-Counts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Idle/Low MPH (750-899 RPM) | 1.1 |  |  |  |  | 1.0 |  |  |  |  | 2.1 |  |  |  |  |  | 0.8 |
|  | NOT Idle (2000-2599 RPM) | 6.8 |  |  |  |  | 2.7 |  |  |  |  | 7.2 |  |  |  |  |  | 2.9 |
|  | All | 4.6 |  |  |  |  | 2.2 |  |  |  |  | 7.1 |  |  |  |  |  | 2.3 |
| % o2 mv Rich (> 450mv) / Lean (< 450mv) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Idle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | % Lean | 28.3 |  |  |  |  | 34.9 |  |  |  |  | 19.5 |  |  |  |  |  | 45.0 |
|  | % Rich | 71.7 |  |  |  |  | 65.1 |  |  |  |  | 80.5 |  |  |  |  |  | 55.0 |
|  | NOT Idle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | % Lean | 30.6 |  |  |  |  | 43.4 |  |  |  |  | 19.3 |  |  |  |  |  | 53.5 |
|  | % Rich | 69.4 |  |  |  |  | 56.6 |  |  |  |  | 80.7 |  |  |  |  |  | 46.5 |
|  | All |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | % Lean | 30.2 |  |  |  |  | 41.5 |  |  |  |  | 19.4 |  |  |  |  |  | 51.8 |
|  | % Rich | 69.8 |  |  |  |  | 58.5 |  |  |  |  | 80.6 |  |  |  |  |  | 48.2 |

Log Set #1 Results

Log Differences (Factory o2 values .vs. -50mv o2 values)

Reducing o2 Calibration values by 50mv provided no noted change in performance. There were also no perceptible BLM or INT differences, despite knowing from Simulation C above that reducing o2 values by 50mv reduces BPW.

Significant differences noted in the -50mv Log versus the Factory Log were:

- The percentage of time spent removing fuel in the -50mv Log was slightly greater than in the Factory Log. This is based on the state of INT and BLM values taken together above and/or below 128 (NOT based on their individual values +/- 128)

- Lower Average and Median FAST o2 voltage values in the -50mv Log .vs. Factory (Average =approximation of Swing Point; Median =mid-point)

- Lower Cross-Count frequency in the -50mv Log, indicating there were fewer changes in FAST o2 voltage direction. That also means there were fewer INT, BPW, and BLM changes.

Each of these differences is discussed below:

- Percentage of time spent removing fuel based on INT and BLM

The data below show the -50mv Log was removing fuel (reducing BPW) in 50%+ more samples than the Factory Log. This supports Simulation C above where lowering o2 mv values slightly leaned BPW. On a Log sample-by-sample basis, the conditions monitored to determine whether fuel was being added or removed were. In the Table below:

“% Rich” indicates “Removing Fuel” if:

INT < 128 & BLM < 128, or INT = 128 & BLM < 128, or INT < 128 & BLM = 128

“% Lean” indicates “Adding Fuel” if:

INT > 128 & BLM > 128, or INT = 128 & BLM > 128, or INT > 128 & BLM = 128

“Indeterminate” = Unknown if adding or removing fuel, and if no fuel change:

INT < 128 & BLM > 128, or INT > 128 & BLM < 128, or INT = 128 & BLM = 128

|  | Results |  |  |
| --- | --- | --- | --- |
|  | % Rich | % Lean | Indeterminate |
| -50mv Log | 81.9% | 13.1% | 5.0% |
| Factory Log | 29.5% | 66.0% | 4.5% |
| -50mv Log Difference | + 52.4% | - 52.9% | + 0.5% |

Details are provided in Appendix D.

- Lower Average FAST o2 mv

As expected, average FAST o2 voltages (approximation of Swing Points) were lower in the -50mv Log:

|  | Idling (mv) | NOT Idling (mv) |
| --- | --- | --- |
| -50mv Log | 570 | 514 |
| Factory Log | 636 | 581 |
| -50mv Log Difference | - 66 | - 67 |

- Lower Median FAST o2 mv

As expected, median voltages (midpoint of all values) were lower in the -50mv Log:

|  | Idling (mv) | NOT Idling (mv) |
| --- | --- | --- |
| -50mv Log | 800 | 698 |
| Factory Log | 840 | 747 |
| -50mv Log Difference | - 40 | - 49 |

Note the significantly rich bias (> 450mv) remained even after reducing values 50mv.

- Lower Cross-Count Frequency

At all RPMs, Cross-Counts were significantly less frequent in the -50mv Log than in the Factory Log indicating there were fewer FAST o2 going-Rich/Lean transitions. This was not expected, but resulted because the voltage pattern at -50mv o2 values was much smoother than at Factory o2 values. The Factory values produced an erratic voltage pattern providing more frequent FAST o2 direction changes. Cross-Counts averaged 4.6 per second in the Factory Log versus 2.2 per second in the -50mv Log -- a 52% reduction.

That means in the -50ms Log more time was spent in a Rich or Lean state without making a transition to the other state than in the Factory Log. This suggests o2 Calibration values should be set to produce the smoothest o2 pattern, which by its nature may reduce the number of Cross-Counts.

Log File Analyzer data excerpts are provided in Appendix D showing Cross-Counts for both Idling (750-899 RPM) and NOT Idling (2000-2599 RPM). The data were captured and reported in 5 second time slices and then sorted on RPM and Sample Min/Max RPM Difference. Summarizing Cross-Counts:

|  | Cross-Counts per Second |  |  |  |
| --- | --- | --- | --- | --- |
| RPM Range | -50mv | Factory | Difference | % Difference |
| 750-899 | 1.0 | 1.1 | - 0.1 | - 9.0% |
| 2000-2599 | 2.7 | 6.8 | - 4.1 | - 60.0% |
| All | 2.2 | 4.6 | - 2.4 | - 52.2% |

Log o2 Patterns

Below are the o2 mv patterns of the 4 Logs at:

- IDLE (750-800 RPM and NOT Moving

- NOT IDLE (2000 RPM and MOVING

Color scheme in the o2mv patterns below:

Red =o2 mv; Black =INT; Orange =BLM; Blue =RPM; Green =TPS%

Factory mv (r67)

Note the significantly irregular Idle and NOT Idle patterns.

IDLE (750-800 RPM)

NOT IDLE (2000 RPM)

Factory -50mv

These are good patterns like those of the 450mv +/- 50 Log below, but not as smooth. Note how the INT is slightly changing above and below a constant BLM.

IDLE (750-800 RPM)

NOT IDLE (2000 RPM)

Factory +50mv

Not the best o2 patterns. The Swing Point and Boundaries have a significantly Rich bias.

IDLE (750-800 RPM)

NOT IDLE (2000 RPM)

450mv Swing Points and +/-50mv Boundaries

These are the best o2 patterns of the 4 Logs. Note how the INT is slightly changing above and below a constant BLM.

IDLE (750-800 RPM)

NOT IDLE (2000 RPM)

Percentage of time spent removing fuel based on INT and BLM

The data presented below was obtained using Version 11 of the Log File Analyzer available here in Post #2: http://www.thirdgen.org/forums/diy-prom/716128-s_aujp-v5-log-file.html.

Factory Log

-50mv Log

Percentage of time FAST o2 voltage Rich/Lean

Factory Log

-50mv Log

Log File Analyzer o2 Cross-Count Data

o2 Cross-Count data are presented below for completeness and review but without comment other than that provided above in Logging o2 Tests, Log Set #1 Log Differences. The data were captured and reported in 5 second time slices and then sorted on RPM and Sample Min/Max RPM Difference.

Factory Log – IDLE

Factory Log – NOT IDLE

-50mv Log – IDLE

-50mv Log – NOT IDLE

Appendix E

S_AUJP v5 o2 Sensor and Fueling Related Calibration Values

All values are defaults of S_AUJP v4/v5.

Scalars

0x1E6:	.byte   0x40		; 800 RPM, [Closed Throttle Idle SA] LO GAIN DESIRED RPM THRESH [Arg x 12.5 = 800]

0x3F1:	.byte   0xF0		;=240, 0.937, 12.5ms FILTER COEFF

0x41A:	.byte   0x01,0xBD		;=0445,  STOICH AF RATIO 14.7  (ARG = 6553.6/RATIO) [AFR = 6553.6 / 445 = 14.7272]

0x41C:	.byte   0x01,0xCC  	;=0460, SEC/GRAM PROD OF INJ FLOW Rate  (0.359 SEC/GRAM, 2.786 g/Sec), 22#/HR

; ARG = ((SEC/GRAM * 256) * 5)

0x45D:	.byte   0xDD		;=221 SCALAR

L8480:      .byte   0x41   		;= 65 [No Conversion], DISABLE [Highway Fuel] IF KPa > [65] KPa

L8481:      .byte   0x85   		;=133, DISABLE HIGHWAY FUEL IF COOL <= 59.8C [139*F] (S_V2 =0xA7 ;=167d [185*F])

L8482:      .byte   0x0A   		;= 10, LOCK OUT HIGHWAY [Fuel] FOR 10 SEC

L8483:      .byte   0x3C   		;= 60, 60 SEC MAX TIME IN HIGHWAY [MODE FUEL]

L8484:      .byte   0x35   		;=53,  DISABLE HIGHWAY FUEL IF MPH =< 53 MPH  (S_V2 =0xff ;=255 MPH)

0x48F:	.byte   0x9E		;=158d, IF o2 < 699 mvDC THEN o2 IS READY

0x490:	.byte   0x2D		;=45d,  IF o2 > 199 mvDC THEN o2 IS READY

0x492:	.byte   0x50		;=80d,  CLS LP MIN INTEGRATOR VALUE

0x493:	.byte   0xB4		;=180d, CLS LP MAX INTEGRATOR VALUE

0x494:	.byte   0x06   		;=  6, 2.3% TPS MAX FOR [Low MPH/]IDLE

0x495:	.byte   0x30   		;= 48, 15 MPH MAX SPD FOR [Low MPH/]IDLE

0x496: 	.byte   0x88		;=136, 601 mvDC, FAST o2 Idle/Low MPH Rich/Lean Threshold (Swing Point)

0x497:	.byte   0x88		;=136, 601 mvDC, SLOW o2 Idle/Low MPH Rich/Lean UPPER Boundary

0x498:	.byte   0x80		;=128  566 mvDC, SLOW o2 Idle/Low MPH Rich/Lean LOWER Boundary

0x499:	.byte   0x40		;= 64, 25% MULT PROP STEP SIZE GAIN [for Idling], ARG = VAL*256

0x49A:	.byte   0x00   		;=  0, [# 12.5ms Fuel Loops to Add Delay to INT Corr at IDLE]

0x49B:	.byte   0xA0		;=160, 0.625 MULT TO CLS LP GAIN FACTOR [for Idling]

0x49C:	.byte   0x0C		;= 12, 053 mvDC, DIFF TO MAKE R/L WINDOW (FAST o2)

0x49D:	.byte   0x00		;= 00, 000 mvDC, DIFF R/L WHEN AIR DIVERTED (S_V2 =0x17 = 23 =100 mvDC)

0x49E:	.byte   0x04		;=  4, 17 mvDC, MIN [SLOW o2] ERROR TO MAKE INTEGRATOR CORRECTION

0x49F:	.byte   0xE0		;=224, 0.875 MULT GAIN FACTOR FOR POS ERRORS

0x4A0:	.byte   0x9A		;=154, 0.602 COEF FOR [SLOW] o2 FILTER CONSTANT

0x51B:	.byte   0x14		;= 20d, 1.0 sec, BLM update rate (Sec * 20) (S_V2 =0x1C =28 =1.4 sec)

0x51C:	.byte   0x02		;=  2d, BLM update amount [BLM changed 2 counts at a time]

0x51F:	.byte   0x05		;=  5d, If [INT] (Clsd Lp) GT 128+5 [Lean Threshold], enable BLM update

0x520:	.byte   0x05		;=  5d, If [INT] (Clsd Lp) LT 128-5 [Rich Threshold], enable BLM update

0x5A4:	.byte   0x00,0x80		; [AE TPS-A/C ON Pulse Width Adder for 1ST Time A/C ON (0x0080=128=1.95ms)]

0x5EC:	.byte   0x01,0x00		; [AE TPS-DFCO Stall Saver Pulse Width Adder (0x0100=3.90ms)]

Switches (Bit Flags)

0x018 b1 - Reset INT if in AE

The Factory default setting is to NOT reset the INT to 128 when in AE.

0x018 b4 - Use Air Flow for Coolant/MAT Diff Multiplier from Table L880E

The Factory default setting is to use air flow to extract a factor from the Table at 0x80E. That factor is used to adjust the coolant temp/MAT differential in the computation of L006D=BPW Scaling Factor.

0x018 b5 - Reset INT if BLM Changed

The Factory default setting is to NOT reset the INT to 128 when the BLM Cell changes.

0x019 b3 - Force INT to 128 if CCP Active (Y/N)

The Factory default setting is to reset the INT to 128 when CCP is active.

Tables

0x3FA Injector PW Correction .vs. Fuel Pump or Ignition Switch Voltage
           (Injector Voltage Offset)

A value from this Table is ALWAYS added twice to the BPW in the Fuel Out Routine based on Fuel Pump or Ignition Switch Voltage (the Factory default used in all AUJP versions is Fuel Pump voltage, 0x016 b6=0).

0x40B Injector PW Adjustment if PW < 3.90ms

In the Fuel Out Routine, if Preliminary Fuel Out BPW #2 is less than 3.90ms. a value from this Table is used to increase the BPW sent to the injectors. Note there is no BPW compensation as BPW begins to exceed 2.20ms. Preliminary Fuel Out BPW #2 includes any AE MAP fuel which has possibly been reduced if TPS AE is active.

0x420 OL AFR % Change .vs. MAP

A value from this Table is used to compute a Final OL AFR, along with the values below:

- Table at 0x45E=OL AFR % Change .vs. Coolant Temp

- Scalar at 0x45D=0xDD =221d.

0x45E OL AFR % Change .vs. Coolant Temp

A value from this Table is used to compute a Final OL AFR, along with the values below,:

- Table at 0x420=OL AFR % Change .vs. MAP

- Scalar at 0x45D=0xDD =221d.

0x4C6 SLOW o2 Lag Filtering Coefficient

A multiplier from this Table, along with the values below, is used to compute the smoothing factor used to Lag Filter L0090=Filtered o2 Voltage for SLOW Error decisions:

- L005D=Coolant Temperature

- Scalar Constant 0x4A0 =60.2% =Coefficient for SLOW o2 Filter Constant used to factor the coolant temperature

0x4A2 SLOW o2 NOT Idling Rich/Lean UPPER Threshold .vs. MAP

This Table provides the NOT Idling Upper SLOW o2 boundary based on MAP kPa used for determining if a SLOW o2 Voltage Error occurred. The variable L0090=Filtered o2 Voltage for SLOW Error is compared with a value from this Table for error determination.

0x4AB SLOW o2 NOT Idling Rich/Lean LOWER Threshold .vs. MAP

This Table provides the NOT Idling Lower SLOW o2 boundary based on MAP kPa used for determining if have a SLOW o2 Voltage Error. The variable L0090=Filtered o2 Voltage for SLOW Error is compared with a value from this Table for error determination.

0x4B4 FAST o2 NOT Idling Rich/Lean Threshold .vs. MAP (Swing Point)

This Table provides the NOT Idling FAST o2 Voltage Swing Point based on MAP used to determine if a FAST o2 Voltage Error occurred. The variable L008E=Current Filtered o2 Voltage for FAST Error is compared to a value from this Table, +/- 53mv (the ‘FAST o2 Swing Point +/- 53mv’), to make that determination.

0x4BD SLOW o2 INT Delay Time .vs. Air Flow

If conditions required to +/- the INT are met, a value from this Table and a value from the Table at 0x504=INT Delay Factor .vs. % Adjusted SLOW o2 Voltage Error, possibly coolant temperature adjusted are multiplied to create the “Calculated # of Passes through the Fuel Minor Loop required before the INT can be +/-‘d”. As can be seen from these Tables:

- The larger the % Adjusted SLOW o2 Voltage Error, the smaller the factor from Table 0x504 and thus the product, resulting in more frequent INT changes.

- The higher the air flow, the shorter the time coefficient in Table 0x4BD and thus the product, resulting in more frequent INT changes.

0x4CF NOT Idling Proportional Step Size Gain Multiplier .vs. Air Flow

A value from this Table, and that extracted from the Table at 0x4DE=CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error, are multiplied. That product provides a range of values 4 to 12. Because all the values in this Table are 100%, the values in the Table at 0x4DE are used without change.

0x4D8 SLOW o2 Proportional Step Size .vs. RPM

While the % Adjusted SLOW o2 Voltage Error is <= 17mv, a value from this Table is added to a value extracted from the Table at 0x4EA=SLOW o2 Proportional Step Size .vs. % Adjusted SLOW o2 Voltage Error to yield Summed Prop Counts. These Counts are based on RPM and the % Adjusted SLOW o2 Voltage Error. Summed Prop Counts represent the number of passes through the 12.5ms Fuel Minor Loop required before the INT can be +/-d and the Rough BPW Correction computed if there has not been a Cross-Count during the time period represented by the Summed Counts.

0x4DE CL o2 Proportional Gain Counts .vs. % Adjusted SLOW o2 Voltage Error

The value in the “o2 mv Error” column of this Table is the % Adjusted SLOW o2mv Voltage Error. A value from this Table, and that extracted from the Table at 0x4CF=NOT Idling Proportional Step Size Gain Multiplier .vs. Air Flow, are multiplied. That product provides a range of values 4 to 12 (larger values at higher % Adjusted Voltage Errors). Because all the values in Table 0x4CF are 100%, the values in this Table are used without change.

The “12.5ms Cnts” column of this Table represents L00EC=Prop Gain Counts. This is the value added/subtracted to/from the INT to create the Rough BPW Correction. Additionally, after BPW is corrected with the Rough BPW Correction, it is next adjusted in the opposite direction by ½ of the value from this Table. This is the Fine BPW Correction which somewhat offsets the Rough BPW Correction.

0x4EA SLOW o2 Proportional Step Size .vs. % Adjusted SLOW o2 Voltage Error

The value in the “o2 mv Error” column of this Table is the % Adjusted SLOW o2mv Voltage Error. While the % Adjusted SLOW o2 Voltage Error is <= 17mv, a value from this Table is added to a value extracted from the Table at 0x4D8=Proportional Step Size Gain .vs. RPM to yield Summed Prop Counts. These Summed Counts are based on both RPM and the % Adjusted SLOW o2 Voltage Error. Summed Prop Counts represent the number of passes through the 12.5ms Fuel Minor Loop required before the INT can be +/-d and the Rough BPW Correction computed if there has not been a Cross-Count during the time period represented by the Summed Counts.

0x504 INT Delay Factor .vs. % Adjusted SLOW o2 Voltage Error

A value extracted from this Table based on % Adjusted SLOW o2 Voltage Error, and a value from the Table at 0x4BD=SLOW o2 INT Delay Time .vs. Air Flow (possibly coolant temperature adjusted), are multiplied. That product is the “Calculated # of Passes through the Fuel Minor Loop’ required before the INT can be +/-‘d”. The calculated # of passes decreases as air flow and the % Adjusted SLOW o2 Voltage Error increase, providing more frequent INT updates.

0x617 Power Enrich AFR % Changed .vs. RPM (Lambda W)

0xB3A Power Enrich AFR % Changed .vs. RPM [Extended Table] (Lambda W)

A Lambda W value is obtained from one of these two Tables above based on RPM (6400 RPM Max Table or 8000 RPM Max Table). That value, plus 128, is added to a value from the Table at 0x60D/E=Power Enrich AFR % Change .vs. Coolant Temperature (Lambda C). The resulting value is used to calculate PE AFR.

0x60D/E Power Enrich AFR % Change .vs. Coolant Temperature (Lambda C)

A Lambda C value is obtained from this Table based on coolant temperature. It is added to a value from the Lambda W Table at (0x617=Power Enrich Fuel/Air % Chg .vs. RPM, or from S_AUJP v4/v5 Extended PE Table if 0x99E b7=1). 128 is added to the sum. The resulting value is used to calculate PE AFR.

This Table is indexed in the S_AUJP v5 XDF on 0x60E, but the Table actually begins in the code at 0x60D, which defines the number of rows in the Table minus 1.

0x7FD Air Flow .vs. MAT or Coolant/MAT Differential

A value from this Table is used along with Cylinder Volume in the computation of L006D=BPW Scaling Factor, a major variable in the computation of BPW.

0x81F VE Mult vs. BARO

VE will be adjusted upward using a value from this Table value if MAP < 105 kPa, with no change if MAP is 105+ kPa (WOT). The lower the MAP, the larger the upward BARO adjustment to VE.

0x80E COOLANT/MAT DELTA MULT .vs. AIR FLOW

If Switch 0x018 b4 =1, a multiplier from this Table is extracted based on air flow. The extracted value is used to factor the Coolant/MAT differential used to extract an air flow value from the Table at 0x7FD. The extracted air flow value is then used along with Cylinder Volume in the calculation of L006D=BPW Scaling Factor.

This Table is not used if 0x018 b4 =0.

Document Revisions

04/02/2017	Revision 1

The titles of the Tables below were changed to better describe their usage:

0x4BD

Prev:	SLOW o2 Filter Coefficient .vs. Air Flow

New:	SLOW o2 INT Delay Time .vs. Air Flow

0x4C6

Prev:	SLOW o2 Filter Coefficient Multiplier .vs. Air Flow

New:	SLOW o2 Lag Filtering Coefficient .vs. Air Flow

02/09/2017	Initial release

***** END of DOCUMENT *****