S_AUJP Version 6

Initial Release: December, 2019

FOR EDUCATIONAL PURPOSES ONLY -- USE AT YOUR OWN RISK

All material contained in and referenced herein is Copyrighted and may not be used in whole or in part without the author’s written permission,

and may not be used for any commercial purpose.

Copyright 2019 Tom Oliphint (84Elky, thirdgen.org) All Rights Reserved

Versions

This Version: 	S_AUJP v6, referred to as “v6”

Prior Version: 	S_AUJP v5 Rev D, referred to as “v5_D”

Initial Version: 	S_AUJP v4, referred to as “v4”

There are Standard and NVSRAM BINs with this Version:

(‘nn’ is the Development Sequence Number of the respective file)

S_AUJP v6-nn.BIN		Standard Version

S_AUJPN v6-nn.BIN	NVSRAM Version

The following TunerPro XDF and ADX files are provided:

(‘nn’ is the Development Sequence Number of the respective file)

S_AUJP v6-nn.XDF

S_AUJP v6-nn.ADX

S_AUJP v6-nn-Sensor Diagnostics.ADX

In the prior Version there were two ADX files; one for NBo2 and one for WBo2. They are combined in this Version.

Acknowledgements

Many thanks to those below for their invaluable contribution to this Version:

- 1Project2Many: For the now-available-all-in-one-place spark information for 4 and 6 cylinder MPFI Distributor and Distributorless engines. This elusive data is now available for the first time as a resource for all.

- BWilcox: For review of documentation and testing two WBo2 controllers independently reporting AFR to the ECM without the need for intermediary hardware channels.

- ULTM8Z: For testing WBo2 code efficiency revisions.

- JP86SS: For his insight and valued counsel.

YOU MUST DO THIS BEFORE ANYTHING ELSE

Transfer Your Calibration to the New S_AUJP BIN

*** Refer to Appendix A (hyperlink) ***

Miscellaneous Information

Calibration Items and RAM Variables

Calibration Items (indicated by ‘0xnnn’)

Scalars, Flags and Tables are identified by their hex address where:

‘nnn’ = the item’s relative hex location from the beginning of the Calibration

Calibration values referenced are S_AUJP defaults. The ‘0xnnn’ hex notation is used in addition to XDF Item Descriptions because the descriptions are not consistent among various XDF files.

RAM Variables (indicated by ‘Lnnnn’)

‘L’ indicates a variable

‘nnnn’ = the variable’s hex address in RAM.

Variables are used by the code to store the state of various conditions, sensor data and calculated values. Many, but not all, are reported in the ALDL Stream. The ‘nnnn’ address is used in a XDF ALDL Reporting Scalar to indicate the variable the ADX is to report.

ADX and XDF Files

Every effort has been made to ensure Titles and Descriptions of each XDF and ADX item is correct. Please post any suggestions for change.

FEATURES AND CHANGES

Each S_AUJP feature, revision and change is listed below. The links provide feature details, how to implement and use them, and the S_AUJP Version in which the feature was added:

Version 6

Launch Control (v5_D)

Launch Control Enhancements (v6)

WBo2 Support: AFR from Internally-Supported Controllers (v4)

Expanded WBo2 Support (v6)

Revisions:

Bypass TCC Coasting Unlock if NOT in DFCO (v5_D)

Default Setting for Bypassing TCC Coasting Unlock if NOT in DFCO

INT Update Delay if Low TPS & MPH

Corrections:

Lock/Don’t Lock BLM at 128 in PE

Transmission Selection

IAC Control

ALDL Reporting of TDC Spark Advance (SA-TDC)

Idle Spark Adjustment

ADX Reporting Additions:

Air Flow

VE% Adjusted for Barometric Pressure

Final PW After Compensations

Distance Traveled

WBo2 AFR Differences

Sensor #1 AFR .vs. Sensor #2 AFR

Sensor #1 and #2 AFR .vs. Commanded AFR

ECM Pins B3, F11 and F12 Support

Engine-off Sensor Diagnostics

Support for 4 and 6 Cylinder MPFI Engines

Appendix A - Transfer Your Calibration to the New S_AUJP BIN

Appendix B - WBo2 AFR Conversion Equations

Version v5_D

Shift Light	ALDL Data Stream Reporting Changes:

Enable Open Loop		New Status Word

Closed Throttle Spark Advance		Display Word (DISPWD) for Various Activities

U5 & U6 Chip Read		Barometric Pressure Adjusted VE%

Display Word (DISPWD) Address Relocation		MPG Calculation (Fuel Economy)

Knock-Detected Bit		Distance Travelled

Highway Mode Spark Advance Disable RPM		WBo2 AFR ALDL Change

Version 4

Light/LED Added	Fuel Cut-off/Re-enable

Extended VE Table to 6400 RPM	Highway Mode Fuel

Wide Band o2 Support	Open-to-Closed Loop Transition Changes

EGR Test Disabled	Flag Changes

RPM/31.25 to 7969	AE-TPS (Pump Shot)

PE Spark Advance Disabled	IAC and Throttle Follower

Low Octane Retard Disabled	ALDL Data Stream Reporting Changes

Forced Knock Test Disabled	ALDL Reporting Addresses

Charcoal Canister Purge Disabled	Custom Chip IDs

AIR Divert Disabled	Launch Control Spark Table

Fan(s) On/Off	Maximum In-Gear Idle RPM

TCC Lock/Unlock Changes	BLM RPM and MAP Cell Boundary Changes

BLM Idle Cell & Cell Boundaries	Other BLM Related Changes

Launch Control (LC)

Version 5_D

This is a limited implementation of LC that only controls spark advance. It does not provide selective cylinder misfiring combined with RPM limiting. But all things being equal, a car with this LC will outperform one without it.

Purpose: To limit wheel spin during a launch by reducing spark advance relative to Distributor Reference Pulses (SA-REF).

Management: LC is disabled by default. It is managed by five Calibration items:

Item	Default					Description

Flag    0x98E b0:	0			V4 OPT 1 (0x98E), b0=0, LC NOT Enabled; 1=Enabled

Flag    0x98E b1:	0 (A)			V4 OPT 1, b1=0, LC Spark Usage Time (0.2 Sec./Entry)

Scalar 0x998:	50% (B)      		LC, Activate/Continue if TPS% >=

Scalar 0x999:	2500 RPM (A)		LC, RPM Staged if RPM >=

Table  0xB4B:	None (A)		LC-SA-REF Degrees .vs. Elapsed Time

- LC Spark Table Entry #1 is used while RPM Staged, and at initial Launch.
When Flag 0x98E b1=0, Spark Advance will be provided for 0.2 seconds per Table entry and up to 3.2 total seconds, or until the last non-zero Table value has been used.
Alternatively, when =1, each Table spark value is used for 0.1 seconds with total Spark Table Advance provided for up to 1.6 seconds (see Spark Usage Time Control below for details).

- Default was changed from 40% to 50% in v6 to prevent undesired LC activation (v5_D was 40%) See TPS% Activation Threshold Increased below.

How to Use:

- To Enable LC: Set Flag OPT 1 (0x98E)  b0=1, Enable Launch Control (Y/N)

- Launch Period: The Launch Period begins at Launch Activation and extends until LC is Deactivated (see “Activation” and  “Deactivation” below)

- SA-REF: The LC Spark Table contains the SA-REF to be applied during the “Launch Period” (see “Spark Table” below).

- Staging: A car must be “Staged" before “Launch” can occur. A car is Staged in one of two ways:

Idle Staged:	MPH=0, in gear and idling normally (below the RPM Staged Threshold)

RPM Staged: 	MPH=0, in gear and RPM >= the RPM Staged Threshold

- Activation: While Staged, when TPS% is greater than or equal to the LC TPS% Threshold:

  - LC is “Activated” (Launch has begun and the “Launch Period” begins), and

  - SA-REF is provided by the LC Spark Advance Table

- Deactivation: After being Activated, LC is Deactivated if:

  - TPS% falls below the LC TPS% Threshold, or

  - All non-zero Spark Table entries have been used

- After Deactivation: Code-calculated SA-REF is used

Spark Table

If LC is Enabled and its use is intended, Entry #1 in the LC Spark Advance Table entry must be non-zero. Subsequent entries should gradually increase until it’s desired that LC Spark control cease. The smallest non-zero entry allowed is 0.35 degrees.

LC Spark Table included in S_AUJP Calibration

----------------------------------------VERY IMPORTANT--------------------------------------

- The values in the above Table ARE EXAMPLES ONLY!

It is the user's responsibility to determine and make the desired entries.

- LC Table spark advance entries are SA-REF, not SA-Top Dead Center (SA-TDC). For example, Entry #6 in the above table of 1.05 degrees is equivalent to 9.05 degrees SA-TDC advance if the Base Distributor setting is 8.00 degrees:

1.05[SA-REF] + 8.00[Base Distributor] = 9.05[SA-TDC]

DO NOT ENTER TDC SPARK ADVANCE!

----------------------------------------VERY IMPORTANT--------------------------------------

While Staged:  Spark Advance is provided:

If “Idle Staged”:	From the Closed Throttle Spark Table

If “RPM Staged”:	From the LC Spark Advance Table “Entry #1”

LC Spark Table Usage Example : Assuming use of the above LC Spark Advance Table values, SA-REF will be provided before and after Launch as shown below:

Spark If Idle Staged	  	Spark If RPM Staged

Before Launch ===>	  From Closed Throttle Table 	0.35 Deg (from LC Spark Table Entry #1)

After Launch: 		  ------------------From LC Spark Table ------------------

Entry #1 (0.1 or 0.2 secs)	     0.35 Deg

Entry #2 (0.1 or 0.2 secs)	     0.35 Deg

Entry #3 (0.1 or 0.2 secs)	     0.35 Deg

Entry #4 (0.1 or 0.2 secs)	     0.70 Deg

. . . .	       . . . .

etc., until the 0.00 degrees Spark entry is encountered at Entry #14. At that point, LC is deactivated and SA-REF returns to code-calculated values.

Logged LC Status: LC Status is reported in a Log as:

“STAGED” 	Reported while RPM Staged until Launch occurs

(Idle Staged is not reported)

“ACTIVE”	Reported when Launch occurs and until it is deactivated

To get LC Status reporting, the Status Word L00FF must be reported in the ADX (see New Status Word).

VERY IMPORTANT CAUTIONS ----- USE AT YOUR OWN RISK:

- Enter SA-REF, NOT SA-TDC values (SA-REF = SA-TDC – Base Distributor Advance).

- Be careful using RPM Staging for other than a short period of time because the engine and exhaust can get very hot with low spark advance at high RPMs.

- Make sure the LC TPS% Activate/Continue Threshold is set to a value well in excess of TPS% that might be encountered during a rapid, but not all-out acceleration from a complete stop. Otherwise, spark advance from the LC Spark Advance Table will be invoked if LC is Enabled. Default = 50% TPS

- At Launch, it takes a few milliseconds before the ECM registers MPH. If thresholds for “RPM Staged” and/or TPS% are set very low, it’s possible that mild acceleration can cause the RPM Threshold to be exceeded while MPH still =0. This will turn on RPM Staged and will cause the SA-REF in Entry #1 to be used. While in this state, if the LC TPS% Threshold is NOT exceeded, Launch will not be activated and the car will continue using the LC Spark Table Entry #1 value until RPM falls below the RPM Staged Threshold.

Under either of Cautions 3 or 4, sluggish performance will be experienced. It will not occur with properly set RPM Staged and TPS% Thresholds.

Launch Control Enhancements

Version v6

Spark Usage Time Control

The following Flag was added, replacing the no longer needed Flag: “Perform U6 Chip Read”:

V4 OPT 1 (0x98E),  b1, Launch Control Spark Usage Time

This Flag determines the time each Entry in the Launch Control Spark Table is used. The Spark Table can contain up to 16 non-zero SA-REF values. Each will be used for 0.2 seconds (200ms) or for 0.1 seconds (100ms) based on the setting of the Flag:

0 = 0.2 second per entry (Default)

1 = 0.1 second per entry (for cars that do not require lengthy retard)

TPS% Activation Threshold Increased

The Launch Control (LC) default activation threshold was increased from 40% to 50%. This change was made to prevent unintended LC activation when accelerating from a complete stop.

With this change, LC if enabled will be activated only when TPS% equals or exceeds 50%.

Wide Band o2 Support: AFR from Internally-Supported Controllers [Don Dibble, 1981TTA & Scott Lopez]

Version 4

WBo2 AFR can be computed directly by the S_AUJP code for the WBo2 controllers below (“internally-supported controllers”). Engine functions continue to be controlled by an installed NBo2 sensor.

0x99A (Selecting a bit 0 – 7 causes calculation of AFR)

WB OPT 1, b7 = Zeitronix Non-Linear

WB OPT 1, b6 = AEM Linear

WB OPT 1, b5 = Innovate LC-1&2 (1-2v)

WB OPT 1, b4 = PLX Linear

WB OPT 1, b3 = TechEdge 2.0 Linear

WB OPT 1, b2 = AEM Non-Linear

WB OPT 1, b1 = FJO Non-Linear

WB OPT 1, b0 = DIY_TechEdge Non-Linear

Setting a bit in 0x99A will cause a “raw” AFR to be computed for the selected controller. This raw value must be converted to AFR in the ADX using following Conversion Equation:

X / 10

0x99B WB OPT 2, b0 = RAW A/D Output

If b0 is set, WBo2 Raw Counts ONLY will be reported. It is the responsibility of the user to convert the Raw Counts to AFR in the ADX using a Conversion Equation other than the above (see Appendix A-WBo2 AFR Conversion Equations).

Expanded WBo2 Support [84Elky]

Version 6

The following additional WBo2 support has been added:

- For other than the 7 internally-supported controllers listed above

- For two WBo2 controllers independently and simultaneously reporting Raw Counts and/or AFR directly to the ECM without the need for intermediary hardware channels:

  - Separately report left and right WB AFR

  - Report AFR to the ECM from one controller, and AFR on a gauge from another controller

  - Report AFR from an internally-supported controller and AFR from a second controller

  - Report AFR from 2 controllers not internally-supported

What are “Raw Counts”? Raw Counts are computed from voltage returned by the sensor to reflect a value that fits within one byte of data. Within one byte of data, there can be 256 bits of data ranging from 0 to 255. Each bit equals 1 “Count”.

For example, a 0-5 volt linear controller has a Count range of 0 to 255. A sensor returning 2.5 volts would yield 128 Counts (both values being at the midpoint of the respective ranges):

5 volts             =   .0.01953125 volts per Count

256 Counts (0-255)

1 Count            x   2.5 volts = 128 Counts

0.01953125 volts

The supplied ADX file provides these Values for WBo2 reporting:

The following Flags and ALDL Reporting Scalar addresses are used for WBo2 reporting:

Flags

WB OPT 2 (0x99B),	b0 = 1=Get Sensor #1 Raw Counts

b1 = 1=Get Sensor #2 Raw Counts

b6 = Sensor #2 Pin (0=F14; 1=A3[Default])

b7 = Sensor #1 Pin (0=F14[Default]; 1=A3)

Selecting to obtain Raw Counts instructs the respective sensor to be read on the specified ECM Pin.

Addresses for use in XDF ALDL Reporting Scalars

01C5:	Reports Sensor #1 AFR from an internally-supported controller, or

Counts from other than an internally-supported controller.

Counts from other than an internally-supported controller are

used to compute AFR in the ADX.

0034: 	Reports Sensor #1 Counts from an internally-supported controller

01C2: 	Reports Sensor #2 Counts from other than an internally-supported

controller. These Counts are also used to compute AFR in the ADX.

Using the above Flags and Addresses, there are three options for reporting AFR and/or Raw Counts:

Option 1: From one internally-supported controller attached to Sensor #1

Option 2: From other than a single internally-supported controller attached to either Sensor

#1 or #2

Option 3: From 2 controllers, in any combination of internally-supported or

otherwise

Note that it is not necessary to specify an internally-supported controller to provide AFR. An alternate, and actually preferable, method is to specify getting Raw Counts. Those Counts can then be converted to AFR in the ADX using the Conversion Equations found in Appendix B - WBo2 AFR Conversion Equations. This method is preferable because both Counts and AFR can be reported using only one ALDL Reporting Scalar as opposed to requiring two Scalars if Counts are desired for an internally-supported controller as described below under Option 1.

Select Sensor ECM Pin

Regardless of the Option or number of sensors used, the ECM Pin to which a WBo2 controller is attached must be specified. The default Pins for each sensor are:

Pin F14:	Sensor #1

Pin A3:	Sensor #2

ECM Pins are selected with these Flags:

WB OPT 2 (0x99B), b6 = Sensor #2 Pin (0=F14; 1=A3[Default])

WB OPT 2 (0x99B), b7 = Sensor #1 Pin (0=F14[Default]; 1=A3)

The Pins can be swapped between Sensors 1 and 2, but the Pin selection in both Flags cannot be the same. This is intuitive because each sensor is attached to a different Pin, but the code cannot catch a Pin designation error. Should Pins be improperly designated, data for Sensor #2 will not be reported.

Summary of XDF Option settings:

| XDF ALDL Reporting Scalar (# = nn) | Scalar Value | ADX Value Reports |
| --- | --- | --- |
| Internally-Supported Controller Sensor #1 AFR (#37) Sensor #1 Counts (Need new Scalar) | 01C5 0034 | WBo2 AFR Sensor #1 WBo2 Counts Sensor #1 |
| Other than Internally-Supported Controller Sensor #1 Counts (#37) Sensor #1 AFR (#37) | 01C5 01C5 | WBo2 Counts Sensor #1 WBo2 AFR Sensor #1 |
| 2 Controllers Sensor #1 Use settings above for Sensor #1 depending upon whether internally-supported or otherwise Sensor #2 Counts (Need new Scalar) AFR (Same Count Scalar above) | 01C2 01C2 | WBo2 Counts Sensor #2 WBo2 AFR Sensor #2 |

The Options explanations below assume use of the supplied ADX file. If not using that file, the four WB-related Values it contains can be used as examples.

Option 1: Report AFR and/or Raw Counts using an Internally-Supported Controller

NOTE: An internally-supported controller will always be reported as Sensor #1 and must be connected to the Pin assigned to Sensor #1.

Report AFR

XDF

- Select an internally-supported controller to provide AFR using the XDF Flag:

WB OPT 1 (0x99A) bits 0-7 (see supported controllers above for bit selection)

- The ALDL Reporting Scalar #37 is set to report AFR only from address 01C5

ADX

- AFR will be reported from the ADX Value:

WBo2 AFR Sensor #1

- The computed raw AFR value must be converted to final AFR in the ADX Value “WBo2 AFR Sensor #1” using the conversion equation:

X / 10

- If a second sensor is NOT installed, the ADX Values below can be deleted. If not deleted, the descriptions will show in a log without data:

WBo2 AFR Sensor #2

WBo2 Counts Sensor #2

WBo2 Sensors: AFR Difference

Wbo2 AFR .vs. Commanded-Sensor #2

Report Raw Counts

Note “Counts”, NOT Voltage, are reported. To report Counts:

- Address "0034" must be entered into one of the 63 ALDL Reporting Scalars.

- Change the Scalar Title to reflect “Sensor #1 WB Raw Counts”

- Counts will be reported from ADX Value “WBo2 Counts Sensor #1”

- The decimal Packet Offset value in the must be one less than the number of the selected Reporting Scalar.

VERY IMPORTANT:

With the exception of “AEM Non-Linear”, AFRs for all internally-supported controllers are accurate when compared to the controller manufacturers’ data tables, graphs or conversion formulas. AEM Non-Linear is questionable and does not appear to provide accurate AFR. AEM says they have never produced a controller that outputs a Non-Linear AFR or Lambda to voltage curve. AEM states all their controllers have always provided linear output. Accordingly, use the AEM Non-Linear internally-supported controller AFR at your own risk.

Option 2: Report AFR and/or Raw Counts for Other Than an Internally-Supported Controller

XDF

- Select to obtain Sensor #1 Raw Counts in XDF Flag:

WB OPT 2 (0x99B),	b0 = Get Sensor #1 Raw Counts

- The ALDL Reporting Scalar #37 is set to report both Counts and AFR from address 01C5 for Sensor #1

ADX

- In the ADX Value below, enter the AFR Conversion Equation for the controller used (see Appendix B-WBo2 AFR Conversion Equations)

WBo2 AFR Sensor #1

- Raw Counts, NOT voltage, will be reported in the ADX Value:

WBo2 Counts Sensor #1

Option 3: Report AFR and/or Raw Counts from 2 Controllers

Again, thanks to BWilcox for his willingness to test dual WBo2 reporting. This was a significant test because it involved disconnecting controller output from external hardware (Moates AutoProm APU-1) and connecting that wiring to ECM Pins for direct reporting of Raw Counts and AFR.

If an Internally-Supported Controller is one of the two Controllers

  - Follow the steps under Option 1 above for Sensor #1

  - Then follow the steps below for Sensor #2

If Both Controllers are Other than Internally-Supported

XDF

  - Select to obtain Sensor #1 and #2 Raw Counts in XDF Flags:

WB OPT 2 (0x99B), b0 = Get Sensor #1 Raw Counts

WB OPT 2 (0x99B), b1 = Get Sensor #2 Raw Counts

  - To report Sensor #2 data, address "01C2" must be entered into one of the 63 ALDL Reporting Scalars.

  - Change the Scalar Title to reflect that “Sensor #2 WB AFR/Counts” are being reported

ADX

  - The ADX decimal Packet Offset value in the ADX Value “WBo2 AFR Sensor #2” must be one less than the number of the selected XDF Reporting Scalar.

  - In the ADX Values below, enter the AFR Conversion Equations for the controllers used (see Appendix B-WBo2 AFR Conversion Equations)

WBo2 AFR Sensor #1

WBo2 AFR Sensor #2

  - Raw Counts, NOT voltage, will be reported in the ADX Values:

WBo2 Counts Sensor #1

WBo2 Counts Sensor #2

Bypass TCC Coasting Unlock (TCC-BCU) if NOT in DFCO

Version 5_D

The default setting of this feature was changed in v6 (see below Revision to Default Setting for Bypassing TCC Coasting Unlock if NOT in DFCO).

Purpose: To provide TCC options if coasting and NOT in DFCO:

- To remain locked as long as possible (“bypassing” immediate unlock), or

- To immediately unlock the TCC

How to Use: TCC-BCU is enabled by setting the Flag:

Flag 0x98E b2 = V4 OPT 1 TCC-Bypass Coasting Unlock if NOT in DFCO (Y/N)

1=Yes: Bypass TCC Coasting Unlock (New v6 Default)

- The TCC will remain locked as long as possible to be later controlled by TPS% .vs. MPH or RPM Unlocking Tables:

LO Gear Table at 0x323

4th Gear Table at 0x33F

0=No: DON’T Bypass TCC Coasting Unlock (Default prior to v6)

- The TCC is immediately unlocked

Any combination of Flag V4 OPT 1 0x98E b2 and Flag Switch 18 (0x018) b3 is allowed. Flag: Switch 18 b3 controls TCC action while IN DFCO. These IN and OUT of DFCO bits can be used to provide the desired operation during coasting. For example:

Keep the TCC locked for as long as possible (S_AUJP Default):

Flag: Switch 18 (0x018) b3=0 =DON’T Unlock TCC if in DFCO

Flag: 0x98E b2=1 =Bypass TCC Coasting Unlock if NOT in DFCO

Unlock the TCC as quickly as possible:

Flag: Switch 18 (0x018) b3=1 =Immediately unlock if IN DFCO

Flag: 0x98E b2=0 =DON’T Bypass TCC Coasting Unlock if NOT in DFCO

Revision: Default Setting for Bypass TCC Coasting Unlock if NOT in DFCO

Version 6

In prior Versions, the S_AUJP default for Flag V4 OPT 1 0x98E b2=’TCC-Bypass Coasting Unlock if NOT in DFCO’ was =0. This meant that if coasting and NOT in DFCO, the TCC would be immediately unlocked. This action is inconsistent with the intent of S_AUJP, which is to remain locked as long as possible while coasting, regardless of whether in or out of DFCO. This is indicated by both the Factory AUJP and S_AUJP default settings of Flag Switch 18 b3=0 =DON’T Unlock if IN DFCO.

In this Version, the default setting of Flag 0x98E b2 was changed from 0 to 1 to be consistent with Switch 18 b3=0. Now the intent of remaining locked as long as possible is consistent both when in and when out of DFCO.

This change will have no effect on current AUJP and S_AUJP users using their existing Calibrations; but for unlocking consistency, a review of these settings should be made.

Revision: INT Update Delay if Low TPS & MPH

Version 6

The Scalar

0x49A	Loop Closed Param, Delay INT Update if Low TPS & MPH

allows delaying INT update while in the following state:

TPS <= 2.3% --AND-- MPH <= 15

The Factory default of zero means there will be no delay in updating the INT. A non-zero entry will delay INT update and is useful in instances where a rich and/or surging idle is experienced.

In prior Versions, the XDF Conversion for this Scalar was “X/10” and the Unit of Measure was “Seconds”. This caused no problem with the Factory Default of zero because the conversion caused zero to be entered in the Calibration. However, entry of a non-zero value resulted in 10 times the entered value being placed into the Calibration. For example, an entry of 1 would place 10 in the Calibration and provide an INT update delay of 125ms (10 * 12.5ms).

In this Version, the changes below were made to make the XDF Scalar more granular such that each entered “Count” delays INT update by 12.5ms:

- The Title was changed to: “Loop Closed Param, Delay INT Update if Low TPS & MPH”

- Conversion was changed from “X/10” to “X”

- The entry’s “units” were changed from: “ Seconds” to “# of 12.5ms Counts”

- ADX output was changed from 2 digits to 0 since the entry is now integer Counts and not fractional seconds

A non-zero value will now delay Low TPS & MPH INT update 12.5ms for each Count entered.

Correction: Lock/Don’t Lock BLM at 128 in PE

The following Flag was added in v4, but it’s usage was not documented:

V4 OPT 1 (0x98E), b6, Lock BLM to 128 in PE (1=Lock; 0=Don’t Lock)

This flag offered a change to the Factory AUJP code which always locked the BLM at 128 when PE was invoked.

This version modified the v4 logic to stop fuel from being continuously added if the BLM is lean when PE is invoked. Previously, a lean BLM at PE invocation would cause an unknown amount of fuel to be added for the duration of PE. A lean BLM always adds fuel, and the BLM cannot be changed in PE because the INT is also locked at 128 and the BLM cannot be changed if the INT=128.

Now, fuel is only added during the first 12.5ms of PE if the BLM is lean at PE invocation. This could be called a “PE Pump Shot”. This change was made because a lean BLM (above 128), can increase BPW up to 25% at BLM=160 (max BLM).

Example:

Assume at PE invocation a BPW of 5.0ms and a BLM of 134.

This would cause approximately 0.2ms of additional fuel for each BLM count above 128:

(134 – 128) * 0.2ms = 1.2ms additional fuel for duration of PE

Unlike AE-TPS and AE-MAP which can be controlled via Calibration settings, this fuel addition cannot be controlled so it was eliminated.

Here’s how this bit now works if elect to NOT lock BLM=128 in PE (b6=0):

If BLM Rich or Neutral (<= 128)

Lock BLM = 128

Else if BLM Lean (> 128)

If first time BLM Lean in PE

Don’t change BLM and allow BLM to adjust BPW to add fuel one time

Else if not first time

Lock BLM = 128

Correction: Transmission Selection

Version 6

Transmission identification is important for many tasks including, but not limited to:

- initial IAC positioning, and

- defining when and how IAC steps are later adjusted while running

See also Correction: IAC Control below.

Factory AUJP and S_AUJP use two Calibration items to identify transmission type:

Flag    0x018 b2:	0=Automatic Transmission (default); 1=Manual Transmission

Scalar 0x671: 	0=Automatic Transmission (default); <>0=Manual Transmission

Everyone knew to make a selection in the Flag 0x018 b2=1, but manual transmission users may have been unaware of the requirement to also make an entry in the Scalar at 0x671 because it was not documented.

This Version sets the Scalar to the value indicated by Flag 0x018 b2. To avoid confusion, the Scalar has been removed from the supplied XDF file. The user remains responsible to set 0x018 b2=1 if Manual Transmission. No action is required for automatic transmissions because the default =0 =Automatic Transmission.

Correction: IAC Control

Version 6

Manual Transmission IAC control logic from the AXYC BIN was added to the Initial Release of S_AUJP (v4). This manual transmission IAC control logic was being incorrectly applied to both manual and automatic transmission cars without regard to transmission type selected.

This Version ensures proper IAC control based on the XDF Transmission selection:

Switch 18 (0x018) - Bit 2 - Manual/Auto Transmission Select

(0 =Automatic default); 1 =Manual)

Despite manual and automatic transmission IAC control differences, automatic transmission users should not experience any noticeable operational change.

Correction: ALDL Reporting of Spark Advance TDC (SA-TDC)

Version 6

There is an error in the ALDL reporting of SA-TDC in the GM Factory version of AUJP. This is not a critical error of engine spark timing. It’s merely an ALDL reporting error, but can be disconcerting when viewing logs of SA-TDC and SA-REF without understanding what’s occurring.

This SA-TDC reporting error was corrected in this S_AUJP Version. It previously occurred:

- immediately after startup, and

- after exiting Deceleration Fuel Cut Off (DFCO)

For example, when DFCO is entered, SA-REF is immediately set to zero degrees advance. When DFCO is exited, SA-REF is gradually returned to its required value, which is the calculated SA-TDC less the Calibration’s Base Distributor Advance. This gradual increase of advance to the required value is called Spark Blending.

A before and after log excerpt of spark values during DFCO and after it is exited is provided below. Each row represents a TunerPro reported sample. Base Distributor Advance is 10.2 degrees.

Factory AUJP: At each sample, note the Factory AUJP SA-REF value and the difference between SA-REF and SA-TDC. During DFCO and afterward during Spark Blending, the Factory AUJP difference (“Diff” column) does not equal the Base Distributor setting of 10.2 degrees, which it should.

S_AUJP: Now, the reporting is correct where during DFCO and after exit, the difference between SA-TDC and SA-REF is 10.2 degrees as is should be:

------------FACTORY AUJP------------		-----------Revised S_AUJP----------

DFCO				DFCO

SA-REF	SA-TDC	Diff    	Status	SA-REF	SA-TDC	Diff	Status

0	18.28   	18.28     In DFCO 	  0   	10.20	10.20     In DFCO

3.16	36.21   	33.05       -----   	  2.10	12.30	10.20       -----

7.73	35.86   	28.13       -----   	  4.57	14.77	10.20       -----

13.36	35.86   	22.50       -----   	  7.03	17.23	10.20       -----

18.28	35.86   	17.58       -----   	10.91	21.11	10.20       -----

23.20	35.86   	12.66       -----   	13.71	23.91	10.20       -----

25.66	35.86   	10.20       -----	16.88	27.08	10.20       -----

25.66	35.86   	10.20       -----   	16.88	27.08	10.20       -----

Correction: Idle Spark Adjustment

Version 6

With this Version, Spark is no longer reduced when Idle RPM equals Commanded Idle RPM. Factory spark was reduced when Idle RPM was “greater than or equal” to Commanded Idle RPM, rather than just when “greater than”. As a result, when Idle RPM = Commanded Idle RPM, spark was reduced 0.35 degrees. While only a small reduction, this should not occur. There are already enough spark corrections being made when Idle RPM is above or below Commanded Idle RPM.

Reporting: Air Flow

Version 6

Purpose: Air Flow in grams per second is now reported in the supplied ADX file. Because $8d is Speed Density, this value is not measured as with MAF. Despite being calculated using the ideal gas law, it’s accurate and is used in determining Injector Base Pulse Width (BPW).

If the supplied ADX file is used, Air Flow will be reported from:

XDF ALDL Reporting Scalar #2 (with address reported =006B)

ADX Value entitled “Air Flow”

If the supplied ADX is NOT used, to report Air Flow in your ADX, make the following changes to a XDF Scalar and ADX Value, or add a new ADX value:

XDF

- Choose an ALDL Reporting Scalar: Choose any one of the 63 ALDL Reporting Scalars (the first 63 Scalars listed)

- Enter Address to Report: Enter “006B” into the Scalar

- Scalar Title: Change the Scalar Title to reflect that “Air Flow” is being reported

XDF Example:

ADX

- Select the Corresponding Value: Select the Value that corresponds to the item reported in the XDF Reporting Scalar. For confirmation that the correct Value is selected, the Packet Offset number should be one less than the number of the selected XDF ALDL Reporting Scalar.

- Source Data Size: Must be “8 bit”

- Title: Change to Reflect ”Air Flow”

- Data Type: Set to “Air flow”

- Stock Units: Set to “Grams per Second”

- Units Display: Enter “Gm/Sec”

- Packet Offset: Enter one less than the number of the selected ALDL Reporting Scalar

- Ranges and Alarms: Enter as desired

- Conversion Association: Set to “<All>”

- Conversion Equation (Not Shown): “X”

ADX example (if ALDL Reporting Scalar #2 is used):

Reporting: VE% Adjusted for Barometric Pressure

Version 6

Purpose: As with Air Flow above, it now possible to report VE% values. The VE value reported is not an exact Table value, but rather is VE% adjusted relative to the higher of Barometric Pressure (BARO) at prior ignition-off or subsequent ignition-on. This BARO adjusted VE% is used in the BPW calculation.

If the supplied ADX file is used, VE%-BARO Adjusted will be reported from:

XDF ALDL Reporting Scalar #1 (with address reported =0067)

ADX Value entitled “VE%-BARO Adjusted”

If the supplied ADX file is NOT used, make the following changes to an existing XDF Scalar and ADX Value, or add a new ADX value:

XDF

- Choose an ALDL Reporting Scalar: Choose any one of the 63 ALDL Reporting Scalars (the first 63 Scalars listed)

- Address to Report: Enter “0067” into the Scalar

- Scalar Title: “ALDL, Word# 01 VE%-BARO Adjusted”

XDF Example:

ADX

- Add a New, or Use an Existing, Value: A Value will need to be added to report VE%. The Packet Offset number should be one less than the number of the ALDL Reporting Scalar (“0” if ALDL Reporting Scalar #1 above is used)..

- Source Data Size: Must be “8 bit”

- Title: “VE%-BARO Adjusted”

- Data Type and Stock Units: “(None)”

- Units Display: Enter “VE%”

- Packet Offset: Enter one less than the number of the selected ALDL Reporting Scalar

- Ranges and Alarms: Enter as desired

- Conversion Association: Set to “<All>”

- Conversion Equation (Not Shown): ‘X’

ADX Example:

Reporting: Final PW After Compensations

Version 6

The Factory $8d ALDL Data Stream only reports Base PW (BPW). This is the calculated Speed Density BPW; which if in CL, is corrected based on INT and BLM changes resulting from NBo2 sensor voltage activity. But in both OL and CL, BPW will be additionally increased by one or both of the following to create a “Final PW” sent to the ECM to fire the injectors:

- Low Pulse Width Compensation (if BPW is < 3.90ms)

- Voltage Compensation based on the choice of voltage at either the ignition switch or fuel pump using:

Switch 16 (0x16) - Bit 6 - Voltage Source for Injector Voltage Offset

0 = Fuel Pump Voltage (Factory AUJP)

1 = Ignition Switch Voltage

This Version provides Final PW for ADX reporting. True Injector Duty Cycle Percent (DC%) can now be properly determined (especially at WOT) because the true time the injectors are held open is known. The equation for Batch Fire MPFI engines in Double Fire mode where there is one injector firing each 360 degrees of crankshaft rotation is:

DC% = ((Final PW * RPM) / 60,000) * 100  =  (Final PW * RPM) / 600

Note that Final PW includes the calculated Speed Density fuel requirement plus AE-MAP, the combination of which is fired each crank rotation. It does NOT include the asynchronous (instantaneous, as-needed), fuel provided for:

AE-TPS, DFCO Stall Saver, or when A/C first turned on

The above PW is inconsequential at WOT once AE-TPS has expired.

To report Final PW:

XDF

- Choose Two Adjacent ALDL Reporting Scalars: The Scalars must be adjacent because 2 bytes are needed. It’s easier to select 2 Scalars that are currently reporting a 2-byte value, such as Spark, Charcoal Canister Purge, etc.

- Enter Addresses to Report: Enter “31C8” and “31C9”, respectively, into the two adjacent Scalars. See examples below:

- Scalar Titles: Change the Scalar Titles to reflect that “Final PW MSB and LSB ” are being respectively reported (MSB=Most Significant Byte; LSB=Least Significant Byte)

- Size: Must be “2 Bytes (16 Bit)”

XDF Examples:

ADX

- Select the Corresponding Value: Select the Value that corresponds to reporting the item associated with the two XDF ALDL Reporting Scalars. Only one ADX Value is needed to report the two ALDL Reporting Scalars. For confirmation that the correct Value is selected, the Packet Offset number should be one less than the number of the first of the two selected ALDL Reporting Scalars.

- Source Data Size: Must be “16 bit”

- Title: Change to Reflect “Final PW”

- Data Type: Set to “Injector Final Pulse Width”

- Stock Units: Set to “Milliseconds”

- Units Display: Enter “mSeconds”

- Ranges and Alarms: Enter as desired

- Conversion Association: Set to “<All>”

- Conversion Equation (Not Shown): “X * 0.015259”

ADX Example:

Reporting: Distance Traveled

Version 6

In this Version, Distance Travelled is reported replacing “Accumulated Fuel” previously in XDF ALDL Reporting Scalars #49 and #50.

Factory AUJP Accumulated Fuel as reported is meaningless. Similarly, Factory provided Distance Travelled is meaningless because it is a continuously rolling value from 0 to 255.

Reporting of Distance Travelled was available in v5_D, but the XDF and ADX files did not contain the appropriate information. It was left up to the user to implement it (see v5_D Distance Travelled). It is also up to the current AUJP or S_AUJP user to implement it in this Version if the supplied S_AUJP ADX file is NOT used. In this instance, the reporting instructions are provided below:

XDF

- Choose Two Adjacent ALDL Reporting Scalars: The items must be adjacent because 2 ALDL Reporting Scalars are needed. It’s recommended to use the following ALDL Reporting Scalars because the data they report is meaningless:

ALDL, Word# 49 Accumulated Fuel MSB

ALDL, Word# 50 Accumulated Fuel LSB

- Change  Addresses to Report: Enter “3085” and “3086”, respectively, into the two adjacent Scalars replacing “3130” and “3131” that report Accumulated Fuel.

XDF Example:

- Scalar Titles: Change the Scalar Titles to reflect that “Distance Travelled MSB and LSB ” are being respectively reported (MSB=Most Significant Byte; LSB=Least Significant Byte)

- Size: Must be “2 Bytes (16 Bit)”

ADX

- Add a New Value: A Value needs to be added because there is no reporting of Accumulated Fuel in v5_D despite there being ALDL Reporting Scalars in the XDF. It only takes one ADX Value to report 2 ALDL Reporting Scalars.

- Source Data Size: Must be “16 bit”

- Title: Change to Reflect “Distance Travelled”

- Data Type and Stock Units: Select “(None)

- Units Display: Enter “Miles”

- Ranges and Alarms: Not Required

- Conversion Association: Set to “<All>”

- Conversion Equation (Not Shown): X / 2000

ADX Example:

Reporting: WBo2 AFR Differences

Version 6

The following differences are reported in the supplied ADX file:

Sensor #1 AFR .vs. Sensor #2 AFR

If 2 sensors are installed, the ADX Value below reports the AFR difference between Sensor #1 and Sensor #2:

WBo2 Sensors-AFR Difference

The difference reported is:

Sensor #1 AFR – Sensor #2 AFR

If Sensors are installed on both sides of dual exhaust, ideally minimal difference should be noted between their AFRs.

Sensor #1 and #2 AFR .vs. Commanded AFR

The supplied ADX file contains Values entitled:

WBo2 AFR .vs. Commanded-Sensor #1

WBo2 AFR .vs. Commanded-Sensor #2

The difference reported is:

Commanded AFR – WBo2 AFR

Commanded AFR in Closed Loop (CL) is always the Stoich AFR in the Calibration Scalar:

0x41A   Speed Density Param, Stoichiometric Air Fuel Ratio

Commanded AFR only changes from Stoich AFR when NOT in CL. Throttle changes that do not cause an exit from CL can cause differences between WBo2-sensor-measured AFR and fixed, Stoich Commanded AFR.

If WBo2 sensor(s) are not installed, the WBo2 ADX Values may be deleted from the ADX. If not deleted, the Values’ titles will be displayed, but without any data. Additionally, if not used, the ADX Hardware Channel Values below can also be deleted:

ECM Pins B3, F11 and F12 Support

This Version provides the capability of reporting data on these Pins. They have been previously used to report the following items. Each is 1-byte:

Address to Use

Item					Pin	in ALDL Reporting Scalar

WBo2 Simulated NBo2 Voltage	B3	    L01B9 (enter 01B9)

Exhaust Gas Temperature		F11	    L012A (enter 012A)

Engine Oil Temperature		F12	    L01BD (enter 01BD)

Each Pin provides a 0-5 volt input (0-255 ECM A/D Counts). It is the user’s responsibility to ensure sensors used match the characteristics of the Pins. Otherwise, hardware changes to the ECM may be required, or the ADX Conversion Equation must account for differences between the ECM’s 0-5v input range (0-255 Counts) and the sensor voltage output range.

Pins F11 and F12 digitized input (volts to ECM A/D counts) is lag filtered using the Oil Temperature scaling factor:

0x3E8=Oil Temp Filter Coefficient (25%)

Pin B3 can also be used to report 0-5v sensor input, and can be used to report WB Simulated NBo2 activity. It CANNOT, however, be used as input for o2-related fueling decisions. Note that WB simulated NBo2 output is generally 0-1v, not 0-5v. The ADX Conversion equation must account for this difference.

Engine-off Sensor Diagnostics

Version 6

Purpose: To read all sensors and report data for troubleshooting. The supplied Sensor Diagnostics ADX file concisely provides all information needed.

Changes: In v5_D, this feature was called U5/U6 Chip Read. In this Version, the name was changed to Sensor Diagnostics, and the requirement to set bit Flags was removed. Only the supplied ADX file is required:

S_AUJP v6-nn-Sensor Diagnostics.adx (where ‘nn’ is the Development Sequence Number)

Usage:  Load the supplied Sensor Diagnostics ADX file, begin logging and turn on the ignition. Do not start the engine. The following is reported for each Factory sensor:

- Raw voltage counts (Hex and Decimal)

- Sensor data converted to proper units

If a WBo2 sensor is installed, free-air raw counts are provided. The TPS sensor can be tested for Closed Throttle and WOT voltage and % by depressing the accelerator to any desired position to record data.

With this change, the following v5_D XDF Flags were removed:

V4 OPT 1, b1, Perform U6 Chip Read (S_V4/5 Only)

V4 OPT 2, b7, Perform U5 Chip Read (S_V4/5 Only)

Support for 4 and 6 Cylinder MPFI Engines

Version 6

In addition to 8 cylinder engines, the 1227730/1227727 ECMs and S_AUJP are capable of supporting the following Multi-Port Fuel Injected (MPFI) Engines using Distributor and Direct Ignition Systems (Distributorless):

6 Cylinder

4 Cylinder with and without TBI

Before making a 4 or 6 cylinder conversion to $8d S_AUJP, the engine should be running properly on a 7730/7727 ECM under another mask. A MEMCAL/NETRES designed for compatible cylinders is recommended if limp mode and proper knock sensor interface is required.

There are five XDF Scalars that require change from the 8 cylinder S_AUJP defaults to ensure proper spark timing and injector firing.

Again, thanks to 1Project2Many for his exhaustive research in providing the 4 cylinder spark values and for verifying the 6 cylinder values.

*************** USE AT YOUR OWN RISK ***************

4 Cylinder: All spark settings below are from code hacks and are recommendations only. They have NOT been S_AUJP in-car tested.

6 Cylinder: DIS values assume a 7 notch crankshaft reluctor wheel with a 10 degree offset Synch notch properly installed relative to the sensor. Both Distributor and DIS settings have been S_AUJP in-car tested.

6 Cylinder

|  |  | XDF Entry |  |
| --- | --- | --- | --- |
| BIN Location | XDF Scalar | Distributor(E) | DIS(A)(F) |
| 0x009 | SETUP, Number of Cylinders(C) (Decimal value) | 192 | 192 |
| 0x293 | ERROR QUAL 41, Cylinder Select(C) (Hex value) | 0x08 | 0x08 |
| 0x025 | SETUP SPARK ADV Base Dist./Reference Angle (Deg) | 10 | 60 |
| 0x026 | SPARK ADV, Maximum Allowed for SA-REF (Deg)(D) | 60 | -10 |
| 0x028 | SPARK RETARD, Maximum Allowed for SA-REF (Deg)(B)(D) | -10 | -70 |

4 Cylinder (TBI)

|  |  | XDF Entry |  |
| --- | --- | --- | --- |
| BIN Location | XDF Scalar | Distributor(G) | DIS(A)(H) |
| 0x009 | SETUP, Number of Cylinders(C) (Decimal value) | 128 | 128 |
| 0x293 | ERROR QUAL 41, Cylinder Select(C) (Hex value) | 0x18 | 0x18 |
| 0x025 | SETUP SPARK ADV Base Dist./Reference Angle (Deg) | N/App | User Defines |
| 0x026 | SPARK ADV, Maximum Allowed for SA-REF (Deg)(D) | N/App | User Defines |
| 0x028 | SPARK RETARD, Maximum Allowed for SA-REF (Deg)(B)(D) | N/App | User Defines |

4 Cylinder (Not TBI)

|  |  | XDF Entry |  |
| --- | --- | --- | --- |
| BIN Location | XDF Scalar | Distributor(I) | DIS(A)(J) |
| 0x009 | SETUP, Number of Cylinders(C) (Decimal value) | 128 | 128 |
| 0x293 | ERROR QUAL 41, Cylinder Select(C) (Hex value) | 0x10 | 0x10 |
| 0x025 | SETUP SPARK ADV Base Dist./Reference Angle (Deg) | User Defines | User Defines |
| 0x026 | SPARK ADV, Maximum Allowed for SA-REF (Deg)(D) | User Defines | User Defines |
| 0x028 | SPARK RETARD, Maximum Allowed for SA-REF (Deg)(B)(D) | User Defines | User Defines |

- Changes are required to the supplied ADX Conversion Equations for both SA-TDC and SA-REF to properly report these values

- Negative values are entered and are shown as positive values in the XDF. The XDF Conversion causes the entered positive value to be stored negatively in the Calibration as a 2s complement (eg: 3.87 Degrees Retard (-3.87 degrees) = 0xFFF5).

- Entry of incorrect values will result in the engine not properly running, improper injector firing and a CEL blink-out Cylinder Select Error 41.

- Fail-safe values only. The $8d S_AUJP code ensures the ECM will never see advance or retard that exceeds these values.

- 7730 ECM: $88, ARPL

- 7730 ECM: $6D, ACRJ, ASMJ, ATRA	 and $88, $A1, AUMX, BFAH

7727 ECM: $8F, AZRC

- No 7730/7727 ECM usage found.

Other ECM usage: 6864; $1C; HHY, FAT, DYN, HFL: 	Ref Angle=   0,  	Max Advance=  60,  	Max Retard=    8

Other ECM usage: 7165; $47; ACLH:		Ref Angle=   0,  	Max Advance=  60,  	Max Retard=    0

Other ECM usage: 7748; $83; APLX:		Ref Angle= 90,  	Max Advance= -30,  	Max Retard= -90

Other ECM usage: 6647; $94; AWRP, AUXM:	Ref Angle=   5,  	Max Advance=  55,  	Max Retard=  - 5

- 7727 ECM: $89; APTW;		Ref Angle= 60,  	Max Advance=    0,   	Max Retard= -44

7727 ECM: $3E; FML;   		Ref Angle= 66,  	Max Advance= -10,   	Max Retard= -66

- No 7730/7727 ECM usage found.

Other ECM usage: 7749; $58; AYBN; 	Ref Angle=   0,  	Max Advance= 46.8, 	Max Retard=    0

- No 7730/7727 ECM usage found.

Other ECM Usage: 7749; $5C; AMDD;		Ref Angle= 61,  	Max Advance=   0,	Max Retard= -70

Other ECM Usage: 7749; $6E; AJUL;		Ref Angle=   6,   	Max Advance= 42, 	Max Retard= - 4

Other ECM Usage: 8707; $A8; ANZF, ANLP, ASDD;	Ref Angle= 75,   	Max Advance= 60, 	Max Retard= -10

Other ECM Usage: 8505; $0D; BHFM;		Ref Angle= 66,   	Max Advance= - 6,	Max Retard= -66

Other ECM Usage: 6285; $36; BMZX;		Ref Angle= 63,   	Max Advance= 60, 	Max Retard=    0

Where 7730/7727 values are not available, required SA-REF values (those sent to the ECM) can be determined as:

SA-REF(Reference Advance to the ECM) = SA-TDC (TDC Advance Required) – Reference Angle (Base)

Appendix A

Transfer Your Calibration to the New S_AUJP BIN

The Calibration is the area in the BIN that contains the data the code uses. It is edited using the XDF file. When viewing any $8d BIN in a Hex Editor, the Calibration begins at the first byte (0x000), and the code begins at 0x3000. There are '00's between the end of the Calibration and start of the code. $8d AUJP reports 63 data items in the ALDL data stream. The addresses of these data items are stored in the Calibration. The ADX file controls how each of these data items is reported.

New AUJP User Without a Calibration

If you are a new AUJP user and do not have an existing Calibration, you will start with the default Calibration of this Version. You can skip the Current AUJP or S_AUJP User section below, but MUST update the default S_AUJP Calibration to be compatible with your setup (see UPDATE Your Calibration to use New Features below).

Current AUJP or S_AUJP User

Again, this is detailed but MUST BE DONE

You will need to transfer your Calibration to the new S_AUJP BIN. There are two ways to do this:

- Use a Hex Editor [Recommended: Quick, easy and error-free if use HxD below]

- Use the Transfer capability within TunerPro (TP) [prone to errors if something overlooked]

- Transfer Calibration Using a Hex Editor (This is the recommended procedure)

To make the transfer easy, you’ll need a good Hex Editor. HxD is recommended due to its ease of use and invaluable error reporting if the incorrect area is pasted after a copy. Download free for personal use here:

https://mh-nexus.de/en/downloads.php?product=HxD20

Select the English Installable or Portable version.

Instructions below assume the use of HxD, but any Hex Editor can be used.

- In the Hex Editor, open the new S_AUJP BIN and the BIN with Calibration to be copied

- In the BIN with the Calibration to be copied, highlight the value at address 0x00A through the end of the Calibration (the “end” is the last non-zero byte), except for S_AUJP v4 users:

IMPORTANT For S_AUJP v4 Users:

[v4 BIN Identification: The first two values are “01”(at 0x000), “30”(at 0x001)]

- The last data to transfer is at address 0xB4A. DO NOT transfer anything after 0xB4A. That area contains the old Launch Control Spark Table which was changed in v5_D.

- Stopping transfer at the value at 0xB4A will cause the current default LC Spark Table values to be in the Calibration

- For pasting below, remember the address of the last value to be transferred

- Ctrl-C to copy the Calibration to the Clipboard

- Highlight in the new S_AUJP BIN the same area that was copied, and Ctrl-V to paste the copied Calibration into the new S_AUJP BIN.

Highlighting the area to receive the copied data may seem unnecessary, but it is a fail-safe to endure errant data are not copied and that copied data is pasted to the exact location from which copied.

- After pasting, if you get a message asking if you want to change the file size, the area pasted did not match that copied. Abort the transfer and start over.

S_AUJP is now Calibrated with the information you selected and is ready to use. See below If You Will be Using the Supplied ADX File and then UPDATE Calibration to use New Features.

- Transfer Calibration Using TP
This is NOT the recommended procedure because it is prone to possible errors. It is not a fault of TP, but is due to differing content of old and new XDF files.
TP can only be used to transfer individual items. It CANNOT BE USED to transfer an entire calibration at one time.

- Ensure the new S_AUJP XDF file is loaded.

- Open the new S_AUJP BIN in TP

- Select “Compare” and then “Select Compare Bins”

- “Browse” to select the BIN with the Calibration you want to use

- Click “OK”

- Ctrl-T to open the Difference Tool

- At the top of the Difference Tool after “Find Difference Between”:

  - The new S_AUJP BIN should be in the window on the left

  - Your BIN with the Calibration on the right

Select the appropriate files if the above are not properly displayed left and right.

- DO NOT select “Limit search to defined items only” and click “Search”

- Highlight the items you want to transfer beginning with address 0x00A

  - Non-S_AUJP Users: The last address that should be copied is 0x989

  - S_AUJP v4 Users:

[v4 BIN Identification: The first two values are “01”(at 0x000), “30”(at 0x001)]

- The last data to transfer is at address 0xB4A. DO NOT transfer anything after 0xB4A. That area contains the old Launch Control Spark Table which was changed in v5_D. This will ensure the current default LC Spark Table values will be in the final Calibration.

- Click “Copy Selections from Right to Left”

S_AUJP is now Calibrated with the information you selected and is ready to use. See below If You Will be Using the Supplied ADX File and then UPDATE Calibration to use New Features.

It is strongly recommended that current AUJP and S_AUJP users use the supplied ADX file because of the many changes it contains.

If You Will be Using the Supplied ADX File

To report ADX items not currently in your Calibration, refer to the Table below and change ALDL Reporting Scalars as shown:

| ALDL Reporting Scalar Number | Enter | To Report from the ADX |
| --- | --- | --- |
| 1 | 0067 | VE%-BARO Adjusted |
| 2 | 006B | Air Flow |
| 7 | 0035 | DISPWD (Various Status Bits) |
| 49 | 3085 | Distance Travelled (MSB) |
| 50 | 3086 | Distance Travelled (LSB) |
| 51 | 00FF | Various Status Bits |

UPDATE Calibration to use New Features

New S_AUJP User

The update is MANDATORY.

Current AUJP or S_AUJP User:

The update is optional for all but S_AUJP v4 users, but is highly recommended. S_AUJP v4 users MUST follow this procedure to update the LC Spark Table values if LC is to be used.

Update Procedure:

All users should use the supplied XDF when editing the Calibration:

- Open the now-calibrated S_AUJP BIN in TunerPro and select to open the provided XDF file.

- If you are a current AUJP or S_AUJP user and will be using your ADX file, open it. Otherwise open the supplied ADX file.

- If you have never used AUJP, open the supplied ADX file and go to FEATURES AND CHANGES below and review each item

- Set Scalars, Flags and Tables as required to implement desired features

+++ S_AUJP is now ready to use +++

Appendix B

WBo2 AFR Conversion Equations

AFR Conversion Equations are provided for widely used WBo2 Controllers.

These equations are exactly as provided by curve-fitting software and are also formatted for TunerPro. Merely copy the TunerPro equations to the ADX. In the Non-Linear equations below, the number of significant decimal digits can be reduced if desired. However, it is necessary to retain the 6th degree of precision shown. If this degree of precision is reduced the accuracy of AFR may suffer, depending upon the shape of the manufacturer’s AFR .vs. voltage curve.

Non-Linear Output Controllers

A Lookup Table can be used if one is provided by the controller manufacturer. However, note that formulas are faster. They are also more accurate than using a Table because a Table will be interpolated linearly when it does not contain an exact match with an input value.

There is no equation provided for the internally-supported AEM Non-Linear controller. According to AEM, they have never produced a controller with Non-Linear output so it’s calculated AFR is questionable.

NOTES:

- AFR Conversion Equations are based on voltage “Counts” and not sensor voltage. Sensor voltage is converted to “Counts”.

- All AFRs here assume gasoline with a Stoich of 14.7:1. If using blended fuels (E10, E15, etc.), AFR values need to be adjusted downward by your estimate of Stoich at 1 Lambda. For example, if E10 fuel is assumed to have a Stoich AFR of 14.2:1, the reported AFR values would need to be adjusted downward by 3.4% [1-(14.2/14.7) = 1-0.966 = 0.034]. In this instance, it is better to use Lambda. Lambda can be reported by dividing the equations below by your estimate of Stoich AFR for the gasoline blend being used.

Voltage	Count	AFR

Non-Linear Controller				Range		Range	Range

FJO Non-Linear (All)			0.81-3.20	41-163	10-20

AFR Conversion Equation (x=COUNTS)

-35.28769072293002 + 3.3940159074766156x - 0.10303687749108925x2 + 0.001621007784606667x3 - 0.00001375827135258658x4 + 5.980148616481e-8x5 - 1.0393547627e-10x6

FJO Non-Linear (All) TunerPro (Copy to ADX)

AFR Conversion Equation (X=COUNTS)

-35.28769072293002 + (3.3940159074766156*X) - (0.10303687749108925*(POW(X;2)) + (0.001621007784606667*(POW(X;3)) - (0.00001375827135258658*(POW(X;4)) + (.00000005980148616481*(POW(X;5)) - (.00000000010393547627*(POW(X;6))

TechEdge DIY Non-Linear			1.4-2.9	72-148	10.08-20.66

AFR Conversion Equation (x=COUNTS)

-578.156153674179 + 35.20718820538851x - 0.8675099880728788x2 + 0.011231735117428398x3 - 0.0000803933433658704x4 + 3.0133595206995e-7x5 - 4.6114638937e-10x6

TechEdge DIY Non-Linear TunerPro (Copy to ADX)

AFR Conversion Equation (X=COUNTS)

-578.156153674179 + (35.20718820538851*X) - (0.8675099880728788*(POW(X;2)) + (0.011231735117428398*(POW(X;3)) - (0.0000803933433658704*(POW(X;4)) + (.00000030133595206995*(POW(X;5)) - (.00000000046114638937*(POW(X;6))

Voltage	Count	AFR

Non-Linear Controller				Range		Range	Range

Zeitronix Non-Linear (All)			0.47-3.12v	24-160	10-20

AFR Conversion Equation (x=COUNTS)

-1.7215066416556963 + 1.1367611821793175x - 0.0422948541377416x2 + 0.0007981659725157204x3 - 0.00000786394929373063x4 + 3.860145708777e-8x5 - 7.394866832e-11x6

Zeitronix Non-Linear (All) TunerPro (Copy to ADX)

AFR Conversion Equation (X=COUNTS)

-1.7215066416556963 + (1.1367611821793175*X) - (0.0422948541377416*(POW(X;2)) + (0.0007981659725157204*(POW(X;3)) - (0.00000786394929373063*(POW(X;4)) + (.00000003860145708777*(POW(X;5)) - (.00000000007394866832*(POW(X;6))

Linear Output Controllers 																	TunerPro Format

AFR Conversion			AFR Conversion

Voltage		Count	AFR 		Equation				Equation (X=COUNTS)

Controller			Range		Range	Range		(x=COUNTS)				(Copy to ADX)

AEM 30-0300 X-Series UEGO Gauge 			0.5-4.5		25-229	8.5-18.0		0.0464x + 7.3124 		(0.0464*X) + 7.3124

AEM 30-2310 & 30-0310 X-Series Inline WB Controller	0.5-4.5		25-229	8.5-18.0		0.0464x + 7.3124 		(0.0464*X) + 7.3124

AEM 30-2320 X-Series Wi-Fi UEGO and EGT Controller	0.5-4.5		25-229	8.5-18.0		0.0464x + 7.3124 		(0.0464*X) + 7.3124

AEM 30-2340 4 Channel UEGO Controller		0.5-4.5		25-229	8.5-18.0		0.0464x + 7.3124 		(0.0464*X) + 7.3124

AEM 30-4100 & 30-4110 Controller & Gauge		0-4.99		0-254	10-19.98		0.0390625x + 10			(0.0390625*X) + 10

AEM 30-4900 Failsafe Gauge			0.5-4.5		25-229	8.5-18.0		0.0464x + 7.3124 		(0.0464*X) + 7.3124

AEM 30-5130 Analog WB Air/Fuel Gauge		0.5-4.5		25-229	8.5-18.0		0.0464x + 7.3124 		(0.0464*X) + 7.3124

Autronic MAFM1				0-1		0-51	10-30		0.3922x + 10 			(0.3922*X) + 10

Autronic MAFM1				0-5		0-255	10-30		0.0784x + 10 			(0.0784*X) + 10

Dynamic EFI TT-1				0-5		0-255	10-20		0.0390625x + 10			(0.0390625*X) + 10

DYNOCOM A/FM					0-5		0-255	9-16		0.0275x + 9 			(0.0275*X) + 9

DynoJet 2						0-5		0-255	10-18		0.0314x + 10 			(0.0314*X) + 1

Evo Prosport Gauge (All)			0-5		0-255	10-19.98		0.0390625x + 10			(0.0390625*X) + 10

FJO						0-5		0-255	10.3-19.1	0.0345x + 10.3 			(0.0345*X) + 10.3

Innovate LC-1 and LC-2				0-5		0-255	7.35-22.39	0.05898x + 7.35			(0.05898*X) + 7.35

Innovate LC-1 and LC-2				0-5		0-255	10-20		0.0390625x + 10			(0.0390625*X) + 10

Innovate LC-1 and LC-2				1-2		51-102	10-20		0.19608x				0.19608*X

NGK AFX						0-5		0-255	9-16		0.0275x + 9 			(0.0275*X) + 9

PLX SM-AFR (All)				0-4.99		0-254	10-19.98		0.0390625x + 10			(0.0390625*X) + 10

TechEdge DIY					0-5		0-255	9-19		0.0392x + 9.0072 		(0.0392*X) + 9.0072

v5_D Changes (Previous Version = Changes to v4)

Shift Light

Version 5_D

Purpose: Uses the Check Engine (CE) Light to indicate a RPM Shift Point when illuminated.

How to Use: The CE Light will be illuminated when RPM is >= the Scalar:

0x365 Scalar: Shift Light-ON if RPM >= (Shift Point)

Default: Shift light illumination is OFF by default with the above Scalar being set = 7969 RPM

NOTE: In reviewing logs, there can at times be what appears to be a lag from when the Shift Light is indicated ON relative to the reported RPM. For example, if the Shift Point is set at 5000 RPM, it’s possible to see the Shift Light indicated ON in a log sample at a slightly higher RPM, say 5025 RPM.

This is caused by timing differences between when the Shift Light is indicated ON and when the corresponding RPM is reported in the ALDL data stream. The light is always indicated on, and turned on, at exactly the Shift Point RPM. Remember that code activities occur as fast as 160 times a second, but log samples are, on balance, provided every 0.1 seconds or so. Many activities are not reported.

Enable Open Loop (OL)

Version 5_D

Purpose: Open Loop (OL) will be enabled as long as RPM is below a RPM Threshold Scalar.

How to Use: OL is maintained as long as RPM is below the Scalar Threshold:

0x992 Scalar: ENABLE OPEN Loop if RPM/31.25 <

Default: Disabled with the above Scalar Default RPM = 0

Examples:

OL Idle: 	Set the Scalar to a value just above the upper limit of idle RPM

fluctuation. OL idle will be maintained while RPM is < the Scalar value.

Once RPM equals or exceeds the Scalar, CL will be entered.

OL Always: 	Set the Scalar to 7969 RPM and OL will always be maintained

Closed Throttle (CT) Spark Advance (SA) .vs. RPM Table if Coolant Temp < 113*F (44.75*C)

Version 5_D

Purpose: To use the CT SA .vs. RPM Table when Coolant Temp is < 113*F.

How to Use: Enable by setting the Flag:

0x98E b4=1: Use Closed Throttle SA Table if < 113*F

Default: Enabled with the above Flag = 1

If the feature is NOT enabled, the 100 kPa column of the Main SA Table is used for Closed Throttle SA until Coolant Temp exceeds 113*F. This is the Factory AUJP default.

U5 & U6 Chip Read

Version 5_D

This feature has been changed and is not referred to as “Sensor Diagnostics” (see v6 Sensor Diagnostics).

Display Word (DISPWD) Address Relocation (Now at L0035)

This is a code change item only and does not affect the user as long as the provided ADX file is used. If NOT being used, the user is encouraged to add reporting of these bits in their ADX.

Purpose: DISPWD is a "Display Word" for ALDL Data Stream reporting that aggregates bits to allow reporting of various activities.

Default: The DISPWD ALDL reporting address is in the supplied XDF here:

ALDL, Word# 07 Display Word-Reports Various Status Bits (L0035)

How to Use: The following items are reported from the supplied ADX using DISPWD:

KS=Knock Sensor

AE=Acceleration Enrichment

PE=Power Enrichment

DFCO=Deceleration Fuel Cut Off

b7, 1 = Knocks Indicated		(status of L00FF, b2)

b6, 1 = KS Failure Indicated    	(status of L003C, b5)

b5, 1 = KS Enab by Diff Coolant   	(status of L0041, b7)

b4, 1 = Hyway Fuel Active		(status of L0039, b7)

b3, 1 = In AE-TPS              		(status of L0045, b7)

b2, 1 = In AE-MAP               	(status of L0045, b3)

b1, 1 = In PE              		(status of L0046, b5)

b0, 1 = In DFCO (New in v5_D)	(status of L0046, b1)

Knock-Detected Bit

Version 5_D

Purpose: When a knock is detected, a bit is set and is updated in DISPWD (L0035 b7 =Knocks Indicated). The user can select to detect either Slow or Fast Knocks, BUT NOT BOTH SIMULTANEOUSLY:

- Fast Knocks: These are the actual knocks from the sensor which are used for Knock Retard Degrees determination, but they are not normally reported.

- Slow Knocks: Once Fast Knocks have accumulated to a count of 256, a single Slow Knock is reported in the ALDL Data Stream, and the Fast Knock count is returned to 0 to again begin counting Fast Knocks. (256 Fast = 1 Slow, 512 Fast = 2 Slow, etc.).

How to Use and Defaults: The Knock-Detected Bit is always active, is reported in DISPWD b7, can be set to remain on for a specified time, and indicates the following:

Flag 0x98F b6=0: V4 OPT 2, b6, Knock-Detected Bit:

(0=Indicate SLOW Knocks (Default); 1=Indicate FAST Knocks)

Bit ON-time is set with:

Scalar 0x993: (0.5 seconds= Default) Knock Detected Bit Maximum ON-Time

Knock bit indication of Slow Knocks is the default because the “count” of Slow Knocks is reported in the GM Factory ALDL Data Stream by default here:

ALDL, Word# 43 Knock Counter (SLOW Counts)

If desired, Fast knocks can be indicated and reported by making the following changes:

Change the Knock Detected Flag to =1 to Indicate Fast Knocks:

Flag: 0x98F b6: V4 OPT 2, b6, Knock-Detected Bit (0=Default)

Change Knock Counter Scalar to “00C2” to report FAST Knock Counts (0031=Default):

Scalar: ALDL, Word# 43 Knock Counter (FAST Counts)

Notes for Bit-ON Timer (Scalar 0x993):

- The time after which the bit will be turned off begins after the last knock in a series of continuous knocks has occurred.

- A setting of less than 0.2 seconds is not recommended because the bit may be turned on and then off before a log sample is reported.

Highway Mode Spark Advance Disable RPM

Version 5_D

Highway Mode Spark Advance Disable RPM was changed from 2400 SCALED RPM (4000 Actual RPM) to 1500 SCALED RPM (2200 Actual RPM) to lower the Disable Speed. Highway Mode Spark Advance now remains enabled at or above 1500 SCALED RPM.

SCALAR 0x1A9: Highway Mode SPARK ADV, Disable if SCALED RPM <

SCALING adjusts Actual RPM to 400 RPM increments for Table lookups and some comparisons to Calibration Scalars. Examples below:

Actual RPM   SCALED RPM

1000 	    	1200

1200 	    	1600

1400 	    	2000

1500	    	2200

1600 	    	2400

1800 	    	2800

2000 	    	3200

2200       	3600

2400        	4000

ALDL Data Stream Reporting Change

New Status Word (L00FF) Replaced Factory AUJP Accumulated Distance

  - Factory Distance Travelled reported meaningless data and was replaced with a new Data Word that reports the status of various events:

b3, 1 = TCC Unlocked: MPH > Max Allowed for Bypassing TCC Coast Unlock

b2, 1 = Slow Knock Detected (for DISPWD b7)

b1, 1 = LC RPM Staged

b0, 1 = LC Active

  - The reporting address of this new Data Word is L00FF, and is in the XDF here:

ALDL, Word# 51 Launch, TCC, Knock Status (L00FF Bits)

S_AUJP Distance Traveled

See v6 Reporting Distance Traveled for details on Reporting this item

Display Word (DISPWD) now at L0035 and “In DFCO” Added

  - DISPWD was reported in v4, but at a different address

  - “In DFCO” was added in v5_D as b0

See v5_D  Display Word (DISPWD) Address Relocation for the items reported.

WBAFR (WBo2 AFR) Reporting Address Changed from L0035 to L01C5

  - WBo2 AFR replaced ‘Keep Alive IAC Minimum Steps’ in v4 ALDL Data Stream reporting

  - In v4, it was reported at address L0035. In later versions, WBAFR is at address L01C5 in ALDL Reporting Scalar #37:

ALDL, Word# 37 WBo2 Sensor #1: AFR/Raw Counts

Miles per Gallon (MPG) Calculation (Fuel Economy)

  - JP86SS included this item in v4 but it was not documented:

  - In v4, the Injector Flow Rate in the Conversion Equation was provided via a link to an ADX Value entitled “Injector Size (User)”. This link is no longer in the supplied ADX file.

  - To properly report Instant MPG, YOUR Injector Flow Rate needs to be entered in the ADX “Fuel Economy” Conversion Equation if it’s different than “22.08”, replacing the default value “22.08” as follows:

- Get your Injector Flow Rate from this XDF Scalar:

0x41c	  SETUP, Injector Flow Rate

- Return to the ADX file. Under ‘Values’, click on “Fuel Economy” and then click the 'Conversion' Tab. Click the 'Set' Tab and replace '22.08' in the Conversion Equation with your Injector Flow Rate.

      - Note that MPG is only reported while moving. Otherwise it =0.

Barometric Pressure (BARO)-Adjusted VE% Reported

Reporting of this item was available in v5_D, but how to report it was not documented. See v6 Reporting VE% Adjusted for Barometric Pressure for details.

v4 Changes

The items listed here are changes made to create v4 from Factory AUJP. The changes remain in all subsequent Versions.

IMPORTANT NOTE: Some Calibration values were changed in v4. Subsequent versions may have also changed these and other values. All values changed are listed below in tow columns: “Factory” and “Current”.

Light/LED (by Junkcltr)

Version 4

This feature provides a Light/LED to indicate various engine conditions. It is disabled by default in the current Version to avoid conflict with the v5_D Shift Light feature (see Shift Light). This is a change from v_5D where “In PE” was indicated with the Light/LED on for 1 second.

The feature uses Pin F5 of the 7727/7730 and 7749 ECMs. Pin F5 provides a ground to turn the Light/LED on, the same connection that activates the check engine (Service Engine Soon [SES]) light. Connect one wire of the Light/LED to Ignition 12 volt power and the other wire to ECM Pin F5.

If using a LED instead of a light, use a current-limiting resistor as required. LED details:

- Recommended resistor = 1K ohms. If 1K ohm not available then use 700 - 1.5K ohm in place of 1K ohm resistor

- Electrical current = 14volts/1K = 14 ma. Check with manufacturer for maximum current rating. LED has a positive and negative lead, it must be wired properly or the LED will never turn on

- Verify LED wiring by setting the "enable knock failure" bit_1 (set bit_1 = 1 [see below]), and disconnect knock sensor at the engine. Start engine and the LED should light within approx. 10 seconds. This verifies the LED is wired properly. Connect the knock sensor at the engine and set knock sensor Light/LED options in the Calibration knock control word as desired (see below).
Alternate LED wiring test: Instead of disconnecting knock sensor at engine, apply a ground to ECM Pin F5. The LED should turn on when the ground is applied.

A LED will be dimly light when using the F5 pin due to leakage current flow. This is not a failure. A light will illuminate brightly when the output is commanded on (Indicating).

The feature is controlled by Flags at 0x990. There are:

      - 5 “Selection Bits” defining the activities that can be reported, and

      - 2 “Control Bits” that control the on/off state of the Light/LED:

SELECTION Bits

0x990 KNOCK Light/LED Selection Bits:

Light/LED Pin F5 SELECT, b7 = Not Used

Light/LED Pin F5 SELECT, b6 = Reports "In PE"

Light/LED Pin F5 SELECT, b5 = Reports "In AE-TPS"

Light/LED Pin F5 SELECT, b4 = Reports "In AE-MAP"

Light/LED Pin F5 SELECT, b1 = Reports "Knock Sensor Failure" (1=Yes; 0=No)

Light/LED Pin F5 SELECT, b0 = Reports "Knock Detected" (1=Yes; 0=No)

Select ONLY ONE "LIGHT BIT" as the highest number bit selected will be reported:

b6 = In PE

b5 = In AE-TPS

b4 = In AE-MAP

The Knock–related bits b0 & b1 can BOTH be selected if used. These bits can be used even if one of the other “Selection” bits b6-b4 is set (see Example below)

CONTROL Bits

0x990 KNOCK Light/LED Control Bits:

Light/LED Pin F5 CONTROL, b3 = Duration ON (1=1 Sec; 0=3 Sec)

Light/LED Pin F5 CONTROL, b2 = ON State (1=Blink; 0=Constant)

Example

The Flag settings below assume “In PE” and “Knock Detected” are to be indicated, and with the Light/LED to be on for 1 second. The bits would be: [0100 1001]

b7, 0 = Not Used

b6, 1 = Report “In PE”

b5, 0 = Can’t be set because b6 set

b4, 0 = Can’t be set because b6 set

b3, 1 = ON 1 Second

b2, 0 = ON Constantly

b1, 0 = Report “Knock Sensor Failure” (Light/LED is FULL ON if knock sensor is failing)

b0, 1 = Report “Knock Detected”

Extended VE Table to 6400 RPM (by Z69 & RBob)

Version 4

This feature is controlled by:

0x98E	Flag:	 V4 OPT 1 b5, Use Extended VE Table to 6400 RPM

(0=Use Factory Table; 1=Use Extended Table)

0x9A0	Table:	 VE Upper Table Extended to 6400 RPM

RPM/31.25 Added (by Scott Lopez)

Version 4

RPM up to 7969 is now available and can be reported by entering “0149” in one of the 63 ALDL Reporting Scalars and adding an ADX Value.

Maximum In-Gear Idle RPM

Version 4

Prior to v4, maximum in-gear idle RPM was hard-coded at 800 RPM. With v4, the hard-coding was replaced by a Scalar with a default of 1000 RPM:

0x991	In-Gear Idle Maximum RPM

In-gear idle RPM will be maintained at or less than this value, plus minor idle adjustments for A/C-On and ignition switch voltage.

Launch Control Spark Table

Version 4

Launch Control of v4 was redesigned in v5_D rendering the v4 Spark Table unusable (see Launch Control (v5_D)).

EGR Test was Disabled

Version 4

Previously, EGR tests were conducted if MPH was >= 30 MPH. V4 changed the MPH to 255 which disabled the test. To reinstate the test, change the Scalar below to < 255 MPH:

ERROR QUAL 32, EGR-Skip Diagnostics if MPH <

Other EGR-related changes were also made:

Address			  XDF Item				     	Factory	Current

0x2AE	EGR, Lower TPS% Threshold: Proceed if TPS% >=   	1.95		0

0x2AF	EGR, Upper TPS% Threshold (NOT USED)			2.34		0

0x2B0	EGR, Skip In-Gear EGR if MAT A/D < 	   		30*F		255*F

0x2B5	EGR, % Duty Cycle RPM .vs. Vacuum (kPa)		Various	All 0

0x2D9	EGR, % Duty Cycle  Multiplier .vs. Coolant Temp		Various	All 0

PE Spark Advance was Disabled

Version 4

Previously, spark would be added if in PE. Spark is no longer added for PE. To reinstate it, change the values in the Table below to something other than zero:

0x1B3: PE, Spark Advance .vs. RPM

Low Octane Knock Retard was Disabled

Version 4

Previously, spark would be subtracted if knocks were detected under certain conditions. That was disabled in v4. The following changes were made and need to be reinstated if the feature is desired. Low Octane Knock Retard is disabled if the Scalar at 0x229 below =104 kPa. Other settings are irrelevant.

Address			  XDF Item				   	   	Factory	Current

0x229	Low Octane Knock Retard, Disable if SCALED MAP kPa <	  	69		104

0x22A	Low Octane Knock Retard, Disallowed if 25ms MAP kPa Chg > 	20.63		20.00

0x22B	Low Octane Knock Retard, Disallowed if Accum Retard Deg <	5.98		5.98

0x22C	Low Octane Knock Retard, Increase Counter		 	50		0

0x22D	Low Octane Knock Retard, Spark Decrease Rate (Degrees)	0.70		0

0x22E	Low Octane Knock Retard, Base Retard				4.92		0

0x22F	Low Octane Knock Retard Mult. .vs. RPM				Various	All 0

0x238	Low Octane Knock Retard Mult. .vs. MAP				Various	All 0

Forced Knock (Err 43B) Test was Disabled

Version 4

Previously if coolant temp and MAP were above thresholds, Spark advance would be gradually increased to see if a knock occurred as a test of both the knock sensor and ESC circuitry. This was disabled in v4. To reinstate it, change to the “Factory” value:

Address			  XDF Item				   	   	Factory	Current

0x29C	Min Coolant Temp to Enable Err 43B Forced Knock Test		203*F		304*F

Charcoal Canister Purge (CCP) was Disabled

Version 4

Address			  XDF Item				   	   	Factory	Current

0x2E6 	CCP, Adjust Highway Mode AFR if DC% >=			50%		0%

0x2EA	CCP, Consider Purge if Coolant Temp >				140*F		304*F

0x2EB	CCP, If Purge NOT Active, Purge Allowed if MPH >			1		0

0x2ED	CCP, Purge Allowed if TPS% >					1.56		0

0x2EE	CCP, If Purge Active, Purge Allowed if MPH >			0		0

0x2F0	CCP, Skip Low TPS% Tests if TPS% >=				0.78		0

TCC Activity Changes

Version 4

Address			  XDF Item				   	   	Factory	Current

0x314	TCC, Lock Enable Minimum Coolant Temp.				123*F		68*F

0x31D	TCC, Don't Unlock if MPH >						75		120

0x31F	TCC, Lock 3rd Gear if MPH >					43		38

0x321	TCC, Unlock 3rd Gear MPH <					41		36

0x323	TCC, Unlock 3rd Gear %TPS .vs. RPM or MPH			(A)		(A)

0x32F	TCC, Lock 3rd Gear TPS% .vs. RPM or MPH			(A)		(A)

0x33B	TCC, Lock 4th Gear if MPH >					43		38

0x33D	TCC, Unlock 4th Gear if MPH <					41		36

0x33F	TCC, Unlock 4th Gear TPS% .vs. RPM or MPH			(A)		(A)

0x34B	TCC, Lock 4th Gear TPS% .vs. RPM or MPH			(A)		(A)

(A) Table TPS% values lowered in v4 to Lock/Unlock earlier

Fan On/Off Changes

Version 4

Address			  XDF Item				   	   	Factory	Current

0x375	Fan 2, High A/C Pressure/Fan 2 Off:  Enable if MPH <=		255		50

0x376	Fan 2, High A/C Pressure/Fan 2 On: Disable if MPH >		255		50

0x377	Fan 1, OFF Temp							221*F		202*F

0x378	Fan 1, ON Temp							226*F		207*F

0x379	Fan 2, OFF Temp							304*F		202*F

0x37A	Fan 2, ON Temp							304*F		207*F

AIR Divert Disabled

Version 4

Address			  XDF Item				   	   	Factory	Current

0x39A	AIR, Divert AIR if Coolant Temp <					58*F		304*F

0x49D	AIR, o2 Rich/Lean Reduction Volts when AIR Diverted		100mv		0mv

Fuel Cut-off RPM Changes

Version 4

Address			  XDF Item				   	   	Factory	Current

0x3F5	SETUP, Fuel Cut Off  if RPM >					10031		6642

0x3F8	SETUP, Fuel Resumed if Cut Off & RPM <				9544		6510

Highway Mode Fuel Changes

Version 4

Address			  XDF Item				   	   	Factory	Current

0x481	Highway Mode Fuel, Disable if Coolant <=				185*F		140*F

0x484	Highway Mode Fuel, Disable if MPH <=				255		53

Open-to-Closed Loop Transition Changes

Version 4

Address			  XDF Item				   	   	Factory	Current

0x489	Loop Closed Param, Enable CL If Coolant Temp >			113*F		68*F

0x48C	Loop Closed Param, If COLD Consider CL if Time >=		190 sec	120 sec

0x48D	Loop Closed Param, If WARM Consider CL if Time >=		150 sec	90 sec

0x48E	Loop Closed Param, If HOT Consider CL if Time >=	  	30 sec	  	20 sec

Open-to-Closed Loop Logic:

Temp (*F)		Type		Wait time for Entering CL (secs)

< 58			Cold				120

> 58 & <= 95		Warm				90

> 95			Hot				20

BLM Idle Cell Change

Version 4

The v4 setting causes idle to NOT be restricted to BLM Cell 4

Address			  XDF Item				   	   	Factory	Current

0x522	CCP, Don't Force Idle in BLM Ceil #4  if CCP DC% >=		23.4		0

BLM RPM and MAP Cell Boundary Changes

Version 4

While the variables are listed separately, they are actually in a Table in the Calibration at:

RPM: 0x510

Factory AUJP:	700   1200   2000

Current:	  	900   1500   2000

MAP 0x513

Factory AUJP:	25     50       80

Current:		30     50       75

Other BLM Related Changes

Version 4

Address		  XDF Item				   	   		Factory	Current

0x51B	BLM Update, Frequency						1.4 sec	1.0 sec

0x51E	BLM Value, Minimum							108		96

AE-TPS  Changes

Version 4

Address			  XDF Item

0x543	AE-TPS, 6.25ms TPS Change Factor (for Pump Shot)

Current Table values slightly increased over Factory AUJP to provide more AE-TPS at all TPS% levels

IAC and Throttle Follower Changes

Version 4

Address			  XDF Item				   	 	Factory	Current

0x65D	IAC Throttle Follower, Slope Gain					150%		39.06%

0x66A	IAC, Maximum IAC Open Position Steps				160		231

0x65E	IAC Throttle Follower, Max Steps Allowed if  In-Gear		40		230

0x69A	IAC, OK to Learn Idle IAC Keep-Alive Steps if Coolant >		191*F		140*F

0x6B2	IAC, Steps Added if In-Gear & Moving <= 63 MPH		Various	All 0

ALDL Data Stream Reporting Changes

Version 4

--------- Factory ----------	 	------------------- Current ------------------

Number	Address	Description		Address	Description

1		8000		LSB PROM ID		0067		VE BARO-Adjusted

2		8001		LSB PROM ID		01B9		Simulated NB Counts from WB

7		0051		New DISPWD		0035		New DISPWD (0x0035 Bits)

37		0035		Unused		01C5		WBo2 AFR Sensor #1

51		0134		Distance Traveled	00FF		LC, TCC, Knock Status Bits

ALDL Reporting Addresses

Version 4

In the v4 XDF file, JP86SS added Scalars providing access to the 63 ALDL Data Reporting addresses. They remain in later Versions and are a great addition. In v6, they were moved to the top of the Scalar List. Errors were moved to the bottom to separate them from other Calibration Scalars.

Custom Chip IDs

Version 4

Chip IDs can be entered to track up to 4 BINs. Use:

CHIP Custom ID 1 (User#)

CHIP Custom ID 2

CHIP Custom ID 3

CHIP Custom ID 4

Flag Changes

Version 4

Changes were made to the following Flags:

Factory	       Current

Hex	Binary		Hex	Binary

0x018	d8	11011000		50	01010000

b7=1 =CC Purge when Energized; 0=Purge when NOT energized using 0% Duty Cycle

b3=1 =Unlock TCC in DFCO; 0=DON'T Unlock

0x019	2d	00101101		29	00101001

b2=1 =Enable Highway Mode Spark Advance; 0=DON’T Enable

0x24A	ce	11001110		c6	11000110

b3=1 =Continue Additional EGR Err 32 Tests; 0=Skip Additional Tests

***** END OF DOCUMENT *****