	Combo
	Disp.
(cu. in.)
	Carburetor
	Heads
	Intake
	Camshaft
	Camshaft Duration @ 0.050" Lift
	Camshaft Lift (in. @ valve)
	Rockers
	Headers
	Pistons
	Rods
	Crankshaft
	Distributor
	Timing
	Comp.
 Ratio
	Maximum Horsepower
	Maximum Torque
(lb.-ft.)
	Engine Article Source

	1
	350
	modified 600 cfm
	World Products Sportsman II
	Dart II Sportsman dual plane
	 
	218/224°
	0.462/0.469
	 
	 
	 
	 
	 
	 
	 
	9.2:1
	350 @ 5500
	 
	World Products website

	2
	350
	modified 750 cfm
	World Products Sportsman II
	Dart II Sportsman dual plane
	 
	224/230°
	0.477/0.480
	 
	 
	 
	 
	 
	 
	 
	9.5:1
	375 @ 6000
	 
	World Products website

	3
	350
	modified 750 cfm
	World Products Sportsman II
	Dart II Sportsman dual plane
	 
	230/236°
	0.487/0.490
	 
	 
	 
	 
	 
	 
	 
	9.8:1
	430 @ 6000
	 
	World Products website

	4
	383
	Holley 750 double pumper
	GM #441 castings, 76cc chambers, 2.02/1.60 in. valves, pocket ported
	Edelbrock Performer
	Comp Cams 280° Magnum hydraulic
	230/230°
	0.480/0.480
	1.5:1
	1 5/8 in. street
	TRW forged
	5.565 in.
	cast
	 
	 
	9.3:1
	382 @ 5300
	400 @ 4500
	Hot Rod Magazine, February 1989, p. 39

	5
	383
	Holley 750 double pumper
	GM #441 castings, 76cc chambers, 2.02/1.60 in. valves, pocket ported
	Edelbrock Performer
	Comp Cams 292° Magnum hydraulic
	244/244°
	0.501/0.501
	1.5:1
	1 5/8 in. street
	TRW forged
	5.565 in.
	cast
	 
	 
	9.3:1
	399 @ 5700
	399 @ 5000 and 5500
	Hot Rod Magazine, February 1989, p. 39

	6
	383
	Holley 750 double pumper
	GM #441 castings, 76cc chambers, 2.02/1.60 in. valves, pocket ported
	Edelbrock Victor Jr.
	Comp Cams 280° Magnum hydraulic
	230/230°
	0.480/0.480
	1.5:1
	1 5/8 in. street
	TRW forged
	5.565 in.
	cast
	 
	 
	9.3:1
	400 @ 5700
	403 @ 4500 and 5000
	Hot Rod Magazine, February 1989, p. 39

	7
	383
	Holley 750 double pumper
	GM #441 castings, 76cc chambers, 2.02/1.60 in. valves, pocket ported
	Edelbrock Victor Jr.
	Comp Cams 292° Magnum hydraulic
	244/244°
	0.501/0.501
	1.5:1
	1 5/8 in. street
	TRW forged
	5.565 in.
	cast
	 
	 
	9.3:1
	420
	400
	Hot Rod Magazine, February 1989, p. 39

	8
	383
	Holley 750 double pumper
	Trick Flow 23° Street
	Edelbrock Victor Jr.
	Comp Cams hydraulic roller
	230/236°
	0.510/0.520
	Trick Flow roller, 1.5:1
	1 3/4 in. primaries, with 3 in. exhaust and Flowmaster mufflers
	KB hypereutectic, 12 cc dish
	Trick Flow forged, 5.7 in.
	Trick Flow forged
	MSD Billet with 6AL ignition
	 
	10.0:1
	460 @ 6000
	451 @ 5000
	Trick Flow Specialties website

	9
	383
	Holley 750 double pumper
	World Products Sportsman II
	Edelbrock Victor Jr.
	Comp Cams hydraulic roller
	230/236°
	0.510/0.520
	Trick Flow roller, 1.5:1
	1 3/4 in. primaries, with 3 in. exhaust and Flowmaster mufflers
	KB hypereutectic, 12 cc dish
	Trick Flow forged, 5.7 in.
	Trick Flow forged
	MSD Billet with 6AL ignition
	 
	10.0:1
	422 @ 5500
	426 @ 4500
	Trick Flow Specialties website

	10
	383
	Holley 750 double pumper
	Dart Iron Eagle
	Edelbrock Victor Jr.
	Comp Cams hydraulic roller
	230/236°
	0.510/0.520
	Trick Flow roller, 1.5:1
	1 3/4 in. primaries, with 3 in. exhaust and Flowmaster mufflers
	KB hypereutectic, 12 cc dish
	Trick Flow forged, 5.7 in.
	Trick Flow forged
	MSD Billet with 6AL ignition
	 
	10.0:1
	446 @ 5500
	433 @ 4500
	Trick Flow Specialties website

	11
	383
	Holley 600 cfm
	Air Flow Research 195cc Street
	Air Flow Research FloPower RPM
	Air Flow Research hydraulic
	218/223°
	0.460/0.470
	 
	1 5/8 in.
	 
	 
	 
	 
	 
	9.0:1
	440 @ 5000
	470 @ 3500
	Air Flow Research website

	12
	350
	Holley 600 cfm
	Air Flow Research 195cc Street
	Air Flow Research FloPower RPM
	Air Flow Research hydraulic
	218/223°
	0.460/0.470
	 
	1 5/8 in.
	 
	 
	 
	 
	 
	9.0:1
	420 @ 5000
	435 @ 3800
	Air Flow Research website

	13
	355
	Holley 750 double pumper
	Air Flow Research 190cc Street
	Air Flow Research FloPower RPM
	Comp Cams 270° Magnum hydraulic
	224/224°
	0.470/0.470
	 
	1 5/8 in.
	 
	 
	 
	MSD
	34°
	9.5:1
	430 @ 5000
	435 @ 3800
	Air Flow Research website

	14
	355
	Holley 750 double pumper
	Air Flow Research 190cc Street
	Air Flow Research FloPower RPM
	Comp Cams hydraulic roller
	230/236°
	0.510/0.520
	 
	1 5/8 in.
	 
	 
	 
	MSD
	36°
	9.5:1
	450 @ 5500
	470 @ 4500
	Air Flow Research website

	15
	383
	Holley 750 double pumper
	Air Flow Research 190cc Street
	Air Flow Research FloPower RPM
	Comp Cams 280° Magnum hydraulic
	230/230°
	0.480/0.480
	 
	1 3/4 in.
	 
	 
	 
	MSD
	34°
	9.5:1
	471 @ 5500
	515 @ 4500
	Air Flow Research website

	16
	383
	Holley 750 double pumper
	Air Flow Research 190cc Street
	Air Flow Research FloPower RPM
	Comp Cams hydraulic roller
	230/236°
	0.510/0.520
	 
	1 3/4 in.
	 
	 
	 
	MSD
	36°
	9.5:1
	491 @ 5500
	501 @ 4000
	Air Flow Research website

	17
	383
	Holley 750 double pumper
	Air Flow Research 190cc Street
	Air Flow Research FloPower RPM
	Comp Cams hydraulic roller
	236/242°
	0.520/0.540
	 
	1 3/4 in.
	 
	 
	 
	MSD
	34°
	9.5:1
	503 @ 5500
	517 @ 4500
	Air Flow Research website

	18
	355
	Holley 750 double pumper
	GM #882 castings, milled to 71 cc, with 2.02/1.60 in. valves
	Holley Contender
	hydraulic
	230/230°
	0.480/0.480
	PAW 1.5:1 long slot
	 
	PAW cast
	stock
	cast
	MSD Pro Billet HEI
	 
	10:1
	317 @ 5200
	368 @ 3700
	Engines 2000 Magazine, p. 50-51

	19
	383
	Holley 650 double pumper
	Trick Flow Specialties Twisted Wedge
	Edelbrock Victor Jr., ported
	Crane hydraulic roller
	214/222°
	0.488/0.510
	Crane 1.5:1 aluminum roller
	 
	KB Performance hypereutectic
	stock
	cast
	MSD Pro Billet HEI
	 
	9.8:1
	451 @ 6000
	445 @ 4500
	Engines 2000 Magazine, p. 50-51

	20
	383
	Holley 750 double pumper
	Trick Flow Specialties Twisted Wedge
	Edelbrock Victor Jr.
	Crane hydraulic roller
	230/236°
	0.510/0.512
	Crane roller, 1.5:1
	1 5/8 in.
	KB hypereutectic
	5.7 in.
	cast
	 
	 
	9:1 to 10:1
	450 @ 5750
	445 @ 4750
	www.findarticles.com/cf_0/m1185/n8_v32/21148174/p1/article.jhtml

	21
	383
	Holley 750
	Brodix-8
	Edelbrock Performer
	Crane hydraulic roller 278
	222/222°
	0.470/0.470
	Crane aluminum roller, 1.6:1
	Cyclone Corvette 1 5/8 in.
	Manley flat tops
	Chevy High Performance forged 5.7 in.
	cast
	MSD with a 7AL module
	 
	10:1
	431 @ 5500
	452 @ 4000
	Hot Rod Magazine, January 1990, p. 60

	22
	355
	Quadrajet
	GM #882 castings, milled to 71cc, with 1.94/1.50 in. valves, and a 3-angle valve job
	GM cast iron
	GM
	204/202°
	0.389/0.408
	 
	1 3/4 in. with 3 in. Flowmaster exhaust
	GM cast domed (2.4cc)
	stock
	cast
	stock
	 
	10:1
	248 @ 4600
	330 @ 3600
	Car Craft Magazine, March 1997, p. 64

	23
	355
	Holley 750 double pumper
	Holley Systemax II, with 2.02/1.60 in. valves, and 185 cc intake runners
	Holley Systemax II
	Holley Systemax II
	234/242°
	0.485/0.502
	Comp Cams Pro Magnum roller 1.52:1
	1 3/4 in. with 3 in. Flowmaster exhaust
	GM cast domed (2.4cc)
	stock
	cast
	MSD Pro Billet HEI
	36°
	10.2:1
	400 @ 6000
	406 @ 4200
	Car Craft Magazine, March 1997, p. 64

	24
	355
	Edelbrock 750
	Edelbrock Performer RPM
	Edelbrock Performer RPM
	Edelbrock Performer RPM
	234/244°
	0.488/0.510
	 
	1 3/4 in.
	TRW
	stock, polished and shotpeened
	stock steel
	MSD 6A
	36°
	9.5:1
	420 @ 6000
	415 @ 4500
	Edelbrock website

	25
	355
	Edelbrock 750
	World Products Sportsman II
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.5:1
	367 @ 5600
	402 @ 4100
	Engine Masters 1999 Magazine, p. 75

	26
	355
	Edelbrock 750
	World Products Sportsman II
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 274° hydraulic
	230/236°
	0.519/0.523
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.5:1
	416 @ 5800
	396 @ 4300
	Engine Masters 1999 Magazine, p. 75

	27
	355
	Edelbrock 600
	World Products Sportsman II
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.5:1
	356 @ 5600
	384 @ 4100
	Engine Masters 1999 Magazine, p. 75

	28
	355
	Edelbrock 600
	World Products Sportsman II
	Edelbrock Performer
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.5:1
	354 @ 5600
	385 @ 4100
	Engine Masters 1999 Magazine, p. 75

	29
	355
	Edelbrock 750
	World Products Sportsman II
	Edelbrock Performer
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.5:1
	356 @ 5600
	395 @ 4100
	Engine Masters 1999 Magazine, p. 75

	30
	355
	Edelbrock 600
	World Products S/R Torquer
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	341 @ 5600
	380 @ 4200
	Engine Masters 1999 Magazine, p. 75

	31
	355
	Edelbrock 750
	World Products S/R Torquer
	Edelbrock Performer
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	338 @ 5600
	381 @ 4200
	Engine Masters 1999 Magazine, p. 75

	32
	355
	Edelbrock 750
	World Products S/R Torquer
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	348 @ 5600
	391 @ 4100
	Engine Masters 1999 Magazine, p. 75

	33
	355
	Edelbrock 750
	World Products S/R Torquer
	Edelbrock Performer
	Comp Cams Xtreme Energy 274° hydraulic
	230/236°
	0.519/0.523
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	355 @ 5800
	379 @ 4400
	Engine Masters 1999 Magazine, p. 75

	34
	355
	Edelbrock 750
	World Products S/R Torquer
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 274° hydraulic
	230/236°
	0.519/0.523
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	362 @ 5800
	382 @ 4300
	Engine Masters 1999 Magazine, p. 75

	35
	355
	Edelbrock 600
	GM #882 castings, 1.94/1.50 in. valves, $1250 of head work
	Edelbrock Performer
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	309 @ 5500
	358 @ 4300
	Engine Masters 1999 Magazine, p. 75

	36
	355
	Edelbrock 750
	GM #882 castings, 1.94/1.50 in. valves, $1250 of head work
	Edelbrock Performer
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	321 @ 5100
	366 @ 4100
	Engine Masters 1999 Magazine, p. 75

	37
	355
	Edelbrock 750
	GM #882 castings, 1.94/1.50 in. valves, $1250 of head work
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 262° hydraulic
	218/224°
	0.493/0.500
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	322 @ 5200
	371 @ 4000
	Engine Masters 1999 Magazine, p. 75

	38
	355
	Edelbrock 750
	GM #882 castings, 1.94/1.50 in. valves, $1250 of head work
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy 274° hydraulic
	230/236°
	0.519/0.523
	Comp Cams Pro Magnum 1.6:1
	 
	Ross Racing flat tops
	Lunati 4340 steel
	cast
	GM HEI with an MSD coil
	 
	9.0:1
	341 @ 5700
	366 @ 4300
	Engine Masters 1999 Magazine, p. 75

	39
	391
	Holley 770 vacuum secondary with both an open and holed spacer
	Holley Street Avenger
	Holley Street Avenger
	Lunati hydraulic flat tappet
	245/255°
	0.569/0.576
	Lunati aluminum roller, 1.6:1
	 
	JE forged
	C.A.T. 4340 H-beam, 5.7 in.
	C.A.T. 4340 forged steel (offset ground to make 3.832 in. stroke)
	 
	 
	10.25:1
	453 @ 5800
	453 @ 4200
	Camaro Performers, Winter 2002, p. 87-93

	40
	350
	 
	World Products S/R Torquer
	Edelbrock Performer RPM
	Crane Cams 278
	222/222°
	0.470/0.470
	Crane Cams Hi-Intensity roller
	 
	TRW hypereutectic
	Eagle forged
	GM cast
	 
	 
	10.0:1
	366 @ 5500
	372 @ 4000
	www.allchevyengines.com

	41
	383
	 
	World Products S/R Torquer
	Edelbrock Performer
	Crane Cams 278
	222/222°
	0.470/0.470
	Crane Cams Hi-Intensity roller
	 
	TRW hypereutectic
	Eagle forged
	Elgin cast steel
	 
	 
	10.0:1
	359 @ 5000
	411 @ 3500
	www.allchevyengines.com

	42
	383
	 
	World Products Sportsman II
	Edelbrock Performer RPM
	Crane Cams 284
	228/228°
	0.488/0.488
	Crane Cams roller
	 
	TRW hypereutectic
	Eagle forged
	Elgin cast steel
	 
	 
	10.0:1
	414 @ 6000
	452 @ 4500
	www.allchevyengines.com

	43
	383
	 
	Dart Pro 1 aluminum, with 2.05/1.60 in. valves
	Edelbrock Performer RPM
	Comp Cams hydraulic roller
	244/244°
	0.525/0.525
	Crane aluminum roller
	 
	TRW hypereutectic
	Eagle forged
	Elgin cast steel
	 
	 
	10.0:1
	436 @ 5500
	448 @ 4000
	www.allchevyengines.com

	44
	383
	 
	Trick Flow Specialties Twisted Wedge
	Edelbrock Victor Jr.
	Crane hydraulic roller
	230/236°
	0.510/0.520
	Crane 1.5:1 aluminum roller
	 
	KB Performance hypereutectic
	5.7 in.
	cast
	 
	 
	 
	450 @ 5750
	445 @ 4750
	small Summit catalog, August 1998, p. 3

	45
	383
	Holley 750 double pumper
	World Products S/R Torquer, bowl blended
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy
	212/218°
	0.447/0.454
	Comp Cams Magnum roller 1.52:1
	 
	cast
	5.7 in.
	SCAT cast
	MSD Pro Billet HEI
	30°
	9.6:1
	338 @ 4800
	426 @ 3600
	Engines 2000 Magazine, p. 31

	46
	420
	Reworked factory electronic Quadrajet
	Dart II (180 cc intake runners and 2.02/1.60 in. valves), bowl-ported and blended, port-matched to the intake, 3-angle valve job
	Reworked factory aluminum, port-matched to the heads
	Comp Cams Magnum 280
	230/230°
	0.512/0.512
	Comp Cams Magnum roller 1.6:1
	Edelbrock tubular with a Turbo City Grand National dual-exhaust system
	JE forged alloy, 4.155 in. bore
	Comp Cams Magnum 6 in.
	9.0:1
	 
	 
	 
	425 @ 5500
	446 @ 4500
	Hot Rod Magazine, March 1990, p. 25-29

	47
	350
	1971 Quadrajet with #74 primary jets and a #45 metering rod
	GM castings, with 78 cc combustion chambers, 1.94/1.50 in. valves, and a 2-angle valve job
	1969 GM non-EGR cast iron
	GM L69
	192/202°
	0.383/0.405
	 
	stock cast-iron exhaust manifolds with 2 1/4 in. pipes and Thrush Turbo mufflers
	cast
	stock
	cast
	 
	 
	7.8:1
	224 @ 4000
	335 @ 3000
	Hot Rod Magazine, October 1990, p. 40-50

	48
	350
	1971 Quadrajet with #74 primary jets and a #45 metering rod
	GM castings, with 78 cc combustion chambers, 1.94/1.50 in. valves, and a 2-angle valve job
	1969 GM non-EGR cast iron
	GM L69
	192/202°
	0.383/0.405
	 
	Hedman 1 5/8 in. headers with 2 1/4 in. pipes and Thrush Turbo mufflers
	cast
	stock
	cast
	 
	 
	7.8:1
	242 @ 4000 and 4250
	345 @ 3000
	Hot Rod Magazine, October 1990, p. 40-50

	49
	350
	Holley 750 double pumper
	GM castings, with 78 cc combustion chambers, 1.94/1.50 in. valves, and a 2-angle valve job
	Holley Contender
	GM L69
	192/202°
	0.383/0.405
	 
	Hedman 1 5/8 in. headers with 2 1/4 in. pipes and Thrush Turbo mufflers
	cast
	stock
	cast
	 
	 
	7.8:1
	256 @ 4250
	345 @ 3250
	Hot Rod Magazine, October 1990, p. 40-50

	50
	350
	Holley 750 double pumper
	GM castings, with 78 cc combustion chambers, 1.94/1.50 in. valves, and a 2-angle valve job
	Holley Contender
	Crane hydraulic
	204/216°
	0.427/0.454
	 
	Hedman 1 5/8 in. headers with 2 1/4 in. pipes and Thrush Turbo mufflers
	cast
	stock
	cast
	 
	 
	7.8:1
	273 @ 4750
	353 @ 3750
	Hot Rod Magazine, October 1990, p. 40-50

	51
	350
	Holley 750 double pumper
	GM castings, with 78 cc combustion chambers, 1.94/1.50 in. valves, and a 2-angle valve job
	Holley Contender
	Crane hydraulic
	216/228°
	0.454/0.480
	 
	Hedman 1 5/8 in. headers with 2 1/4 in. pipes and Thrush Turbo mufflers
	cast
	stock
	cast
	 
	 
	7.8:1
	297 @ 5250
	355 @ 3500 and 3750
	Hot Rod Magazine, October 1990, p. 40-50

	52
	350
	Holley 750 double pumper
	Corvette aluminum, with 58 cc combustion chambers and 1.94/1.50 in. valves, (PN 10134336)
	Holley Contender
	Crane hydraulic
	216/228°
	0.454/0.480
	 
	Hedman 1 3/4 in. headers with 2 1/4 in. pipes and Thrush Turbo mufflers
	cast
	stock
	cast
	 
	 
	9.9:1
	312 @ 5250 and 5500
	369 @ 3250 and 3500
	Hot Rod Magazine, October 1990, p. 40-50

	53
	355
	Holley 750 vacuum secondary
	GM #993 castings, ported, with 2.02/1.60 in. valves and a 3-angle valve job
	Edelbrock Victor Jr. and a 2 in. open-plenum spacer
	Cam Dynamics 290
	234/234°
	0.494/0.494
	 
	1 7/8 in.
	Speed-Pro forged flat tops
	stock
	cast
	 
	 
	9.0:1 to 10.0:1
	421 @ 5300
	438 @ 4700
	Hot Rod Magazine, October 1990, p. 58-60

	54
	355
	Holley 750 vacuum secondary
	GM castings, ported, with 2.05/1.60 in. valves and a 3-angle valve job
	Edelbrock Performer RPM and a 1 in. open-plenum spacer
	GM #141 mechanical (PN 3927140)
	256/268°
	0.469/0.483
	Crane roller
	1 7/8 in.
	Speed-Pro forged flat tops
	stock
	cast
	 
	36°
	9.0:1 to 10.0:1
	510 @ 5600
	481 @ 5300
	Hot Rod Magazine, October 1990, p. 58-60

	55
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job
	Holley Contender
	Crane 260
	204/216°
	0.427/0.454
	1.5:1 intake; 1.5:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	305 @ 5000
	381 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	56
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job
	Holley Contender
	Crane 260
	204/216°
	0.427/0.454
	1.5:1 intake; 1.6:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	311 @ 5000
	391 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	57
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job
	Holley Contender
	Crane 272
	216/228°
	0.454/0.480
	1.5:1 intake; 1.5:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	322 @ 5500
	376 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	58
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job
	Holley Contender
	Crane 272
	216/228°
	0.454/0.480
	1.5:1 intake; 1.6:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	326 @ 5500
	378 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	59
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job, bowl-blending, intake gaskets are matched to the intake
	Holley Contender
	Crane 260
	204/216°
	0.427/0.454
	1.5:1 intake; 1.5:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	319 @ 5000
	390 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	60
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job, bowl-blending, intake gaskets are matched to the intake
	Holley Contender
	Crane 260
	204/216°
	0.427/0.454
	1.5:1 intake; 1.6:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	323 @ 5000
	392 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	61
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job, bowl-blending, intake gaskets are matched to the intake
	Holley Contender
	Crane 272
	216/228°
	0.454/0.480
	1.5:1 intake; 1.5:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	343 @ 5500
	390 @ 4000
	Hot Rod Magazine, April 1991, p. 30-34

	62
	355
	Holley 650 double pumper
	Dart SR, with 1.94/1.50 in. valves, 4-angle valve job, bowl-blending, intake gaskets are matched to the intake
	Holley Contender
	Crane 272
	216/228°
	0.454/0.480
	1.5:1 intake; 1.6:1 exhaust
	Hedman 1 5/8 in. headers with Flowmaster dual-crossflow mufflers
	Summit cast
	Summit
	Summit cast
	MSD
	 
	9.0:1
	349 @ 5500
	390 @ 3500
	Hot Rod Magazine, April 1991, p. 30-34

	63
	408
	Holley 750 double pumper with a 2 in. Edelbrock spacer
	GM #441, with 2.02/1.60 in. valves, some minor bowl-work, and a 3-angle valve job
	Edelbrock Performer RPM
	Lunati Street Master hydraulic
	235/235°
	0.507/0.507
	Lunati
	Hedman 1 3/4 in. headers
	Lunati forged
	Lunati
	Lunati
	MSD 6A
	 
	9.5:1
	384 @ 5000
	448 @ 4000
	Hot Rod Magazine, May 1990, p. 70-74

	64
	408
	Holley 750 double pumper with a 2 in. Edelbrock spacer
	Brodix street heads, with 165 cc intake runners and 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Lunati Street Master hydraulic
	235/235°
	0.507/0.507
	Lunati
	Hedman 1 3/4 in. headers
	Lunati forged
	Lunati
	Lunati
	MSD 6A
	 
	10.2:1
	397 @ 5000
	466 @ 4000
	Hot Rod Magazine, May 1990, p. 70-74

	65
	408
	Holley 750 double pumper with a 2 in. Edelbrock spacer
	Brodix -8 heads, with 185 cc intake runners, 2.05/1.60 in. valves, and a back-cut on the exhaust valve
	Edelbrock Performer RPM
	Lunati Street Master hydraulic
	235/235°
	0.507/0.507
	Lunati
	Hedman 1 3/4 in. headers
	Lunati forged
	Lunati
	Lunati
	MSD 6A
	 
	10.2:1
	410 @ 5000
	476 @ 4000
	Hot Rod Magazine, May 1990, p. 70-74

	66
	408
	Holley 750 double pumper with a 2 in. Edelbrock spacer
	GM #441, with 2.02/1.60 in. valves, some minor bowl-work, and a 3-angle valve job
	Edelbrock Victor Jr.
	Lunati Street Master hydraulic
	235/235°
	0.507/0.507
	Lunati
	Hedman 1 3/4 in. headers
	Lunati forged
	Lunati
	Lunati
	MSD 6A
	 
	9.5:1
	393 @ 5000
	434 @ 4000
	Hot Rod Magazine, May 1990, p. 70-74

	67
	408
	Holley 750 double pumper with a 2 in. Edelbrock spacer
	Brodix street heads, with 165 cc intake runners and 2.02/1.60 in. valves
	Edelbrock Victor Jr.
	Lunati Street Master hydraulic
	235/235°
	0.507/0.507
	Lunati
	Hedman 1 3/4 in. headers
	Lunati forged
	Lunati
	Lunati
	MSD 6A
	 
	10.2:1
	401 @ 5000
	455 @ 4000
	Hot Rod Magazine, May 1990, p. 70-74

	68
	408
	Holley 750 double pumper with a 2 in. Edelbrock spacer
	Brodix -8 heads, with 185 cc intake runners, 2.05/1.60 in. valves, and a back-cut on the exhaust valve
	Edelbrock Victor Jr.
	Lunati Street Master hydraulic
	235/235°
	0.507/0.507
	Lunati
	Hedman 1 3/4 in. headers
	Lunati forged
	Lunati
	Lunati
	MSD 6A
	 
	10.2:1
	417 @ 5000
	460 @ 4500
	Hot Rod Magazine, May 1990, p. 70-74

	69
	355
	Holley 750 vacuum secondary (110 octane gas)
	Holley Systemax, with 2.02/1.60 in. valves, a 20° valve angle, and 185 cc intake runners
	Holley dual plane
	Holley
	234/242°
	0.473/0.488
	Crane Energizer aluminum 1.5:1
	1 5/8 in. Flowtech
	TRW forged
	stock
	cast
	Performance Distributors HEI
	 
	9.2:1
	405 @ 5600
	408 @ 4600
	1996 reprint of a Holley article in Chevy High Performance Magazine

	70
	383
	Two (2) Edelbrock 500 cfm PN# 1404
	Edelbrock E-Tec, with 170 cc intake ports
	Edelbrock Vortec dual quad
	Edelbrock Performer hydraulic flat
	204/214°
	0.420/0.442
	Lunati roller 1.5:1
	Hooker
	flat-tops
	 
	 
	Pertronix HEI
	 
	10.5:1
	370 @ 4800
	445 @ 3700
	Chevy High Performance Magazine, February 2005, p. 54-50

	71
	383
	Barry Grant Silver Claw 750
	Air Flow Research 190, with 2.02/1.60 in. valves, 190 cc intake runners, CNC porting, and extra hand-porting
	Edelbrock Performer RPM
	Comp Cams hydraulic roller
	235/242°
	0.592/0.614
	Comp Cams 1.6:1
	 
	TRW forged
	Lunati 5.7 in.
	Scat 8060 cast
	 
	36°
	9.4:1
	525 @ 5800
	535 @ 4200
	Super Chevy Magazine, March 1998, p. 84-94

	72
	350
	Demon Speed Demon 750
	GM #441, with 1.94/1.50 in. valves
	Edelbrock Performer RPM
	hydraulic
	224/224°
	0.447/0.447
	 
	 
	cast dished
	stock
	cast
	 
	 
	7.5:1–8.0:1
	324 @ 5200
	363 @ 3900
	Super Chevy Magazine, February 2000, p. 65-71

	73
	350
	Demon Speed Demon 750
	GM L31 Vortecs, with 1.94/1.50 in. valves
	Edelbrock Performer RPM Vortec
	hydraulic
	224/224°
	0.447/0.447
	 
	 
	cast dished
	stock
	cast
	 
	35°
	9.0:1
	375 @ 5700
	408 @ 4300
	Super Chevy Magazine, February 2000, p. 65-71

	74
	355
	Demon Speed Demon 750
	GM L98, with 1.94/1.50 in. valves and 163 cc intake runners
	Edelbrock Performer RPM
	Comp Cams 270 Magnum hydraulic
	224/224°
	0.470/0.470
	GM aluminum roller, 1.5:1
	Hooker Super Competition headers with Flowmaster 3 in., 3-chamber mufflers
	Federal-Mogul forged aluminum
	stock
	cast
	 
	35°
	9.4:1
	380 @ 6000
	386 @ 4100
	Popular Hot Rodding Magazine, December 1999, p. 54-57

	75
	355
	Demon Speed Demon 750
	Trick Flow 23° Street Heads, with 2.02/1.60 in. valves and 195 cc intake runners
	Edelbrock Performer RPM
	Comp Cams 286 Magnum hydraulic
	236/236°
	0.490/0.490
	GM aluminum roller, 1.5:1
	Hooker Super Competition headers with Flowmaster 3 in., 3-chamber mufflers
	Federal-Mogul forged aluminum
	stock
	cast
	 
	35°
	9.4:1
	423 @ 6200
	417 @ 4900
	Popular Hot Rodding Magazine, December 1999, p. 54-57

	76
	350
	Edelbrock Performer 600
	Edelbrock Performer, with 2.02/1.60 in. valves and a 3-angle valve job
	Edelbrock Performer
	Edelbrock hydraulic
	204/214°
	0.420/0.442
	 
	shorty 1 5/8 in. headers and dual pipes
	 
	 
	 
	 
	 
	8.5:1
	310 @ 5000
	378 @ 4000
	Super Chevy Magazine, July 2000, p. 125-130

	77
	355
	Holley 650
	GM #186, with 2.02/1.60 in. valves, reworked 160 cc intake runners, and bowl blending
	Edelbrock Victor Jr.
	Comp Cams 268
	 
	0.474/0.474
	Comp Cams 1.6:1 Magnum roller
	1 3/4 in. headers, a crossover pipe, and Cyclone Sonic turbo mufflers
	Sealed Power forged
	stock
	cast
	 
	34°
	10.5:1
	383 @ 5250
	404 @ 4100
	Chevy High Performance Magazine, April 1991, p. 64-67

	78
	355
	Demon Road Demon 625
	GM L31 Vortecs, with 1.94/1.50 in. valves
	Edelbrock Performer RPM Vortec
	Comp Cams Xtreme Energy 268° hydraulic
	224/230°
	0.508/0.512
	Comp Cams roller, 1.6:1
	Hedman Elite 1 5/8 in.
	Speed Pro dished
	stock
	Scat
	Pertronix HEI
	36°
	8.75:1
	390 @ 6000
	426 @ 4000
	Chevy High Performance Magazine, April 2001, p. 65-68

	79
	350
	Holley 750 double-pumper
	Air Flow Research 190, with 190 cc intake runners and 2.02/1.60 in. valves
	Holley Street Dominator single-plane
	Comp Cams Xtreme Energy 274° hydraulic
	230/236°
	0.487/0.490
	Comp Cams steel roller, 1.5:1, w/ Comp Cams 980 valvesprings (91 pounds of seat load and 229 pounds at 0.450 in. valve lift)
	Hooker 1 3/4 in. headers and Flowmaster 2 1/2 in. mufflers
	Federal-Mogul lightweight forged
	 
	 
	MSD-6A
	 
	9.8:1
	388 @ 5500
	380 @ 5000
	Chevy High Performance Magazine, April 2001, p. 72-74

	80
	350
	Holley 750 double-pumper
	Air Flow Research 190, with 190 cc intake runners and 2.02/1.60 in. valves
	Holley Street Dominator single-plane
	Comp Cams Xtreme Energy 274° hydraulic
	230/236°
	0.487/0.490
	Comp Cams steel roller, 1.5:1, w/ Comp Cams 981 valvesprings (105 pounds of seat load and 295 pounds at 0.450 in. valve lift)
	Hooker 1 3/4 in. headers and Flowmaster 2 1/2 in. mufflers
	Federal-Mogul lightweight forged
	 
	 
	MSD-6A
	 
	9.8:1
	408 @ 6300
	388 @ 5000
	Chevy High Performance Magazine, April 2001, p. 72-74

	81
	350
	Quadrajet
	GM #882 heads, rebuilt, with 2.02/1.60 in. valves.
	Edelbrock Performer
	GM stock hydraulic
	194/203°
	0.390/0.410
	roller, 1.5:1
	Hooker 1 5/8 in.
	Speed Pro forged
	Federal Mogul
	Federal Mogul
	HEI
	 
	9.0:1
	275 @ 4400
	351 @ 3500
	Hot Rod Magazine, July 1997, p. 40-46

	82
	350
	Quadrajet
	GM #882 heads, rebuilt, with 2.02/1.60 in. valves.
	Edelbrock Performer
	Federal Mogul
	214/224°
	0.472/0.496
	roller, 1.5:1
	Hooker 1 5/8 in.
	Speed Pro forged
	Federal Mogul
	Federal Mogul
	HEI
	 
	9.0:1
	296 @ 5000
	349 @ 4200
	Hot Rod Magazine, July 1997, p. 40-46

	83
	350
	Holley 750
	GM L31 Vortecs
	Edelbrock Performer RPM Vortec
	Federal Mogul
	214/224°
	0.503/0.529
	roller, 1.6:1
	Hooker 1 5/8 in.
	Speed Pro forged
	Federal Mogul
	Federal Mogul
	HEI
	 
	10.6:1
	375 @ 5400
	413 @ 4300
	Hot Rod Magazine, July 1997, p. 40-46

	84
	350
	Holley 750
	GM L31 Vortecs
	Edelbrock Performer RPM Vortec
	Federal Mogul
	224/224°
	0.512/0.524
	roller, 1.6:1
	Hooker 1 5/8 in.
	Speed Pro forged
	Federal Mogul
	Federal Mogul
	HEI
	 
	10.6:1
	364 @ 5900
	384 @ 4400
	Hot Rod Magazine, July 1997, p. 40-46

	85
	357
	Holley 600
	GM, with 1.94/1.50 in. valves, 76 cc combustion chambers, and a 3-angle valve job.
	Edelbrock Performer
	Elgin hydraulic
	 
	0.488/0.488
	Elgin
	1 7/8 in.
	Powerhouse
	stock
	cast
	HEI with MSD
	36°
	9.7:1
	360 @ 5000
	394 @ 4500
	Chevy High Performance Magazine, October 1996, p. 25-28

	86
	350
	Holley 650 double pumper
	Trick Flow Twisted Wedge, with 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Summit hydraulic
	 
	0.454/0.454
	 
	 
	TRW forged
	stock
	Engine Shop
	Accel Blueprint HEI with an Accel HEI Supercoil
	 
	streetable
	363 @ 5500
	400 @ 4750
	Chevy High Performance Magazine, October 1996, p. 141

	87
	355
	Holley 750
	Trick Flow Twisted Wedge
	Edelbrock Victor Jr.
	Lunati mechanical street roller
	 
	0.661/0.661
	Summit roller, 1.6:1
	 
	forged
	forged
	steel
	 
	35°
	10.0:1
	455 @ 6100
	435 @ 5000
	Super Chevy Magazine, September 1998, p. 82-89

	88
	350
	Demon 750
	GM L98 Corvette aluminum, with 1.94/1.50 in. valves and angled plugs
	GM aluminum Eliminator dual-plane, PN 12464340
	GM steel roller
	 
	0.474/0.510
	GM stamped steel, 1.5:1
	Hooker headers and Flowmaster 3-chamber mufflers
	GM
	GM forged
	GM
	HEI
	35°
	10.0:1
	356 @ 5500
	408 @ 3800
	Popular Hot Rodding Magazine, March 2000, p. 42-46

	89
	350
	Demon 750
	GM L31 Vortec, with 1.94/1.50 in. valves
	Edelbrock Super Victor Vortec
	GM steel roller
	 
	0.474/0.510
	GM stamped steel, 1.5:1
	Hooker headers and Flowmaster 3-chamber mufflers
	GM
	GM forged
	GM
	HEI
	32°
	9.5–10.0:1
	376 @ 5600
	380 @ 4700
	Popular Hot Rodding Magazine, March 2000, p. 42-46

	90
	350
	Demon 750
	GM Fast Burn
	GM aluminum dual-plane, PN 12464340; or an Edelbrock Performer RPM.
	GM steel roller "hot cam"
	210/220°
	0.525/0.525
	GM self-aligning roller, 1.6:1
	Hooker headers and Flowmaster 3-chamber mufflers
	GM
	GM forged
	GM
	HEI
	35°
	9.5–10.0:1
	418 @ 5900
	406 @ 4300
	Popular Hot Rodding Magazine, March 2000, p. 42-46

	91
	350
	Demon 750
	GM Fast Burn
	Edelbrock Super Victor Vortec
	GM steel roller "hot cam"
	210/220°
	0.525/0.525
	GM self-aligning roller, 1.6:1
	Hooker headers and Flowmaster 3-chamber mufflers
	GM
	GM forged
	GM
	HEI
	35°
	9.5–10.0:1
	432 @ 6100
	412 @ 4700
	Popular Hot Rodding Magazine, March 2000, p. 42-46

	92
	427 cu. in. World Products MoTown (4.185 in. bore x 3.875 in. stroke)
	Holley 750 with a Holley 850 base—flow is 870 cfm.
	World Products MoTown 220, with 2.08/1.60 valves
	World Products dual plane
	Crane solid
	244/252°
	0.516/0.525
	Manley aluminum roller, 1.5:1
	 
	forged aluminum
	Manley 4340 forged steel H-beam 6.0 in.
	4130 forged steel
	HEI
	36°
	10.0:1
	482 @ 5800
	545 @ 3800
	Hot Rod Magazine, August 2001, p. 24-28

	93
	350
	Speed Demon 850, with a 1 in. open spacer
	Air Flow Research 195, with 2.02/1.60 in. valves.
	Edelbrock Victor Jr.
	Comp Cams mechanical roller
	242/248°
	0.578/0.584
	Comp Cams Pro Magnum roller, 1.52:1
	1 3/4 in. headers with 3 in. Flowmasters
	GM high-silicon aluminum
	stock
	GM forged
	GM HEI
	35°
	9.97:1
	523 @ 6600
	454 @ 5200
	Car Craft Magazine, August 2001, p. 82-85

	94
	350
	Holley 750 double pumper
	Air Flow Research 190, with 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Comp Cams High Energy hydraulic
	206/206°
	0.431/0.431
	Comp Cams Pro Magnum roller, 1.52:1
	1 3/4 in. Hooker headers with 3 in. Flowmasters
	GM high-silicon aluminum
	stock
	GM forged
	Mallory distributor with an MSD 6AL box
	36°
	9.5:1
	348 @ 5000
	398 @ 3800
	Super Chevy Magazine, April 1998, p. 40-46

	95
	350
	Holley 750 double pumper
	Air Flow Research 190, with 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Comp Cams High Energy hydraulic
	218/218°
	0.460/0.460
	Comp Cams Pro Magnum roller, 1.52:1
	1 3/4 in. Hooker headers with 3 in. Flowmasters
	GM high-silicon aluminum
	stock
	GM forged
	Mallory distributor with an MSD 6AL box
	36°
	9.5:1
	380 @ 5400
	402 @ 4100
	Super Chevy Magazine, April 1998, p. 40-46

	96
	350
	Holley 750 double pumper
	Air Flow Research 190, with 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Comp Cams Magnum hydraulic
	230/230°
	0.486/0.486
	Comp Cams Pro Magnum roller, 1.52:1
	1 3/4 in. Hooker headers with 3 in. Flowmasters
	GM high-silicon aluminum
	stock
	GM forged
	Mallory distributor with an MSD 6AL box
	36°
	9.5:1
	404 @ 5600
	403 @ 4400
	Super Chevy Magazine, April 1998, p. 40-46

	97
	350
	Holley 750 double pumper
	Air Flow Research 190, with 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Comp Cams Magnum hydraulic
	244/244°
	0.507/0.507
	Comp Cams Pro Magnum roller, 1.52:1
	1 3/4 in. Hooker headers with 3 in. Flowmasters
	GM high-silicon aluminum
	stock
	GM forged
	Mallory distributor with an MSD 6AL box
	36°
	9.5:1
	431 @ 6700
	400 @ 4500
	Super Chevy Magazine, April 1998, p. 40-46

	98
	355
	Holley 750
	World Products S/R Torquer, with 2.02/1.60 in. valves
	Edelbrock Performer RPM
	Comp Cams Xtreme Energy hydraulic
	206/212°
	0.432/0.444
	 
	1 5/8 in.
	 
	 
	 
	 
	 
	9.25:1
	330 @ 5000
	410 @ 3500
	Chevy High Performance Magazine, September 2001, p. 23-30

	99
	355
	Holley 750 double-pumper
	GM L31 Vortecs, pocket-ported, with 1.94/1.50 in. valves
	Edelbrock Performer RPM Vortec
	Comp Cams Xtreme Energy 268 hydraulic
	224/230°
	0.478/0.480
	 
	Hedman 1 5/8 in. headers and Borla XR-1 mufflers
	 
	 
	 
	HEI
	 
	9.4:1
	409 @ 6000
	427 @ 3500
	Chevy High Performance Magazine, September 2001, p. 23-30

	100
	383
	Holley 750 double-pumper
	Trick Flow 23° aluminum
	Edelbrock Performer RPM
	Comp Cams mechanical street roller
	242/248°
	0.570/0.576
	 
	Hedman 1 5/8 in. headers and Borla ZR-1 mufflers
	SRP forged
	SCAT
	SCAT
	Crane HI-6
	 
	10.0:1
	454 @ 6000
	469 @ 4000
	Chevy High Performance Magazine, September 2001, p. 23-30

	101
	406
	Holley 750
	Air Flow Research 210, with 2.08/1.60 in. valves.
	Edelbrock Victor Jr.
	Crane mechanical street roller
	236/244°
	0.525/0.543
	 
	Hedman 1 5/8 in. headers with Borla XR-1 mufflers
	forged
	stock
	GM
	MSD 6AL
	 
	10.0:1
	487 @ 5500
	494 @ 5000
	Chevy High Performance Magazine, September 2001, p. 23-30

	102
	350
	Demon 850
	Air Flow Research 195, with 2.02/1.60 in. valves
	Edelbrock Victor Jr.
	Comp Cams hydraulic roller
	248/254°
	0.562/0.580
	 
	Hooker 1 3/4 in. headers and 2 1/2 in. Flowmaster mufflers
	GM high-silicon aluminum
	GM
	GM forged
	MSD 6AL
	 
	10.0:1
	506 @ 6500
	436 @ 5500
	Chevy High Performance Magazine, September 2001, p. 23-30

	103
	415 cu. in. World Products MoTown (4.125 in. bore x 3.875 in. stroke)
	Holley 750 with a Holley 850 base—flow is 870 cfm.
	World Products MoTown 220, with 2.08/1.60 valves
	World Products Sportsman dual plane
	Crane hydraulic
	244/252°
	0.516/0.525
	Manley aluminum roller, 1.5:1
	1 3/4 in.
	Wiseco-JE forged aluminum
	Manley 4340 forged steel H-beam 6.0 in.
	4130 forged steel
	HEI
	38°
	9.0:1
	485 @ 5600 and 5700
	515 @ 4300
	Car Craft Magazine, October 2001, p. 36-41

	104
	355
	Edelbrock 750 Performer.
	Edelbrock E-Tec 170, with 1.94/1.50 valves
	Edelbrock RPM Air-Gap Vortec
	Erson TQ40H hydraulic flat-tappet
	218/228°
	0.503/0.503
	1.6:1
	1 3/4 in. primaries with 3 in. collectors
	stock.
	stock.
	stock.
	 
	 
	8.5:1
	400 @ 5500
	417 @ 4500
	Car Craft Magazine, September 2001, p. 76-77

	105
	355
	Edelbrock 750 Performer.
	Edelbrock E-Tec 200, with 2.02/1.60 valves
	Edelbrock RPM Air-Gap Vortec
	Erson TQ40H hydraulic flat-tappet
	218/228°
	0.503/0.503
	1.6:1
	1 3/4 in. primaries with 3 in. collectors
	stock.
	stock.
	stock.
	 
	 
	8.6:1
	391 @ 5400
	413 @ 4400
	Car Craft Magazine, September 2001, p. 76-77

	106
	383
	Holley 750 double pumper
	World Products MoTown 220 iron (217 actual cc), with 2.055/1.60 valves
	World Products Sportsman dual plane
	Crane hydraulic roller
	232/240°
	0.525/0.543
	Crane gold aluminum roller, 1.5:1
	Hedman 1 5/8 in. headers with 2 1/2 in. Borla mufflers.
	SRP forged aluminum
	SCAT forged 4130 I-beam 6.0 in.
	SCAT internally balanced cast-iron
	Moroso HEI with a Crane HI-6 ignition
	37°
	9.9:1
	407 @ 5600
	449 @ 4200
	Chevy High Performance Magazine, March 2002, p. 60-64

	107
	383
	Holley 750 double pumper
	World Products MoTown 220 aluminum (224 actual cc), with 2.055/1.60 valves
	World Products Sportsman dual plane
	Crane hydraulic roller
	232/240°
	0.525/0.543
	Crane gold aluminum roller, 1.5:1
	Hedman 1 5/8 in. headers with 2 1/2 in. Borla mufflers.
	SRP forged aluminum
	SCAT forged 4130 I-beam 6.0 in.
	SCAT internally balanced cast-iron
	Moroso HEI with a Crane HI-6 ignition
	37°
	9.9:1
	408 @ 5600
	454 @ 4200
	Chevy High Performance Magazine, March 2002, p. 60-64

	108
	355
	Speed Demon 750 with mechanical secondaries
	GM L31 Vortec
	GMPP Vortec dual-plane
	GMPP HOT hydraulic roller
	218/228°
	0.525/0.525
	 
	Hooker 1 5/8 in. headers with Flowmaster 2 1/2 mufflers
	stock
	stock
	GM
	Stinger electronic with MSD 6
	34°
	9.75:1
	412 @ 5600
	423 @ 4400
	Chevy High Performance Magazine, November 2001, p. 80-87

	109
	355
	Speed Demon 750 and a 1 1/2 in. spacer
	GM L31 Vortec
	Edelbrock Super Victor single plane
	GMPP HOT hydraulic roller
	218/228°
	0.525/0.525
	 
	Hooker 1 5/8 in. headers with Flowmaster 2 1/2 mufflers
	stock
	stock
	GM
	Stinger electronic with MSD 6
	32°
	9.75:1
	422 @ 5700
	418 @ 5000
	Chevy High Performance Magazine, November 2001, p. 80-87

	110
	383
	Holley 850 Hardcore
	World Products Motown 220
	Edelbrock Performer RPM Air Gap
	Comp Cams hydraulic roller
	242/248°
	0.540/0.562
	Comp Cams Pro Magnum 1.52:1
	Summit
	Speed Pro forged 12 cc dish
	stock
	Summit forged
	Tri-Power
	 
	10.0:1
	505 @ 5800 and 5900
	502 @ 4700
	Hot Rod Magazine, March 2005, p. 116-118

	111
	355
	Holley 750
	Holley Systemax, with 2.02/1.60 in. valves, a 20° valve angle, and 185 cc intake runners
	Holley dual plane
	Lunati hydraulic flat tappet
	235/240°
	0.490/0.490
	Lunati roller 1.5:1
	Flowtech 1 5/8 in.
	hypereutectic
	stock
	stock
	MSD w/ Blaster 2 coil
	36°
	9.5:1
	412 @ 5800
	432 @ 4200
	Chevy High Performance Magazine, March 2005, p. 74-84

	112
	359
	Holley Avenger 770
	Holley Systemax, with 2.02/1.60 in. valves, a 20° valve angle, and 185 cc intake runners
	Holley Street Dominator dual-plane
	Lunati Wild Street hydraulic flat tappet
	235/240°
	0.523/0.523
	Lunati aluminum roller 1.6:1
	Hooker 1 3/4 in.
	Lunati 4.04 in. dia.
	 
	3.500 in. stroke
	Holley billet distributor, with Street Avenger Annihilator ignition box.
	 
	9.74:1
	461 @ 6300
	446 @ 4500
	Hot Rod Magazine, November 2002, p. 51-55

	113
	407
	Holley HP 750 double pumper w/ 1 in. open spacer
	AFR 210 (70% CNC machined)
	Edelbrock Victor Jr.
	Crane hydraulic roller
	234/242°
	0.558/0.556
	 
	 
	SRP flattops
	CAT Power H-beam
	Scat 4340 lightened
	MSD Pro Billet HEI
	36°
	10.2:1
	513 @ 5700
	511 @ 4900
	Hot Rod Magazine, February 2003, p. 42-43

	114
	383
	Holley HP 750 double pumper w/ 2 in. four-hole spacer
	AFR 195
	Edelbrock Victor Jr.
	Comp Cams solid Magnum 294S
	248/248°
	0.525/0.525
	Comp Cams Pro Magnum 1.5:1
	 
	Probe forged flattops
	stock
	Summit cast steel
	 
	34°
	10.5:1
	542 @ 6300 w/ 100 octane
(533 w/ 91 octane)
	517 @ 4600 w/ 100 octane
	Car Craft Magazine, June 2002, p. 58-63

	115
	383
	Road Demon 625
	Vortec
	GM medium-rise aluminum dual plane
	GM hydraulic roller
	196/207°
	0.431/0.451
	1.52:1
	Hedman Tork-step 1 1/2"
	hypereutectic dish
	powdered metal
	forged
	Pertronix HEI
	34°
	9.1:1
	338 @ 4400
	444 @ 3400
	Hot Rod Magazine, March 2002, p. 25-32

	116
	383
	Road Demon 625
	Vortec
	GM medium-rise aluminum dual plane
	GM hydraulic roller
	196/207°
	0.431/0.451
	Comp Cams Pro Magnum 1.6:1
	Hedman Tork-step 1 1/2"
	hypereutectic dish
	powdered metal
	forged
	Pertronix HEI
	34°
	9.1:1
	355 @ 4700
	453 @ 3500
	Hot Rod Magazine, March 2002, p. 25-32

	117
	383
	Road Demon 625
	Vortec
	Edelbrock Performer RPM Air Gap
	GM hydraulic roller
	196/207°
	0.431/0.451
	Comp Cams Pro Magnum 1.6:1
	Hedman Tork-step 1 1/2"
	hypereutectic dish
	powdered metal
	forged
	Pertronix HEI
	34°
	9.1:1
	370 @ 4600
	467 @ 3700
	Hot Rod Magazine, March 2002, p. 25-32

	118
	383
	Speed Demon 750
	Vortec
	Edelbrock Performer RPM Air Gap
	GM hydraulic roller
	196/207°
	0.431/0.451
	Comp Cams Pro Magnum 1.6:1
	Hedman Tork-step 1 1/2"
	hypereutectic dish
	powdered metal
	forged
	Pertronix HEI
	34°
	9.1:1
	379 @ 4600
	474 @ 3700
	Hot Rod Magazine, March 2002, p. 25-32

	119
	383
	Speed Demon 750
	Vortec
	Edelbrock Performer RPM Air Gap
	Comp Cams XE282HR
	230/236°
	0.544/0.555
	Comp Cams Pro Magnum 1.6:1
	Hedman Tork-step 1 1/2"
	hypereutectic dish
	powdered metal
	forged
	Pertronix HEI
	34°
	9.6:1
	455 @ 5600
	493 @ 4200
	Hot Rod Magazine, March 2002, p. 25-32

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	


